3GPP TSG-RAN WG2 Meeting #113-electronic	 R2-2100142

Online, January 25th – February 5th, 2021

Source: 	vivo

Title:  	Supporting small data transmission via CG configuration

Agenda Item:	8.6.5

Document for:	Discussion and Decision

Introduction

In the past RAN2 meetings [1], the following agreements regarding CG-SDT had been achieved:

Agreements: - The configuration of configured grant resource for UE uplink small data transfer is contained in the RRCRelease message. FFS if other dedicated messages can configure CG in INACTIVE CG. Configuration is only type 1 CG with no contention resolution procedure for CG.  - The configuration of configured grant resource can include one type 1 CG configuration. FFS if multiple configured CGs are allowed.  - A new TA timer for TA maintenance specified for configured grant based small data transfer in RRC_INACTIVE should be introduced. FFS on the procedure, the validity of TA, and how to handle expiration of TA timer. The TA timer is configured together with the CG configuration in the RRCRelease message.  - The configuration of configured grant resource for UE small data transmission is valid only in the same serving cell. FFS for other CG validity criteria (e.g. timer, UL/SUL aspect , etc).  - The UE can use configured grant based small data transfer if at least the following criteria is fulfilled  - user data is smaller than the data volume threshold;  - configured grant resource is configured and valid;  - UE has valid TA. FFS for the candidate beam criteria.  - From RAN2 point of view: An association between CG resources and SSBs is required for CG-based SDT. FFS up to RAN1 how the association is configured or provided to the UE. Send an LS to RAN1 to start the discussion on how the association can be made. Mention that one option RAN2 considered was explicit configuration with RRC Release message.  - A SS-RSRP threshold is configured for SSB selection. UE selects one of the SSB with SS-RSRP above the threshold and selects the associated CG resource for UL data transmission.

In this contribution, we would like to further provide our views on the CG-SDT procedure in terms of:

- CG configuration;

- TA validation;

- UL skipping;

- Response reception;

- Fallback.

Discussion

CG configuration

Comparing between the RA-SDT and CG-SDT, in our understanding, the preamble transmission is skipped to improve power efficiency, and contention among UEs can be avoided to increase the success ratio. Additionally, since the PUSCH resource is provided by configured grant configuration, thus it should be assumed that the gNB is able to know well about the traffic pattern. Otherwise, the resource may be wasted. Based on this, we think the CG-SDT is mainly supposed to use for some low-end RedCap UE performing meters reporting.

According to the current 38.306 spec [2], in Rel-15, the UE capability of multiple configured grants is optional and at most one CG configuration can be used per BWP. Further, in Rel-16, the capability of supporting up to 12 configured grant configuration is also optional.

| multipleConfiguredGrants Indicates whether UE supports more than one configured grant configurations (including both Type 1 and Type 2) in a cell group. For each cell, the UE supports at most one configured grant per BWP and the maximum number of configured grant configurations per cell group is 2. If absent, for each configured cell group, the UE only supports one configured grant configuration on one serving cell. | UE | No | Yes | No |
| --- | --- | --- | --- | --- |
| activeConfiguredGrant-r16 Indicates whether the UE supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. This field includes the following parameters: - maxNumberConfigsPerBWP-r16 indicates the maximum number of configured/active configured grant configurations in a BWP of a serving cell. - maxNumberConfigsAllCC-r16 indicates the maximum number of configured/active configured grant configurations across all serving cells in a MAC entity. The UE can include this feature only if the UE indicates supports of either configuredUL-GrantType1 or configuredUL-GrantType2. | Band | No | N/A | N/A |

Therefore, to reduce the UE complexity, we propose at most 1 CG configuration can be configured for the CG-SDT.

Proposal 1: RAN2 confirms that at most 1 CG configuration can be configured for CG-SDT.

Moreover, in the NR SDT procedure, subsequent transmission can be performed after the first UL transmission with state transition controlled by the network. Then the next question that comes to us is whether the DL SPS configuration used for subsequent transmission can be configured simultaneously along with the CG configuration in the RRC Release message. Given that the NR SDT procedure is mainly introduced for MO data transmission and the MT data arrives within CG SDT procedure usually is acknowledgment information for the previous MO data, we think there is no need to include DL SPS configuration in the RRC Release message for CG SDT.

Proposal 2: RAN2 confirms that SPS configuration is not supported during the CG-SDT procedure.

TA validation

In the previous RAN2 meeting, A new TA timer for TA validation is agreed to be introduced. And the UE behavior is FFS when this timer expires. In our thinking, the CG-SDT mechanism is friendly for some stationary reporting device in a small cell (e.g. family network), which might only perform one-shot UL transmission within a day. Therefore, considering the use case of this type of device, we think this new TA timer is optionally configured for a specific UE. The CG resource can be considered valid as long as it is not explicitly or implicitly release.

Proposal 3: The new TA timer for TA validation is optionally configured in CG-SDT.

UL skipping

Given that the realistic traffic pattern is difficult to precisely predict, thus, it is possible that the UE does not have any available UL UP date at the transmission occasion of CG PUSCH. In legacy, the UL skipping will be performed (i.e. no MAC PDU is generated) to avoid unnecessary power consumption. Considering that the UL skipping feature is conditionally mandatory for type 1 and type 2 CG, so we propose the UL skipping feature for the configured grant is also applicable to CG based SDT.

Proposal 4: The UL skipping feature for the configured grant is supported for CG-SDT.

At the network side, since the NW cannot know whether the UE will skip the CG PUSCH, it has to keep trying decoding the PUSCH. To reduce resource waste, an autonomous CG release mechanism should be considered. For example, after N times of UL skipping, the UE will give up the CG-SDT procedure. At the NW sides, it can also assume that the CG resource has been release if it can not successfully decoding a PUSCH for N times.

Proposal 5: The UE can autonomously release the CG configuration and abort the CG-SDT procedure after N times of UL skipping.

Response reception

Assuming that the resource selection issue has been solved, then the UE can transmit the corresponding CG PUSCH. Similar to the RACH procedure, we think a response window should be started, during which the UE monitors the PDCCH for a response.

Proposal 6: UE starts a window for PDCCH monitoring after the CG PUSCH transmission in SDT.

If the response window expires, the UE should perform retransmission in the next available transmission occasion of CG PUSCH. Further, a power ramping operation should be considered to improve the robustness. Otherwise, the UE may be stuck in the poor radio quality environment.

Proposal 7: Autonomous retransmission on CG resource is supported for CG-SDT.

Proposal 8: Power ramping is supported for CG-SDT.

Furthermore, for RRC-less CG based SDT procedure, it is possible that there is no available DL data at the gNb side and the RRC message cannot be transmitted in this case. A DL positive acknowledgment is needed as a response to the UE so that the misleading timer expiry can be avoided.

Proposal 9: A DL positive acknowledgment should be supported for CG based SDT.

Fallback

To improve transmission efficiency, a fallback mechanism should be considered. For example, the network can control the number of times 'N'. If the CG based SDT procedure is not successfully completed even after 'N' times of PUSCH transmission, the UE will fallback to use legacy RA resources (e.g. 4-step RA resources or 2-step RA resources) and re-initiate the RRC connection resume procedure. For another example, the network may explicitly indicate a fallback indication considering the netwrok congestion or channel condition. Upon receiving the fallback indication, the UE will abort CG based SDT procedure and may re-initiate the RRC connection resume procedure.

Proposal 10: Fallback mechanism from CG-SDT to legacy RRC connection resume should be considered.

Conclusions

In this contribution, we have provided our understanding of the CG-SDT procedure. The following proposals are made:

CG configuration:

Proposal 1: RAN2 confirms that at most 1 CG configuration can be configured for CG-SDT.

Proposal 2: RAN2 confirms that SPS configuration is not supported during the CG-SDT procedure.

TA validation:

Proposal 3: The new TA timer for TA validation is optionally configured in CG-SDT.

UL skipping:

Proposal 4: The UL skipping feature for the configured grant is supported for CG-SDT.

Proposal 5: The UE can autonomously release the CG configuration and abort the CG-SDT procedure after N times of UL skipping.

Response reception:

Proposal 6: UE starts a window for PDCCH monitoring after the CG PUSCH transmission in SDT.

Proposal 7: Autonomous retransmission on CG resource is supported for CG-SDT.

Proposal 8: Power ramping is supported for CG-SDT.

Proposal 9: A DL positive acknowledgment should be supported for CG based SDT.

Fallback:

Proposal 10: Fallback mechanism from CG-SDT to legacy RRC connection resume should be considered.

- 5. Reference

[1] RAN2#112-e Session Chair Notes (Small data transmission), E-meeting, 2nd - 13th Novemebr 2020.

[2] 3GPP TS 38.306, “User Equipment (UE) radio access capabilities”.