3GPP TSG-RAN WG2 Meeting #113-e                             R2-2100200

Online, Jan 25 – Feb 5 2021

Agenda item:	8.3.4

Source:	Samsung

Title:	Discussion on support of paging cause for Multi-SIM devices

Document for:	Discussion & Decision

Introduction

At RAN2#112-e meeting, RAN2 has discussed whether and how to indicate to the UE why it is paged. But, no conclusion was made as per below:

Provide SA2 with information on paging cause costs based on the email discussion + contributions. Indicate that this may change if assumptions change.

From RAN2 perspective, we haven't decided on paging cause feasibility yet.

RAN2 will evaluate short/long time switching in this WI

This contribution discusses this aspect based on the latest SA2 progress with detailed impacts on RAN2.

Discussion

Paging cause feasibility

According to the latest conclusion in SA2 [1], only single paging cause is agreed to indicate to the UE whether it is paged for voice service for both EPS and 5GS.

Observation 1: SA2 agreed to support only one paging cause to indicate to the UE whether it is paged for voice service in paging message for both EPS and 5GS.

For one paging record in paging message, it entails two bits at a minimum i.e. one bit to encode the paging cause for voice service and one optional bit whether paging cause is included or not.

Observation 2: For one paging record in paging message, two bits are required at a minimum i.e. one bit to encode the paging cause for voice service and one optional bit whether paging cause is included or not.

We think that its overhead will have a negligible impact on the paging size. Thus it is feasible to include paging cause per UE record in paging message for both EPS and 5GS.

Proposal 1: From RAN2 perspective, it is feasible to support paging cause per UE record in paging message for both EPS and 5GS.

How to signal paging cause

It is of importance to discuss how to signal paging cause for each paging record in details. In [2], it was proposed to define a new parallel list as listed in the same order in pagingRecordList i.e. see ASN.1 example below.

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList               OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                   OPTIONAL,

nonCriticalExtension                SEQUENCE{Paging-v17xy-IEs}     OPTIONAL

}

Paging-v17xy-IEs ::=                SEQUENCE {

pagingRecordList-v17xy             PagingRecordList-v17xy     OPTIONAL, -- Need N

nonCriticalExtension               SEQUENCE{}                OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v17xy  ::=         SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v17xy

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v17xy ::=                SEQUENCE {

pagingCause-r17                   ENUMERATED  {voice}     OPTIONAL -- Need R

}

Its drawback is to signal the whole new parallel list in case at least one UE record needs to be associated with the paging cause.

Observation 3: It is signalling inefficient to define new parallel list, as listed in the same order in pagingRecordList, because it needs to be signalled in case at least one UE record needs to be associated with the paging cause.

Rather than defining new parallel list as listed in the same order in pagingRecordList, it is more signalling efficient to define new separate list including the complete paging records for only multi-SIM UEs to indicate whether they are paged for voice-service or non-voice service i.e. see ASN.1 example below.

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList               OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                   OPTIONAL,

nonCriticalExtension                SEQUENCE{Paging-v17xy-IEs}     OPTIONAL

}

Paging-v17xy-IEs ::=                SEQUENCE {

pagingRecordList2-r17             PagingRecordList2-r17     OPTIONAL, -- Need N

nonCriticalExtension               SEQUENCE{}                OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList2-r17  ::=         SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-r17

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-r17 ::=                SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

paginingCause-r17                   ENUMERATED  {voice}     OPTIONAL -- Need S

}

Proposal 2: Define new paging record list where each paging record includes existing fields and new paging cause in paging message. This new paging record list is used for only multi-SIM UEs (supporting paging cause feature).

Given that SA2 agreed to support only single paging cause, it seems unnecessary to define some spare paging cause values as it may have a negative impact on the size of paging message. On the other hand, defining some spare values may be beneficial for forward compatibility.

Proposal 3: RAN2 to discuss whether to define spare paging cause values in paging message for forward compatibility.

Which layer to trigger resume procedure in RRC_INACTIVE

Currently, UE AS layer (e.g. RRC) initiates the RRC connection resume procedure in response to RAN paging.

Observation 4: Currently UE AS layer (e.g. RRC) initiates the RRC connection resume procedure in response to RAN paging.

If paging cause can be included in RAN paging and UE supports its feature, it is not clear which layer needs to respond to RAN paging. Although it may be a modeling issue, the prerequisite to be decided is that either RRC or NAS of one USIM needs to be aware of whether/which a certain service on other USIM is ongoing i.e.

- If NAS of one USIM is aware of whether/which a certain service on other USIM is ongoing, RRC needs to forward paging cause (including none) to NAS to decide whether NAS responds to RAN paging.

- If AS of one USIM is aware of whether/which a certain service on other USIM is ongoing, RRC itself can decide whether to respond to RAN paging as it is.

Proposal 4: RAN2 to discuss which layer (e.g. RRC or NAS) determines to respond to RAN paging if UE supports paging cause feature in RAN paging.

Conclusion

In section 2, we made the following observations:

Observation 1: SA2 agreed to support only one paging cause to indicate to the UE whether it is paged for voice service in paging message for both EPS and 5GS.

Observation 2: For one paging record in paging message, two bits are required at a minimum i.e. one bit to encode the paging cause for voice service and one optional bit whether paging cause is included or not.

Observation 3: It is signalling inefficient to define new parallel list, as listed in the same order in pagingRecordList, because it needs to be signalled in case at least one UE record needs to be associated with the paging cause.

Observation 4: Currently UE AS layer (e.g. RRC) initiates the RRC connection resume procedure in response to RAN paging.

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposals:

Proposal 1: From RAN2 perspective, it is feasible to support paging cause per UE record in paging message for both EPS and 5GS.

Proposal 2: Define new paging record list where each paging record includes existing fields and new paging cause in paging message. This new paging record list is used for multi-SIM UEs (supporting paging cause feature).

Proposal 3: RAN2 to discuss whether to define spare paging cause values in paging message for forward compatibility.

Proposal 4: RAN2 to discuss which layer (e.g. RRC or NAS) determines to respond to RAN paging if UE supports paging cause feature in RAN paging.

References

[1] 3GPP TR 23.761, "Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM)."

[2] R2-2009325, "Summary of [Post111-e][917] Multi-Sim (vivo)."