3GPP TSG-RAN WG2 Meeting #113-e	R2-2100209

Online, 25 Janurary–5 February 2021

# Agenda item:	8.12.2.2

# Source:	Samsung

# Title:	UAC enhancements for RedCap UE

# Document for:	Discussion and Agreement

# 1	Introduction

The contribution discusses enhancement of Unified Access Control (UAC) for REDCAP UEs.

# 2	Discussion

At the last RAN2 meeting, RAN2 discussed whether to send an LS to SA1 for the enhancement of UAC for REDCAP UEs, but decided not to send it for now since RAN2 did not make notable progress on the issue [1].

Agreements:

- Include the possible options (msg1, msg3, msg5) in the TP without saying anything on RAN2 preferences on when identification is required

- Do not send a LS on RedCap UE identification to RAN1 and wait for more RAN1 process

- Postpone the LS to SA1 on UAC enhancement for RedCap UEs.

- Postpone the discussion on the camping indicator for RedCap UEs to the WI phase.

- Postpone the discussion on intraFreqReselection indicator for RedCap UEs to the WI phase.

For the UAC, network broadcasts barring control information associated with Access Identities and Access Categories, and with this information, UE determines whether to access.

Access Identities and Access Categories defined in TS 22.261 [2] are as follows:

| Access Identity number | UE configuration |
| --- | --- |
| 0 | UE is not configured with any parameters from this table |
| 1 | UE is configured for Multimedia Priority Service (MPS). |
| 2 | UE is configured for Mission Critical Service (MCS). |
| 3-10 | Reserved for future use |
| 11 | Access Class 11 is configured in the UE. |
| 12 | Access Class 12 is configured in the UE. |
| 13 | Access Class 13 is configured in the UE. |
| 14 | Access Class 14 is configured in the UE. |
| 15 | Access Class 15 is configured in the UE. |

Also the special access classes (i.e. access classes 11 to 15) are for the following use, as defined in TS 22.011 [3]:

Class 15 - PLMN Staff; -"- 14 - Emergency Services; -"- 13 - Public Utilities (e.g. water/gas suppliers); -"- 12 - Security Services; -"- 11 - For PLMN Use.

| Access Category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 | All | MO signalling resulting from paging |
| 1 | UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN. | All except for Emergency, or MO exception data |
| 2 | All | Emergency |
| 3 | All except for the conditions in Access Category 1. | MO signalling on NAS level resulting from other than paging |
| 4 | All except for the conditions in Access Category 1. | MMTEL voice |
| 5 | All except for the conditions in Access Category 1. | MMTEL video |
| 6 | All except for the conditions in Access Category 1. | SMS |
| 7 | All except for the conditions in Access Category 1. | MO data that do not belong to any other Access Categories |
| 8 | All except for the conditions in Access Category 1 | MO signalling on RRC level resulting from other than paging |
| 9 | All except for the conditions in Access Category 1 | MO IMS registration related signalling |
| 10 | All | MO exception data |
| 11-31 |  | Reserved standardized Access Categories |
| 32-63 | All | Based on operator classification |

For the Access Identity which is about device type, it can be observed from the table and RRC procedure (i.e. subclause 5.3.14.5 in RRC) that the current Access Identity is designed for the high priority access (also observed in the R2-2009010 [4]). However, we cannot say that RedCap UE should be treated as higher priority than normal UEs, and thus to introduce a new Access Identity for RedCap UE makes little sense.

Proposal 1:	No new Access Identity is introduced for the RedCap UE.

Furthermore, for the Access Category which is about service type, it can be observed from the table above that all the services are applicable to RedCap UEs. Hence, unless a new service type for RedCap UE is identified, no additional Access Category needs to be introduced.

Proposal 2:	No new Access Category is introduced for the RedCap UE, unless a new service for the RedCap UE is identified.

# 3	Conclusion

Proposal 1:	No new Access Identity is introduced for the RedCap UE.

Proposal 2:	No new Access Category is introduced for the RedCap UE, unless a new service for the RedCap UE is identified.

# 4	References

[1]	RAN2-112e - R16 eMIMO-CLI-PRN-RACS - R17 NTN-REDCAP (Sergio)_EOM.docx

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 22.011: "Service accessibility"

[4]	R2-2009010	UAC for RedCap UE	Intel Corporation, Facebook