3GPP TSG-RAN WG2 Meeting #113-e	R2-2100215

Online, 25th Jan - 5th Feb, 2021

Agenda item:	8.5.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on the time synchronisation enhancements

Document for:	Discussion and Decision

# 1. Introduction

RAN2 has evaluated the synchronicity budget for Uu interface and sent it to RAN1 in [1] and left it up to RAN1 to finalize the PDC options for the time synchronisation enhancements.

This paper further discusses the general issues about the time synchronisation enhancements.

# 2. Discussion

## 2.1 Time synchronization differentiation issues

RAN2#112e meeting has agreed the synchronicity budget for Uu interface as tabulated below, where the UE-UE synchronicity budget could be from ±145ns to ±275ns in the control-to-control scenario.

| Scenario | Single Uu interface Budget |
| --- | --- |
| Control-to-Control | ±145ns to ±275ns |
| Smart Grid | ±795ns to ±845ns |

RAN1 has, in R16, concluded that the Uu synchronicity budget is no more than ±540ns for 15 kHz SCS and can be fulfilled without extra propagation delay compensation. For R17, the extra propagation delay compensation enhancement is necessary for the time synchronisation enhancement in at least the Control-to-Control scenario in [2].

Observation 1: The propagation delay compensation enhancements for R17 is necessary at least for Control-to-Control scenario.

RAN2 has sent the synchronicity budget for Uu interface to RAN1 and RAN1 can decide on which PDC options to adopt. No matter which PDC options RAN1 choose, there is one further question as:

- Whether gNB distinguish different time synchronization schemes for different UEs

RAN2 in R16 supports to send the network-side reference time to UE, and leave the PD compensation up to UE implementation (i.e. based on TA/2).  In R17, we understand it is better to only apply the R17 PDC for UE in the control-to-control scenario as the most demanding scenario.  This approach can avoid the additional Uu signalling overhead and processing cost for the UE when the synchronicity budget is relaxed.

Proposal 1: RAN2 discuss whether UEs involved in different scenarios shall be differentiated regarding PDC scheme.

If gNB uses different time synchronization schemes for UEs in different scenarios, it can be known from SMF which is aware of that UE is in the Control-to-Control scenario or not from the Application Function entity. That is to inform the NG-RAN that UE having strict Uu synchronicity budget or not by the core network. NG-RAN can decide whether to apply the R17 time synchronisation enhancements or to apply the R16 time synchronisation scheme.  Another approach could be to inform the NG-RAN through UE assistance information, though it might be difficult for UE to understand it is Control-to-Control scenario or not.

Proposal 2:  The assistance information for gNB to determine the R17 time synchronisation scheme for a UE is needed from 5GC.

If RAN2 agrees proposal 2. A LS to SA2 is needed.

Proposal 3: Send a LS to SA2 to inquire whether 5GC can provide an assistance information for gNB to determine the R17 timing time synchronisation scheme for a UE.

## 2.2 Issues for time synchronisation scheme during handover

Considering the duration of handover may be hundreds of milliseconds, the 5G time maintained by the UE might lose its accuracy if no 5G reference time is received during such a long period. RAN2#111-e meeting has discussed this potential issues where UE may lose the accuracy of its 5G reference time. Some enchantments are proposed, such as, the reference time of the target cell is included in the handover message which is sent to UE.

However, even UE receives the reference time in the handover command, UE cannot use it until UE accesses the target cell and receives the TA command for the propagation delay compensation. During a legacy handover, UEAssistanceInformation message, contained in the HO request message, will be transferred to the target gNB by the source gNB. The target gNB will be made aware of UE’s preference for 5G reference time, and send the time synchronization message to UE after receiving the “HANDOVER SUCCESS” message. We think this enhancement won’t bring much gain compared with the legacy procedure.

Observation 2: The propagation delay compensation enhancements for mobility scenario can be fulfilled through existing mechanisms.

Proposal 4: No need to indicate the reference time of the target cell to the UE by the source gNB during the handover.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: The propagation delay compensation enhancements for R17 is necessary at least for Control-to-Control scenario.

Observation 2: The propagation delay compensation enhancements for mobility scenario can be fulfilled through existing mechanisms.

Proposal 1: RAN2 discuss whether UEs involved in different scenarios shall be differentiated regarding PDC scheme.

Proposal 2:  The assistance information for gNB to determine the R17 time synchronisation scheme for a UE is needed from 5GC.

Proposal 3: Send a LS to SA2 to inquire whether 5GC can provide an assistance information for gNB to determine the R17 timing time synchronisation scheme for a UE.

Proposal 4: No need to indicate the reference time of the target cell to the UE by the source gNB during the handover.

# 5. Reference

- R2-2010837, Reply LS on propagation delay compensation enhancements, RAN WG2.

- R2- 2008855, Discussion on enhancements for support of time synchronization, Huawei, HiSilicon

# 6. Annex

Title:	Draft LS on network-aware of the synchronization requirement

Response to:	-

Release:	Rel-17

Work Item:	NR_IIOT_URLLC_enh

Source:	[to be RAN2]

To:	SA2, RAN3

Cc:

Contact Person:

#### Name:

E-mail Address:

1. Overall Description:

RAN2 discussed the need to differentiate UE in different scenarios and agreed that the CN can inform the NG-RAN on UE scenarios and/or Uu synchronicity budget to assist NG-RAN to choose the propagation delay compensation scheme, especially between control-to-Control scenario and other scenarios.

2. Actions:

To SA2, RAN3:

ACTION: 	RAN2 respectfully asks SA2 and RAN3 to take the above into account and provide feedback on the requirement and the assistant information scheme from 5GC to NG-RAN to differentiate the Control-to-Control scenario from other scenarios.

3. Date of Next TSG-RAN WG3 Meetings:

3GPP RAN2#113bis-e		12 April – 20 April			Electronic Meeting

3GPP RAN2#114-e			19 May – 27 May			Electronic Meeting