3GPP TSG-RAN WG2 Meeting #113-e	R2-2100218

# Online, 25th Jan - 5th Feb, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1009 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for DG and CG UL skipping with UCI overlap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to current specification [38.321-g30], the MAC entity shall not generate a MAC PDU if the following conditons are satisfied: - the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant; and - there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and - the MAC PDU includes zero MAC SDUs; and - the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR. But in RAN1#102-e and RAN1#103-e meetings, RAN1 agreed for DG and CG respectively that, when there is PUCCH carrying UCI overlapping with a set of PUSCHs, the PUSCH with UCI multiplexing from the set cannot be skipped. At the moment, the (later) CG agreement is only valid when lch-basedPrioritization is not configured and there is a single PHY priority for UL transmissions. Corresponding LSs were sent to RAN2 in R1-2007338 and R1-2009772: Agreement For UL skipping of dynamic UL grant in non-CA and CA case, when there is PUCCH carrying UCI overlapping with a set of PUSCHs, the PUSCH with UCI multiplexing from the set cannot be skipped. MAC generates MAC PDU for the PUSCH and the UCI is multiplexed on the PUSCH. Based on above agreements, RAN1 in principle agreed the corrections for Rel-16 TS 38.214 (R1-200xxxx), assuming that RAN2 will update the Rel-16 sepcification TS 38.321 corresponding to the above agreement so that UE generates the MAC PDU for the PUSCH with UCI multiplexing.. Agreement: For the case (Case 1-2) where only one or more CG PUSCHs overlapping with PUCCH In Rel.16, for CA and non-CA case, when Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, and when PUSCH repetition is not applied, in case of one or more CG PUSCHs overlapping with UCI and there is no DG PUSCH overlapping with the UCI and there is no DG PUSCH overlapping with the one or more CG PUSCHs, the CG PUSCH with UCI multiplexing from the one or more CG PUSCHs cannot be skipped. MAC generates MAC PDU for the CG PUSCH and delivers the MAC PDU to PHY and the UCI is multiplexed on the CG PUSCH.. And both LSs ask RAN2 to take into account the above agreements and update Rel-16 TS 38.321 to support Rel-16 PUSCH skipping. |
| Summary of change: |  | Add to the conditions for the MAC entity to not generate a PDU the condition that no UCI is multiplexed by lower layers in the PUSCH for DG and CG cases. Impact analysis Impacted 5G architecture options: Standalone and Non-Standalone Impacted functionality: Resources allocation Inter-operability: If the network is implemented according to the CR and the UE is not, the network may not receive UCI. If the UE is implemented according to the CR and the network is not, the network can decode UCI via PUSCH. So there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is mismatch between RAN1 and RAN2 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall not generate a MAC PDU for the HARQ entity if the following conditions are satisfied:

-	the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant; and

-	there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

-	the MAC PDU includes zero MAC SDUs; and

-	the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR; and

-	there is no UCI multiplexed by lower layers in the PUSCH transmission as specified in TS 38.213 [6] when the grant indicated to the HARQ entity was addressed to a C-RNTI and the MAC entity is configured with skipUplinkTxDynamicWithoutUci; and

-	there is no UCI multiplexed by lower layers in the PUSCH transmission as specified in TS 38.213 [6] when the grant indicated to the HARQ entity is a configured uplink grant not part of a bundle and the MAC entity is configured with skipUplinkCgWithoutUci and lch-basedPrioritization is not configured and there is a single PHY priority index configured for configured grant transmission.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	C-RNTI MAC CE or data from UL-CCCH;

-	Configured Grant Confirmation MAC CE or BFR MAC CE or Multiple Entry Configured Grant Confirmation MAC CE;

-	Sidelink Configured Grant Confirmation MAC CE;

-	LBT failure MAC CE;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for BSR, with exception of BSR included for padding;

-	Single Entry PHR MAC CE or Multiple Entry PHR MAC CE;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among Configured Grant Confirmation MAC CE, Multiple Entry Configured Grant Confirmation MAC CE, and BFR MAC CE is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over transmission of NR sidelink communication.

End of change