3GPP TSG-RAN WG2 Meeting #113-e	                                                                        R2-2100325

Online, January 25th – February 5th, 2021

Title:	[draft] LS to RAN4 on measurement in connected mode for NB-IoT

Response to:

Release:	Rel-17

Work Item:	NB_IOTenh4_LTE_eMTC6-Core

Source:	ZTE Corporation [to be RAN2]

To:	RAN4

Cc:

Contact Person:

Name:	Ting Lu

E-mail Address:	lu.ting@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

For Rel-17 WID of additional enhancements for NB-IoT, RAN2 has some discussion on introducing measurement in connected mode for NB-IoT.

RAN2 has made some progress on the following issues:

Issue#1: For NB-IoT measurement in connected mode, RAN2 assumes a neighbour cell is known if it has been measured by the UE and not a long time has passed or the channel quality hardly changes.

Issue#2: For NB-IoT measurement in connected mode, RAN2 has the following assumption on definitions for connected mode measurements:

- The neighbor cell measurements performed by the UE in connected mode are intra-frequency measurements when anchor carrier of the current cell and anchor carrier of the neighbor cell operates on the same carrier frequency.

- The neighbor cell measurements performed by the UE in connected mode are inter-frequency measurements when anchor carrier of the neighbor cell operates on a different carrier frequency, compared to anchor carrier of the current cell.

Issue#3: For NB-IoT UE in connected mode, RAN2 assume UE can perform one shot measurement, e.g., once the measurement is triggered, the measurement is performed until the measurement result is got. Unless the measurement results turn to invalid, the measurement wouldn’t be performed again.

Issue#4: For one shot measurement in connected mode, RAN2 assume continuous measurement in a one-shot measurement may be infeasible in some cases.

Issue#5: For one shot measurement in connected mode, RAN2 assume the minimum length of a measurement occasion can be with 21ms, and in some scenarios 6ms for Frame structure type 1 and 7ms for Frame structure type 2.

2. Actions:

ACTION to RAN4:

RAN2 respectfully asks RAN4 to take the above into account and start the corresponding checks:

- For issue#1, RAN4 can further determine the time duration in which the measurement results can be thought as valid and/or the acceptable channel quality variation range.

- For issue#2, RAN4 can check the feasibility of connected mode intra-frequency measurements and inter-frequency measurement definitions.

- For issue#3, RAN4 can check whether one-shot neighbour cell measurement is feasible/better for NB-IoT UE in connected mode.

- For issue#4, RAN4 can check whether UE can perform measurement occasionally for the neighbour cell that operates on a different carrier frequency compared with the downlink dedicated carrier.

- For issue#5, RAN4 can check whether the minimum length of a measurement occasion can be 21ms, and in some scenarios whether it can be 6ms for Frame structure type 1 and 7ms for Frame structure type 2. Moreover, whether interval between the available measurement occasions is needed.

3. Date of Next TSG-RAN2 Meetings:

3GPPRAN2#113bis-e 	12-20 April 2021	E-meeting

3GPPRAN2#114-e 	19-27 May 2021	E-meeting