3GPP TSG-RAN WG2 Meeting #113-e                                                    R2-2100330

Electronic, 25 January – 5 February 2021‎‎

Source:	CATT

Title:	Discussion on geographical fixed CGI

Agenda Item:	8.10.1

Document for:	Discussion and Decision

Introduction

In RAN2#112-e meeting, there was an email discussion [Offline-116][NTN]Reply LS to RAN3 about the Cell ID as User Location information on the  NG/N2 interface corresponds to a fixed geographical area, and the Tracking Area is coupled with geographical area.

In the LS [R2-2011041], RAN3 provided the following two options:

Option A: On Uu, SIB content corresponds to momentary coverage area of a satellite beam related to the geographically fixed areas of TAs/Cells - irrespective of whether the beam is fixed or moving.

Option B: The cell ID used on Uu SIB content (and probably on Xn) are decoupled from cell ID used on NG(N2). The respective mapping is performed in RAN. This requires gNB to acquire the UE’s location information.

RAN2 discussed the two options above, and had no consensus yet.

In this contribution, we will further analyze the two options and provide corresponding observations and proposals.

Discussion

In the Option A, geographical fixed Cell ID is been used in N2 and Uu interfaces. UE would need to obtain the geographical fixed Cell ID via the GNSS capability and the boundary of each cells.

Here’re two options to notify UE the boundary of geographical fixed cells:

Option A1: Include the Cell ID list and the boundary of the cell for each Cell ID in the SIB

In this situation, UEs select the Cell ID based on the boundary information in SIB and its GNSS capability. Currently, Cell ID is broadcasted in SIB1. If RAN wants to broadcast the Cell ID list and the boundary of each Cell in system information, it will occupy too much space. Whether it’s feasible to broadcast the boundary info of each geographical fixed cell in SIB1 should be further investigated, maybe new SIB(s) are needed.

- Broadcast the boundary info of each geographical fixed cell may occupy too much space in SIB1.

Option A2: The boundary of each planned Cell ID is pre-configured in the UE

UEs select the cell ID based on the GNSS and the per-configured information about the boundary of Cell ID. In this case, RAN doesn’t need to broadcast the geographical fixed Cell IDs and corresponding boundary info to UE. However, if the network updates the information of geographical fixed Cell IDs, how to update the pre-configured info for the UEs? Especially for the UEs which are switched off for a long time. Thus, there would be some mismatch between the UE and the network on the boundary info of the Cell IDs. E.g. a geographical fixed area X was mapped to Cell ID X, now it’s split to Cell ID X1, X2, UEs in this area still think they’re in cell X. Whether it will cause any problem should be further investigated.

- There would be some mismatch between UE and network on the boundary info of each geographical fixed cell in case the pre-configured info is updated in network but not in UE.

Based on the above analysis, the SIB space is limited in Option A1, and UEs cannot select the Cell ID after network updating the boundary info of geographical fixed Cells in Option A2.

In case of earth moving cell, the real physical cell is moving on earth, it assumes the measurement, cell selection, cell reselection, handover should base on the real physical cell, not the geographical fixed cell (virtual cell). We do not see any necessary to broadcast the geographical fixed cell ID(s) to UE. How to use the geographical fixed Cell IDs in NG-RAN and UE is not clear.

- What’s the usage and benefit to use the geographical fixed Cell IDs in Uu interface is not clear.

In option B, geographically fixed areas Cell ID is only used in N2, when NG-RAN report the UE location information to AMF.  In Uu interface, the existing CGI is used.

In this case, the Cell ID broadcast in SIB has different meaning with the Cell ID used in NG.

To satisfy the requirement of SA2 and RAN3, NG-RAN can obtain the geographical fixed Cell ID of the UE by the following two ways:

Option B1: RAN obtains the geographical fixed Cell ID based on positioning mechanism.

In this case, RAN obtains the UE location info via network based positioning mechanism or UE reporting, then RAN maps the UE location info to a geographical fixed Cell ID and reports the CGI to AMF.  There’s no impact to Uu interface, Xn interface and other RAN behaviors.

Option B2: RAN obtains the geographical fixed Cell ID regarding to the momentary area of moving cell.

If B1 is not feasible, i.e. RAN could not get the detail GNSS info of the UE, RAN could map the momentary coverage of UE’s serving cell to the geographical fixed Cell ID when report the UE location to AMF. How to do such kind of mapping is up to the RAN implementation, e.g. NG-RAN may do the mapping by using the center of the momentary coverage of UE’s serving cell, may provide an invalid CGI to AMF, etc. Detail design is pending to RAN3 and SA2.

Above all, we see the option B (B1+B2) can satisfy the requirement of SA2 without any impact to Uu. Thus option B is preferred as it could satisfy the requirement of SA2 without any impact to Uu.

Base on the discussion above, we could have the following proposal:

- It’s suggested to go for option B, NG-RAN does the mapping and reports geographical fixed Cell ID to CN without any impact to Uu interface.

Base on above discussion, it’s proposed to reply the LS to RAN3 and SA2 to indicate our decision. The draft LS is in [3].

- RAN2 is request to discuss and agree the draft LS reply to RAN3 and SA2 in [3].

Conclusion

In section 2, we made the following observations:

- Broadcast the boundary info of each geographical fixed cell may occupy too much space in SIB1.

- There would be some mismatch between UE and network on the boundary info of each geographical fixed cell in case the pre-configured info is updated in network but not in UE.

- What’s the usage and benefit to use the geographical fixed Cell IDs in Uu interface is not clear.

Based on the discussion in section 2, we propose the following:

- It’s suggested to go for option B, NG-RAN does the mapping and reports geographical fixed Cell ID to CN without any impact to Uu interface.

- RAN2 is request to discuss and agree the draft LS reply to RAN3 and SA2 in [3].

Reference

- R2-2010793 [Offline-116][NTN]Reply LS to RAN3

- R2-2011041 Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G

- R2-2100331 Reply LS on geographical fixed Cell ID