3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2100346

Online, 25thJanuary – 5th February, 2021

Agenda Item:	8.10.3.3

Source:	Ericsson

Title:	Connected mode aspects for NTN

Document for:	Discussion, Decision

# 1	Introduction

A new WI on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86, with an updated WID approved at RAN#88 [1]. The WI aims to specify the following control plane enhancements:

- Idle mode:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

  - Definition of NTN (satellite/HAPS) cell specific information in SIB

- Connected mode

  - Enhancement necessary to take into account location information (UE & Satellite/HAPS) and/or ephemeris in determining when to perform hand-over, in order to have a high degree of hand-over control for hand-over robustness and coverage management.

  - Enhancement to existing measurement configurations to address absolute propagation delay difference between satellites (e.g. SMTC measurement gap adaptation to the SSB/CSI-RS measurement window) [RAN2/4].

- Service continuity for mobility from TN to NTN and from NTN to TN systems (to be addressed when connected mode mobility has sufficiently progressed)

- Identify potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3]

Agreements related to connected mode from RAN2#102:

Agreements:

- RAN2 to consider the case where gNB is co-located at the GW with higher priority.

- RAN2 will continue working with the assumption that service link switch implies L3 mobility (meaning that at least in case the SSBs are on the same sync raster point the PCIs need to be different). Check if an LS to RAN1 asking for feasibility of having same PCI as well can be agreed

Agreements

- Reconfiguration with sync is the baseline for connected mode mobility in NTN (the use of legacy RLF and re-establishment mechanism are not excluded)

- The CHO can be used in NTN for both moving cell and fixed cell scenarios, and the CHO procedure and execution condition defined in Rel-16 is the baseline for NTN CHO.

3.	NTN specific CHO execution condition can be further discussed.

4.	The existing measurement framework (e.g. measurement configuration, execution and reporting) is the baseline, and all the existing measurement criteria and event can be used in NTN. Support for new measurement is not excluded.

5.	Legacy SSB periods (as in TN) shall be supported in NTN

Agreements via email - offline 105:

- Time or timer based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario.  FFS on how to configure the time or timer based CHO triggering event. Also FFS how to consider the feeder/service link switch timing.

- DAPS HO for NTN is de-prioritized in this release.

- Location based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario. FFS on how to configure the location based CHO triggering event. FFS if location based CHO triggering event only (not in combination with other events) can also be considered.

- The Location-based measurement event, in combination with the existing measurement event in NR, should be supported in NTN for both moving cell and fixed cell scenarios. FFS on how to configure the location based measurement event.

Agreements via email - offline 106:

- RAN2 understanding that UE shall not be forced to detect the SSB burst outside the corresponding configured SMTC window in NTN, just like the principle in TN.

Agreements:

- SMTC and gap configuration in NTN are configured based on the timing of PCell

- RAN2 can first identify the scenarios and discuss how serious the impact is before addressing any enhancement for SMTC configuration in NTN.

- RAN2 can’t assume that the network will always have UE accurate location info for SMTC window configuration in NTN

- UE along with the network in NTN should also have the same understanding of the timing, including the timing for measurement gap, to avoid any un-synchronized scheduling between UE and the network, just like the way we have in TN

# 3 Conditional Handover (CHO) and location based measurement event

As CHO is defined in Rel-16, the UE can be configured with multiple candidate target cells where each candidate target cell may be configured with up to two CHO trigger conditions, A3 and A5 measurement based events. The CHO trigger condition is autonomously evaluated by the UE, and if two CHO trigger conditions are configured, the UE may only trigger the handover if both CHO trigger conditions are fulfilled for one of the candidate target cells.

This is expected to work well when CHO is configured in a terrestrial network where the cells are characterized by a clear difference in signal strength between the cell centre and the cell edge. But for a CHO configuration in an NTN deployment, it is not obvious the CHO procedure as defined in Rel-16 will always work if the time/timer based and location based trigger conditions are added as additional trigger conditions to the already existing Rel-16 CHO trigger conditions and the CHO procedure is kept unchanged.

In current Rel-16 CHO procedure, one or two CHO trigger conditions may be configured to the UE.

If two CHO trigger conditions are configured, the UE may only trigger a CHO to a candidate target cell if both trigger conditions are fulfilled for that cell.

Only relying on a time/timer or a location based event to trigger a CHO, without assessing the signal strength/quality of the candidate target cell, will not be sufficient in many NTN handover scenarios since this could easily end up in unpredictable behavior such as subsequent handovers or RLF. To avoid this, the time/timer or location based event need to be configured with a signal strength/quality measurement based event to secure a CHO to a ‘safe’ candidate target cell. The existing CHO events A3 and A5 can possibly serve as signal strength/quality measurement event, but it should also be considered if the A4 event (Neighbour becomes better than threshold) should be defined as an additional CHO event.

A4 event only compares neighbor to a threshold and not to serving cell thus it may be useful for NTN.

RAN2 to considered if the A4 event (Neighbour becomes better than threshold) should be defined as an additional CHO event.

But assuming that the network can predict the time when the UE needs to perform the handover to a neighbor cell (e.g. when a periodic feeder/service link switch is to be performed), the time (or timer) triggering the CHO will typically be set in time before the signal strength/quality of the serving cell becomes too weak, but late enough so that the signal strength/quality of the target cell is guaranteed, thus a single time/timer event triggering a CHO to a candidate target cell should not be precluded.

However, in a situation where the configured time/timer is, for whatever reason, cannot be accurately set for some reason(e.g. if the timer is set too long delaying the handover beyond a point where the signal strength/quality of the serving cell becomes too weak), the measurement based events (A3 or A5) can assist as a ‘last resort’ to trigger a handover to the target cell. This could be useful for example in an earth fixed cell scenario where it may be difficult to pinpoint the exact point in time when to trigger a handover if the UE speed and the direction of the UE movement is not known to the network. Potentially, it may also be useful at an initial stage of an NTN deployment when the network parameters are not fully adjusted.

Similarly, if a location based trigger condition is configured to the UE and the present location in relation to the serving cell centre or to the target cell centre is temporarily unknown or incorrect thus misleading the UE, the measurement based trigger conditions can be used as a fallback solution to trigger a CHO.

Considering the large variety of handover scenarios and use cases expected in NTN deployments, RAN2 should discuss if changes to the CHO evaluation logic is needed when multiple CHO trigger conditions are configured to the UE.

Following options can be discussed:

- UE evaluates all CHO events simultaneously but only when all CHO events are fulfilled for a candidate target cell, a CHO is triggered.

- UE evaluates all CHO events simultaneously and when one CHO event fulfills the execution condition for a candidate target cell, a CHO is triggered.

- UE evaluates one CHO event at a time in a hierarchical order, e.g. when the first CHO event is fulfilled, the UE starts to evaluate the second CHO event. Only when both CHO events are fulfilled for a candidate target cell, a CHO is triggered.

The first option listed above is the legacy procedure while the two following require changes to the procedural text, or a new procedural text. As an example, TP1 in Appendix gives procedural text change for Option 2. Option 3 would likely require alternative procedural text section.

Additionally, RAN2 should discuss whether the current limit of two CHO events per candidate target cell is enough or if the number of CHO events needs to be increased.

RAN2 to discuss how the conditional HO should be triggered in NTN when location and/or time/timer based trigger is configured possibly in addition to the RSRP based event.

RAN2 to discuss how many CHO events per candidate target cell can be configured for NTN CHO.

## 3.1 Event definitions for location and time/timer

Location was agreed to be adopted for both CHO trigger condition as well as measurement report trigger condition. As the event definition is likely similar we discuss these commonly here.

In addition to define the location trigger with respect to source cell, or as absolute line on Earth surface, it is also possible to define the event entry condition with respect to the target cell such that event entry condition is fulfilled when UE steps in the area. Further, in addition to circle, especially for fixed beam case, an elliptic shape might describe the actual area better. The radius, or in case of elliptic shape one or both of the radius may change as the satellite moves/elevation angle changes.

In addition to the source cell, it is also possible to define the event entry condition with respect to the target cell such that event entry condition is fulfilled when UE steps in the area.

In addition to circle, especially for fixed beam case, an elliptic shape might describe the actual area better.

RAN2 to discuss further the most suitable way of defining the entry condition for the event based measurement trigger and CHO.

In addition, the event definition in TS 38.331 Clause 5.5.4 needs to consider whether TTT and hysteresis are implemented and whether a threshold with offset is used. Our proposal is to use TTT and hysteresis in order not to trigger the measurement report or the CHO if UE does not stay beyond the threshold for a defined amount of time and to use hysteresis in order to avoid the ping pong effect. Further, in expressing the location, a threshold with offset should be used. That is to use threshold to give the distance from the reference location with a certain accuracy and then use offset to fine tune value.

The location event includes hysteresis and TTT

The trigger is configured with a threshold in MeasConfig and offset in ReportConfigNR as shown in TP3.

In addition to event based trigger, the network may configure a UE to report its location to the network periodically. If this period is sparse enough it avoids UE to keep constant up to date of its location. Disadvantage is that it consumes uplink resources. Regardless, it provides an option for the UE capability with respect to maintaining up to date location.

Periodic reporting of UE’s location may provide an option where UE does not need to maintain location info constantly.

RAN2 to discuss the feasibility of periodic location reporting as an addition to the event triggered based.

## 3.1.1 Time/timer based event

For the time/timer based event, it should be discussed how the feeder/service link switch scenario is supported. For example, is it possible to support with one time/timer based event the case that current serving cell ceases to serve the area of the cell. One option is to configure a UTC time as the CHO trigger condition. Then, as time in this Universe is understood only to go forward, it seems we need only an entering condition for the event. It is likely that hysteresis and TTT are a bit useless here as well.

RAN2 to adopt absolute time as CHO trigger condition.

The time event does not include TTT or hysteresis.

The time event has entry condition only.

# 4 SMTC and measurement gaps

The SMTC configuration helps the UE to find the SSB but does not have implications to the network. Then, if the window size is extended it only means UE has less information from where to measure. Thus, this option does not seem worth the specification effort. We can discuss whether additional information is useful at the UE or whether UE would deduce itself when to measure. Then again, when measurement gaps are configured it means that during the gap period UE does not need to listen to PDCCH of the serving cell needs to be considered. That is, gap window cannot be too long and network needs to be aware when UE is not monitoring PDCCH. There may also be need for the UE to inform network if current gap configuration is not enough.

For both SMTC and meas gap design extending the window length does not seem fruitful.

For SMTC, the main consideration should be what information would be actually useful for the UE.

For measurement gap consideration, the total gap length should be minimized in order to minimize the time UE does not listen to the PDCCH.

RAN2 should aim to minimize the total measurement gap length for NTN.

# 5 RACH-less Handover

RACH-less handover was introduced as part of the mobility enhancements in LTE Rel-14 and consists of that Msg1 and Msg2 are skipped in the target cell. The UE’s first transmission in the target cell is instead the message confirming the completion of the handover, i.e. RRCConnectionReconfigurationComplete in LTE and RRCReconfigurationComplete in NR, i.e. the RRC message which is normally included in Msg3 in a regular RACH-based handover.

UL transmission resources for UE’s first message in the target cell are allocated either through pre-allocated UL grants or UL grants dynamically allocated through PDCCH in the target cell without preceding scheduling request due to the missing random access procedure in the target cell. If pre-allocated UL grants are configured, these are periodic UL transmission resources allocated in the RRCConnectionReconfiguration or RRCReconfiguration message constituting the HO Command. The pre-allocated UL grants potentially achieves shorter interruption, since the PDCCH allocation adds some additional delay, but PDCCH allocation is more flexible.

Since the random access procedure, and therefore also the Random Access Response message is skipped in the target cell, the target gNB cannot determine a precise timing advance (TA) for the UE. Hence, RACH-less HO is limited to scenarios where the TA is known in advance. In practice, this means scenarios where the TA in the target cell is the same as in UE’s source cell or when the target cells is so small that the TA can be assumed to always be 0. Since the coverage area of a satellite is likely to include several cells, all intra satellite HO will experience the same TA value and thus even in this case the RACH-less HO procedure could be used. The assumption that all NTN UE´s are GNSS capable could allow the UE to estimate its TA to the source satellite but also the TA to the target cell in an inter satellite HO and even in this case, the use of RACH-less HO with a TA based on UE estimates should be considered for NTN.

Even with a PRACH configuration that maximizes the available PRACH opportunities in a cell, the collision rate will be high. Adding attach attempts from other UEs than the ones that are doing HO, the collision rate will increase even further. This means that in order to support mobility, the signaling needs to be reduced.

It is believed that the RACH capacity will be limited even in sparsely populated areas due to the large supported cells in NTN. RACH-less will offload the PRACH resources and thus reduce the collision rate. It also has the potential to reduce the interruption time since it is possible to use dense pre-allocated grants in the HO command.

Our companion contribution [8] discusses further details of the RACH-less HO and it is preferred that detailed discussion takes place under UP agenda rather than CP agenda.

The assumption that all NTN UE´s are GNSS capable could allow the UE to estimate its TA to the source satellite but also the TA to the target cell in an inter satellite HO and even in this case, the use of RACH-less HO with a TA based on UE estimates seems feasible.

The PRACH collision rate will be high in NTN systems and ways to limit this especially for cases where there is expected to be large number of HOs like feeder/service link switch, ways to limit this is definitely needed.

RACH capacity will be limited as seen during SI phase and the available RACH capacity needs to consider, initial access, L3 mobility as well as L1 beam management(BFR) and RLF caused access attempts.

RAN2 should support RACH-less HO in Rel-17 WI for NTN.

Further discuss the details under user plane agenda item.

# 6 Conclusion

In the previous sections we made the following observations:

Observation 1	In current Rel-16 CHO procedure, one or two CHO trigger conditions may be configured to the UE.

Observation 2	If two CHO trigger conditions are configured, the UE may only trigger a CHO to a candidate target cell if both trigger conditions are fulfilled for that cell.

Observation 3	A4 event only compares neighbor to a threshold and not to serving cell thus it may be useful for NTN.

Observation 4	In addition to the source cell, it is also possible to define the event entry condition with respect to the target cell such that event entry condition is fulfilled when UE steps in the area.

Observation 5	In addition to circle, especially for fixed beam case, an elliptic shape might describe the actual area better.

Observation 6	Periodic reporting of UE’s location may provide an option where UE does not need to maintain location info constantly.

Observation 7	For both SMTC and meas gap design extending the window length does not seem fruitful.

Observation 8	For SMTC, the main consideration should be what information would be actually useful for the UE.

Observation 9	For measurement gap consideration, the total gap length should be minimized in order to minimize the time UE does not listen to the PDCCH.

Observation 10	The assumption that all NTN UE´s are GNSS capable could allow the UE to estimate its TA to the source satellite but also the TA to the target cell in an inter satellite HO and even in this case, the use of RACH-less HO with a TA based on UE estimates seems feasible.

Observation 11	The PRACH collision rate will be high in NTN systems and ways to limit this especially for cases where there is expected to be large number of HOs like feeder/service link switch, ways to limit this is definitely needed.

Observation 12	RACH capacity will be limited as seen during SI phase and the available RACH capacity needs to consider, initial access, L3 mobility as well as L1 beam management(BFR) and RLF caused access attempts.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to considered if the A4 event (Neighbour becomes better than threshold) should be defined as an additional CHO event.

Proposal 2	RAN2 to discuss how the conditional HO should be triggered in NTN when location and/or time/timer based trigger is configured possibly in addition to the RSRP based event.

Proposal 3	RAN2 to discuss how many CHO events per candidate target cell can be configured for NTN CHO.

Proposal 4	RAN2 to discuss further the most suitable way of defining the entry condition for the event based measurement trigger and CHO.

Proposal 5	The location event includes hysteresis and TTT

Proposal 6	The trigger is configured with a threshold in MeasConfig and offset in ReportConfigNR as shown in TP3.

Proposal 7	RAN2 to discuss the feasibility of periodic location reporting as an addition to the event triggered based.

Proposal 8	RAN2 to adopt absolute time as CHO trigger condition.

Proposal 9	The time event does not include TTT or hysteresis.

Proposal 10	The time event has entry condition only.

Proposal 11	RAN2 should aim to minimize the total measurement gap length for NTN.

Proposal 12	RAN2 should support RACH-less HO in Rel-17 WI for NTN.

Proposal 13	Further discuss the details under user plane agenda item.

# 7 References

RP-201256, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #88e, June 2020.

TR 38.821, “Solutions for NR to support Non-Terrestrial Networks (NTN),” V16.0.0.

TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks,” V15.3.0

R2-10xxxx, Earth fixed and moving beams for NTN, Ericsson

R2-2007537, “NTN scope, scenarios, architecture, and requirements,” 3GPP TSG RAN WG2 Meeting #111, Ericsson, August 2020.

Aida, S., Patzelt, T., Leushacke, L., Kirschner, M., and Kiehling, R., ‘Monitoring and Mitigation of Close Proximities in Low Earth Orbit’, 21st International Symposium on Space Flight Dynamics – 21st ISSFD, Toulouse, France, 2009.

R1-2005502, “On UL time and frequency synchronization enhancements for NTN,” 3GPP TSG RAN WG1 Meeting #102, Ericsson, August 2020.

R2-21xxxxx, “On Random Access in NTNs” Ericsson

# Appendix

------------------------------------ start TP 1------------------------------------------------------------

##### 5.3.5.13.4	Conditional reconfiguration evaluation

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync in the received condRRCReconfig to be applicable cell;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the condExecutionCond associated to condReconfigId:

3>	if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4>	consider the event associated to that measId to be fulfilled;

3>	if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4>	consider the event associated to that measId to be not fulfilled;

2>	if the condTriggerConfig within the VarConditionalReconfig includes the EventEvaluationConfig:

3>	if the EventEvaluationConfig is set to anyEvent:

4>	if one event associated to any measId within condTriggerConfig for a target candidate cell within the stored condRRCReconfig is fulfilled:

5>	consider the target candidate cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

5>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

2>	else if event(s) associated to all measId(s) within condTriggerConfig for a target candidate cell within the stored condRRCReconfig are fulfilled:

3>	consider the target candidate cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

3>	initiate the conditional rconfiguration execution, as specified in 5.3.5.13.5;

NOTE:	Up to 2 MeasId can be configured for each condReconfigId. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold.

------------------------------------ end TP 1------------------------------------------------------------

------------------------------------ start TP 2------------------------------------------------------------

#### 5.5.4.xx	Event L1 (The UE location is beyond a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition L1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition L1-2, as specified below, is fulfilled;

Inequality L1-1 (Entering condition)

Inequality L1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the UE location, not taking into account any offsets.

Hys is the hysteresis parameter (i.e. l1-Hysteresis as defined within ReportConfigNR) for this event.

Thresh is the reference threshold parameter for this event given in MeasConfig(i.e. locationThreshRef as defined within MeasConfig).

Offset is the offset value to locationThreshRef to obtain the absolute threshold for this event. (i.e. l1-ThresholdOffset as defined within ReportConfigNR)

Ms is expressed in meters.

Thresh is expressed in the same unit as Ms.

NOTE 2:	The definition of Event L1 also applies to CondEvent L1.

#### 5.5.4.14	 CondEvent T1 (Absolute time has passed a threshold time)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition T1-1, as specified below, is fulfilled;

Inequality T1-1 (Entering condition)

Definition FFS

------------------------------------ end TP 2------------------------------------------------------------

------------------------------------ start TP 3------------------------------------------------------------

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                                                   OPTIONAL    -- Need R

]],

[[		locationThreshRef-r17	FFS		OPTIONAL	--Need ON

]]

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps.

locationThreshRef Reference location threshold for eventL1 in reportConfigNR. The locationThreshRef is a distance from a cell center reference location(definition FFS) --Editors note: definition and for cell center reference location is FFS and it is assumed it is given is SI or elsewhere thus not added here--

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

#### –	ReportConfigNR

The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event or of a CHO or CPC event. For events labelled AN with N equal to 1, 2 and so on, measurement reporting events and CHO or CPC events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

Event A6:	Neighbour becomes amount of offset better than SCell;

Event L1:	UE location becomes further than threshold;

CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell;

CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2;

CondEvent T1: Absolute time is more than configured threshold;

For event I1, measurement reporting event is based on CLI measurement results, which can either be derived based on SRS-RSRP or CLI-RSSI.

Event I1:	Interference becomes higher than absolute threshold.

ReportConfigNR information element

-- ASN1START

-- TAG-REPORTCONFIGNR-START

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16

}

}

ReportCGI ::=                     SEQUENCE {

cellForWhichToReportCGI          PhysCellId,

...,

[[

useAutonomousGaps-r16            ENUMERATED {setup}                OPTIONAL   -- Need R

]]

}

ReportSFTD-NR ::=                 SEQUENCE {

reportSFTD-Meas                  BOOLEAN,

reportRSRP                       BOOLEAN,

...,

[[

reportSFTD-NeighMeas             ENUMERATED {true}                                OPTIONAL,   -- Need R

drx-SFTD-NeighMeas               ENUMERATED {true}                                OPTIONAL,   -- Need R

cellsForWhichToReportSFTD        SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellId   OPTIONAL    -- Need R

]]

}

CondTriggerConfig-r16 ::=        SEQUENCE {

condEventId                      CHOICE {

condEventA3                      SEQUENCE {

a3-Offset                        MeasTriggerQuantityOffset,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

condEventA5                      SEQUENCE {

a5-Threshold1                    MeasTriggerQuantity,

a5-Threshold2                    MeasTriggerQuantity,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

...,

[[

condEventL1-r17							SEQUENCE {

l1-ThresholdOffset-r17			FFS,

l1-Hysteresis-17			    FFS,

timeToTrigger                   TimeToTrigger

},

condEventT1-r17							SEQUENCE {

t1XXX-r17			FFS

}

]]

},

rsType-r16                       NR-RS-Type,

...

}

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

eventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA3                                     SEQUENCE {

a3-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA5                                     SEQUENCE {

a5-Threshold1                               MeasTriggerQuantity,

a5-Threshold2                               MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA6                                     SEQUENCE {

a6-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

...,

[[

eventL1-r17							SEQUENCE {

l1-ThresholdOffset-r17			FFS,

l1-Hysteresis-17			    FFS,

timeToTrigger                   TimeToTrigger

]]

},

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                     MeasReportQuantity                                            OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

reportAddNeighMeas                          ENUMERATED {setup}                                             OPTIONAL,   -- Need R

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

useT312-r16                                 BOOLEAN                                                        OPTIONAL,   -- Need M

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]]

}

PeriodicalReportConfig ::=                  SEQUENCE {

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                    MeasReportQuantity                                             OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

useWhiteCellList                            BOOLEAN,

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL,   -- Need M

ul-DelayValueConfig-r16                     SetupRelease { UL-DelayValueConfig-r16 }                       OPTIONAL,   -- Need M

reportAddNeighMeas-r16                      ENUMERATED {setup}                                             OPTIONAL    -- Need R

]]

}

NR-RS-Type ::=                              ENUMERATED {ssb, csi-rs}

MeasTriggerQuantity ::=                     CHOICE {

rsrp                                        RSRP-Range,

rsrq                                        RSRQ-Range,

sinr                                        SINR-Range

}

MeasTriggerQuantityOffset ::=               CHOICE {

rsrp                                        INTEGER (-30..30),

rsrq                                        INTEGER (-30..30),

sinr                                        INTEGER (-30..30)

}

MeasReportQuantity ::=                      SEQUENCE {

rsrp                                        BOOLEAN,

rsrq                                        BOOLEAN,

sinr                                        BOOLEAN

}

MeasRSSI-ReportConfig-r16 ::=               SEQUENCE {

channelOccupancyThreshold-r16               RSSI-Range-r16         OPTIONAL   -- Need R

}

CLI-EventTriggerConfig-r16 ::=              SEQUENCE {

eventId-r16                                 CHOICE {

eventI1-r16                                 SEQUENCE {

i1-Threshold-r16                            MeasTriggerQuantityCLI-r16,

reportOnLeave-r16                           BOOLEAN,

hysteresis-r16                              Hysteresis,

timeToTrigger-r16                           TimeToTrigger

},

...

},

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

CLI-PeriodicalReportConfig-r16 ::=          SEQUENCE {

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCLI-r16                       MeasReportQuantityCLI-r16,

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

MeasTriggerQuantityCLI-r16 ::=              CHOICE {

srs-RSRP-r16                                SRS-RSRP-Range-r16,

cli-RSSI-r16                                CLI-RSSI-Range-r16

}

MeasReportQuantityCLI-r16 ::=               ENUMERATED {srs-rsrp, cli-rssi}

-- TAG-REPORTCONFIGNR-STOP

-- ASN1STOP

CondTriggerConfig field descriptions

a3-Offset Offset value(s) to be used in NR conditional reconfiguration triggering condition for cond event a3. The actual value is field value * 0.5 dB.

a5-Threshold1/ a5-Threshold2 Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a5. In the same condeventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

condEventId Choice of NR conditional reconfiguration event triggered criteria.

timeToTrigger Time during which specific criteria for the event needs to be met in order to execute the conditional reconfiguration evaluation.

t1XXX

ReportConfigNR field descriptions

reportType Type of the configured measurement report. In EN-DC, network does not configure report of type reportCGI using SRB3. The condTriggerConfig is used for CHO or CPC configuration.

ReportCGI field descriptions

useAutonomousGaps Indicates whether or not the UE is allowed to use autonomous gaps in acquiring system information from the NR neighbour cell. When the field is included, the UE applies the corresponding value for T321.

EventTriggerConfig field descriptions

a3-Offset/a6-Offset Offset value(s) to be used in NR measurement report triggering condition for event a3/a6. The actual value is field value * 0.5 dB.

aN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event number aN. If multiple thresholds are defined for event number aN, the thresholds are differentiated by M. The network configures aN-Threshold1 only for events A1, A2, A4, A5 and a5-Threshold2 only for event A5. In the same eventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

channelOccupancyThreshold RSSI threshold which is used for channel occupancy evaluation.

eventId Choice of NR event triggered reporting criteria.

l1-ThresholdOffset An offset value to locationThreshRef to obtain the threshold to be used in for event L1 and CondEvent L1

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report for A1-A6 events.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1.

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

ul-DelayValueConfig If the field is present, the UE shall perform the actual PDCP queueing delay measurement per DRB as specified in TS 38.314 [53] and the UE shall ignore the fields reportQuantityCell and maxReportCells. The applicable values for the corresponding reportInterval are (one of the) {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30}. The reportInterval indicates the periodicity for performing and reporting of UL PDCP Delay per DRB measurement as specified in TS 38.314 [53].

useT312 If value TRUE is configured, the UE shall use the timer T312 with the value t312 as specified in the corresponding measObjectNR. If value FALSE is configured, the timer T312 is considered as disabled. Network configures value TRUE only if reportType is set to eventTriggered.

useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1.

CLI-EventTriggerConfig field descriptions

i1-Threshold Threshold value associated to the selected trigger quantity (e.g. SRS-RSRP, CLI-RSSI) to be used in CLI measurement report triggering condition for event i1.

eventId Choice of CLI event triggered reporting criteria.

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a CLI measurement resource in srsTriggeredList or rssiTriggeredList, as specified in 5.5.4.1.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

CLI-PeriodicalReportConfig field descriptions

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportQuantityCLI The CLI measurement quantities to be included in the measurement report.

------------------------------------ end TP 3------------------------------------------------------------