3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2100348

Online, 25thJanuary – 5th February, 2021

Agenda Item:	8.10.3.4

Source:	Ericsson

Title:	NTN location reporting and network identifiers

Document for:	Discussion, Decision

# 1	Introduction

RAN2 received the below LS from RAN3 [4]:

RAN3 would like to inform SA2 and RAN2 that RAN3 agreed that a Cell ID as used in the User Location Information on the NG/N2 interface corresponds to a fixed geographical area, and the Tracking Area is coupled with geographical area.

Although it is up to RAN2 to decide how cell IDs and TAIs are broadcast in SIB1 for NTN, RAN3 would like to share the approaches so far considered in RAN3:

a)	On Uu, SIB content corresponds to momentary coverage area of a satellite beam related to the geographically fixed areas of TAs/Cells - irrespective of whether the beam is fixed or moving.

b) 	The cell ID used on Uu SIB content (and probably on Xn) are decoupled from cell ID used on NG(N2). The respective mapping is performed in RAN. This requires gNB to acquire the UE’s location information.

To progress further, RAN3 would also like to ask RAN2 to provide any feedback on the above approaches including, e.g. SIB aspects, and how the RAN could acquire information on the UE’s location if this is needed e.g. for above approach, and for registration, etc.

Note: NTN WID includes “identification of potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3]”. This could be used to determine the UE location with sufficient level of accuracy if needed, as requested in R3-206842.

Additionally, RAN2 had an email discussion related to location aspects:

[Post112-e][151][NTN] LCS for NTN (Fraunhofer)

Scope: Identify potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and discuss adaptations if any

Intended outcome: email discussion report

Deadline:  Long

In this contribution, we provide our view on aspects related to the LS response from RAN2 as well as address the email discussion #151.

# 3 Discussion of the LS

Here, we discuss approaches a) and b), as presented in the LS above.

In approach a), the satellite transmits in SIB1 always the cell ID and TAI that correspond to the geographical area the beam is serving. The same identifiers are used on the NG/N2 interface, and thus the requirement that cell IDs and Tracking Area correspond to a geographical area is fulfilled.

For Earth-fixed beams, the “momentary coverage area of a satellite beam” is approximately constant. This means that a satellite can transmit in SIB1 the same cell ID and TAI throughout the time the beam is active. When the beam starts serving a particular area, it gets a certain cell ID, and since the beam serves always approximately the same geographical area, the cell ID stays always the same, too.

For moving beams, the geographical area a satellite beam covers, is constantly changing while the beam sweeps the surface of the Earth. Since the SIB1 content is supposed to correspond to a fixed geographical area, this means that the satellite has to constantly change the cell ID it broadcasts in SIB1. Whenever the beam footprint has moved by about one cell diameter, cell ID in SIB1 need to be adapted to the new geographical area the beam now covers. It should be noted that the PCI has to  stay the same as it is encoded in SSB, while the cell ID (parameter in SI) changes. However, this would mean that the satellite needs to keep a mapping from PCI to cell ID that would need to be updated constantly. A clear drawback of this approach is that, as the cell ID changes, all UEs would need to re-acquire SI. Since this might happen quite often, it would lead to quite high load resulting from SI updates. Also, the update of the system information is not instantaneous, but always comes with a certain delay, since the network needs to respect the SI modification periods, i.e. SI update is only possible at certain specific radio frames. Further, there is delay and uncertainty associated when a UE can be assumed to have read the updated SI.

Approach a) has clear drawbacks in terms of generating high load due to frequent SI change, and SI update latency.

In approach b), different identifiers are used in SIB1 and on the NG/N2 interface. A mapping between these identifiers is maintained by the RAN. This allows to transmit always the same cell ID in SIB1, thus avoiding frequent SI updates in the moving beam case. The Earth-fixed beam case is very similar to approach a). The mapping between the identifiers in SIB1 and NG/N2 would be constant. It should be noted that RAN3 means by ”This requires gNB to acquire the UE’s location information.” Cell ID and tracking area is UE location information that is provided from RAN to CN.

RAN3 means by ” This requires gNB to acquire the UE’s location information.” Cell ID and tracking area is UE location information that is provided from RAN to CN.

Since approach b) avoids the drawback of frequent SI updates, we prefer approach b),  with a mapping between SIB1 and NG/N2 identifiers It should be noted that the need to maintain this mapping is not a drawback of approach b), since in approach a), a similar mapping between PCI and cell ID needs to be maintained.

RAN2 to indicate in the LS response that approach b) is preferred.

# 4 Location Services

In the email discussion #131 [3] many detailed questions have been raised about adapting the existing LCS for NTN (e.g. about down-selecting different methods). There was also some discussion about which use cases can be expected and which requirements. In our view, however, it is not in RAN2 scope to define these use cases and requirements. That should come from other groups, and RAN2 can then define methods to support these use cases or review the existing methods and define adaptations if needed.

RAN2 should not define use cases for positioning and associated requirements.

As the email discussion #131 addresses RAT dependent and RAT independent postioning methods to be used in NTN we given our views on these here as well. The most straightforward solution is the UE reported GNSS position. All UEs are anyway assumed to have GNSS capabilities as agreed in [1]. The GNSS position error is usually well below a typical terrestrial cell size, even in unfavorable situations. The use of RAT-dependent methods based on NR signals (e.g. multi-RTT, TDoA, …) seems unnecessary to fulfill the requirements.

It is certainly possible to obtain a “reliable” UE GNSS position by having the UE report not only its position, but also the GNSS waveform, which is then sent to a server for validation. Another possibility is to use UE-assisted GNSS, where the UE does the GNSS measurements, but instead of calculating its own position, it sends the measured data (e.g. code phase, Doppler, C/No Accumulated Delta Range) to the network, which then does the position calculation [2]. No adaptations of the existing methods seem to be necessary for this method to work in NTN.However, if UE is intending to fake its position, it can also fake the reports for location calculation.

Another option would be to use RAT-dependent methods, but it is unclear why these would be superior. The position accuracy will hardly be better than that from GNSS-based methods. Also, since the current positioning framework was developed with the use case of indoor positioning in mind, with a geometry completely different from NTN. Thus the RAT dependent measurements from NTN geometry would not likely provide finer granulary than cell ID.,.

Current positioning framework was developed with the use case of indoor positioning in mind, with a geometry completely different from NTN.

Even from a trustworthiness point of view, it seems that a UE, that has enough resources to fake the inputs for the GNSS position calculations, might also be able to produce faked inputs to the RAT-based positioning methods.

RAT-dependent methods don’t appear to be superior to RAT-independent methods in any way.

If UE is intending to fake its position, it can also fake the reports for location calculation thus network based and UE based procedures, RAT dependent or independent, seem both potentially reliable or unreliable depending on intention of the UE.

RAN2 to conclude that for RAN2 specific use cases, location based RRM and/or location reporting and location based CHO, the UE calculated GNSS position fulfils the need without need for RAT dependent positioning methods.

The WID has the following objective:

- Identify potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3]

In order to discuss positioning in NTN context in RAN2, we should know what the RAN2 specific problem is that we are trying to solve with this positioning discussion. That is what is the “use” of the LCS. For example, the accuracy for the location based CHO and RRM triggering should be decided during the WI. This may or may not stem from regulatory requirements. However, RAN2 may receive requests from other groups to support a certain use case but the initiative should come from there. That is, RAN3 and SA2 should discuss the matter and let RAN2 know if any enhancement is needed.

That said, some relevant input may potentially be found in a Reply LS from SA2 to RAN3 (without RAN2 involvement) in R3-206842. The relevant parts are pasted below:

  - While the cell ID today is deemed sufficient as indication of UE location in great majority of use cases, SA2 is of the opinion that in some scenarios with LEO satellite access the network will have to rely on Location Services (LCS) to determine the UE location with sufficient level of accuracy. This is particularly valid for Emergency services and other regulatory requirements.

  - SA2 further points out that SA3-LI has stated, in an LS sent to SA2 and RAN3 (in S3i200056), that “The logical location information (Cell ID) shall be reliable, i.e. network-provided or network-verified. The logical location shall unambiguously map to the geographical area of the UE physical location. Granularity of such geographical areas needs to be able to provide network location accuracy comparable with terrestrial networks”.

  - On the other hand, SA2 thinks that there will still be use case scenarios where the cell ID will be deemed sufficient as indication of UE location, despite its coarse granularity. To support such scenarios, it is important that the cell ID received by the CN should always correspond to a fixed geographical area.

  - In that sense, SA2 would like to encourage RAN3 to further study possible solutions for associating the cell ID sent to the CN to a fixed geographical area and to consider whether TN level granularity may be possible in selected instances.

We extract three key points from this LS:

- The cell ID sent from the RAN to the CN needs to correspond to a fixed geographical area.

- SA2 and SA3 seem to foresee use cases where the network needs to know the UE position with an accuracy that is comparable to cell-level in a terrestrial network.

- The UE position needs to be reliable, i.e. network-provided or network-verified.

The first issue would be solved by the methods provided in the LS[4] discussed in Section 3.

Regarding the second issue (location accuracy comparable to terrestrial cell size),It as well falls outside of RAN2 scope. Based on LS [4] it seems clear that either option a) or b) is enough for UE location for CN. That is RAN provides cell ID and TAI to CN where cell ID is either the same cell ID that is also broadcasted in SI (option a)) or there is mapping in RAN to enable the cell ID be coupled with PCI as it is in TN networks.

It is not RAN2 area to assess whether cell ID is or is not enough for UE location purposes for CN.

Based on LS [4] it seems clear that either option a) or b) is enough for UE location for CN.

RAN2 to conclude that the requirement discussion on should CN need finer UE location information than cell ID and TAI from RAN is outside of RAN2 scope.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Approach a) has clear drawbacks in terms of generating high load due to frequent SI change, and SI update latency.

Observation 2	RAN3 means by ” This requires gNB to acquire the UE’s location information.” Cell ID and tracking area is UE location information that is provided from RAN to CN.

Observation 3	Current positioning framework was developed with the use case of indoor positioning in mind, with a geometry completely different from NTN.

Observation 4	RAT-dependent methods don’t appear to be superior to RAT-independent methods in any way.

Observation 5	If UE is intending to fake its position, it can also fake the reports for location calculation thus network based and UE based procedures, RAT dependent or independent, seem both potentially reliable or unreliable depending on intention of the UE.

Observation 6	It is not RAN2 area to assess whether cell ID is or is not enough for UE location purposes for CN.

Observation 7	Based on LS [4] it seems clear that either option a) or b) is enough for UE location for CN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to indicate in the LS response that approach b) is preferred.

Proposal 2	RAN2 should not define use cases for positioning and associated requirements.

Proposal 3	RAN2 to conclude that for RAN2 specific use cases, location based RRM and/or location reporting and location based CHO, the UE calculated GNSS position fulfils the need without need for RAT dependent positioning methods.

Proposal 4	RAN2 to conclude that the requirement discussion on should CN need finer UE location information than cell ID and TAI from RAN is outside of RAN2 scope.

# References

RP-201256, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #88e, June 2020.

TR 38.305, “Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN,” V16.3.0.

R2-10xxxx, [Post112-e][151][NTN] LCS for NTN (Fraunhofer)

R3-207062 Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G