3GPP TSG-RAN WG2 Meeting #113e	R2-2100354

Online, January 25 – February 5, 2021

Agenda item:		8.1.2.1

Source:		Futurewei

Title:		L2 Retransmission (PDCP vs. RLC) for MBS

Document for:		Discussion and Decision

# Introduction

RAN2 has reached the following agreements on MBS –

- “Confirm that We will, for multicast services introduce support for PTP and PTM transmission of shared traffic delivered by 5GC, at least for connected mode (this is not intended to exclude other cases).”

- “For a UE, gNB dynamically decides whether to deliver multicast data by PTM or PTP (Shared delivery)”

In “Email discussion [Post111-e][904][MBS] L2 Architecture” [1], majority of companies think that at least PDCP can be the anchor for PTP and PTM dynamic switch, and MBS protocol structure can be based on split bearer like design as shown in Figure 1:

Figure 1: Split Bearer Like Architecture for PTP/PTM Dynamic Switch

This contribution analyzes and compares the specs impact and expected performance of applying L2 retransmission at either PDCP sublayer or RLC sublayer to enhance reliability of a PTM transmission.

# Discussion

Though PDCP duplication has been specified in R15 and R16, its operation is not based on the status report of a receiver. Duplication is done blindly for all PDCP PDUs to support URLLC services, at the expense of radio resource utilization. Given the potentially large amount of MBS traffic, it is not wise to apply PDCP duplication for reliable PTM transmission.

Observation 1: PDCP duplication blindly transmits multiple copies of every PDCP PDU, and it is not suitable for MBS.

With split bearer like architecture of PTP/PTM transmission, a UE’s reception feedback can be carried on the UL of its PTP leg, either as PDCP status report or as RLC status report, without much specs impact. Hence, for transmission over PTM, the status reports from MBS UEs should be transmitted over their respective dedicated logical channel over PTP leg.

Observation 2: Without much specs impact, either PDCP status report or RLC status report can be carried on the UL of a UE’s PTP leg in a split bearer like architecture of PTP/PTM transmission.

Proposal 1: The status reports from MBS UEs should be transmitted over their respective dedicated logical channel over PTP leg.

PDCP status report can be generated when [2]: “

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery;

-	upper layer requests a uplink data switching;

-	upper layer reconfigures the PDCP entity to release DAPS and daps-SourceRelease is configured in TS 38.331 [3].”

That is, PDCP status report is triggered by RRC, not by the transmission/reception status of PDCP PDU. On the other hand, RLC status report can be triggered by polling and t-Reassembly timer for the transmission/reception status of RLC PDU [3].

Observation 3: While RLC status report can be triggered by polling and t-Reassembly timer for the transmission/reception status of RLC PDU, PDCP status report is triggered by RRC, not by the transmission/reception status of PDCP PDU.

When loss of RLC PDU is detected and RLC status report is triggered by the receiving side of an AM RLC entity or receiving UM RLC entity, the lower edge of RLC receiving window RX_Next is not moved, and the retransmitted RLC PDU will still fall within the RLC receiving window [3] – “

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received;

-	if RX_Next_Highest> RX_Highest_Status +1: or

-	if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.”

When loss of a PDCP PDU is detected, however, the lower edge of PDCP reception buffer RX_DELIV is moved forward, and any retransmission of missing PDCP PDU would fall out of reception buffer [2] – “

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.”

Observation 4: While RLC sublayer already supports the retransmission of a lost RLC PDU, the lower edge of PDCP reception buffer RX_DELIV is moved forward when loss of a PDCP PDU is detected, and any retransmission of missing PDCP PDU would fall out of reception buffer.

In order to allow retransmission of lost PDCP PDU without RRC involvement, management of the reception buffer at PDCP sublayer needs to be modified such that its lower edge is not moved until the corresponding PDCP PDU is received. In other words, the procedure of handling t-Reordering timer expiration at PDCP sublayer should be modified in a way similar to the handling of t-Reassembly timer expiration at RLC sublayer. This basically leads to the replication of existing RLC functionality at PDCP sublayer.

Observation 5: In order to allow retransmission of lost PDCP PDU without RRC involvement, management of the reception buffer at PDCP sublayer needs to be modified, practically replicating existing RLC functionality at PDCP sublayer.

A PDCP entity may be associated with multiple RLC entities, which in turn are associated with different logical channels. With the split bearer like architecture for PTP/PTM dynamic switch, these RLC entities/logical channels need to operate at different modes, like one in UM and another one in AM. This makes it difficult to configure proper t-Reordering timer, as different legs would take various time to finish transmission of a PDCP PDU.

Observation 6: PDCP retransmission suffers performance degradation in split bearer like architecture for PTP/PTM dynamic switch, as a single t-Reordering timer at PDCP sublayer wouldn’t fit to transmission time budgets of both UM and AM RLC legs.

There had been extensive discussions during the study of NR in Rel-14 on where L2 retransmission should be done [4] – “ R2-166828	Second level retransmissions in NR	Ericsson	discussion

-	CATT understand that ARQ should not deal with front haul errors but ARQ above the front haul can bring benefits in multiconnectivity scenarios to help cope with link failures. We should not preclude that ARQ is in the CU

-	LG would like to discuss this again after reordering is decided

=>	Comeback to discuss P3

-	LG think that ARQ could be in PDCP based on PDCP SN.

-	MediaTek support ARQ in RLC especially for DC cases.

-	CATT think it should be possible to configure ARQ in PDCP, to deal with the fact that the link can break quickly. It should not be ruled out at the moment. Ericsson think if it is in PDCP then we have backhaul issues. This doesn't preclude duplication in PDCP to send packets on different links and it is also a lower delay solution. Huawei also think that an eNB could resend packets on the other link if it detects low throughput on the other link.

-	ZTE agree with the proposal.

Agreement

-	The ARQ will be supported in RLC.

”

Performing L2 retransmission at PDCP layer may subject a system to additional delay over front-haul between DU and CU, and degrade system performance.

Observation 7: Extensive study in Rel-14 led to the agreement of having ARQ supported in RLC sublayer to avoid performance degradation from additional delay over front-haul between DU and CU.

Proposal 2: If L2 retransmission is needed to enhance reliability of PTM transmission, it should also be done at RLC sublayer.

# Conclusions

This paper analyzes the specs impact and expected performance of applying L2 retransmission at either PDCP sublayer or RLC sublayer to enhance reliability of a PTM transmission, which leads to the following observations and proposals:

Observation 1: PDCP duplication blindly transmits multiple copies of every PDCP PDU, and it is not suitable for MBS.

Observation 2: Without much specs impact, either PDCP status report or RLC status report can be carried on the UL of a UE’s PTP leg in a split bearer like architecture of PTP/PTM transmission.

Proposal 1: The status reports from MBS UEs should be transmitted over their respective dedicated logical channel over PTP leg.

Observation 3: While RLC status report can be triggered by polling and t-Reassembly timer for the transmission/reception status of RLC PDU, PDCP status report is triggered by RRC, not by the transmission/reception status of PDCP PDU.

Observation 4: While RLC sublayer already supports the retransmission of a lost RLC PDU, the lower edge of PDCP reception buffer RX_DELIV is moved forward when loss of a PDCP PDU is detected, and any retransmission of missing PDCP PDU would fall out of reception buffer.

Observation 5: In order to allow retransmission of lost PDCP PDU without RRC involvement, management of the reception buffer at PDCP sublayer needs to be modified, practically replicating existing RLC functionality at PDCP sublayer.

Observation 6: PDCP retransmission suffers performance degradation in split bearer like architecture for PTP/PTM dynamic switch, as a single t-Reordering timer at PDCP sublayer wouldn’t fit to transmission time budgets of both UM and AM RLC legs.

Observation 7: Extensive study in Rel-14 led to the agreement of having ARQ supported in RLC sublayer to avoid performance degradation from additional delay over front-haul between DU and CU.

Proposal 2: If L2 retransmission is needed to enhance reliability of PTM transmission, it should also be done at RLC sublayer.

# References

R2-2009337, Email discussion [Post111-e][904][MBS] L2 Architecture.

3GPP TS 38.323, “NR; Packet Data Convergence Protocol (PDCP) specification”.

3GPP TS 38.322, “NR; Radio Link Control (RLC) protocol specification”.

R2-167461, “Report of 3GPP TSG RAN WG2 meeting #95bis”.