3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2100372

Elbonia, 25 January – 05 February 2021

Agenda item:	8.1.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Handling of Measurement Gaps

WID/SID:	NR_MBS-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses the handling of Measurement Gaps in the context of MBS.

# 2	Measurement Gaps

Measurement gaps are provided to UE in RRC_CONNECTED to measure neighbouring cells. Whether a measurement is non-gap-assisted or gap-assisted depends on the capability of the UE, the active BWP of the UE and the current operating frequency. This baseline mechanism was further enhanced in Rel-16 by introducing NeedForGap as the UE capability to perform gapless measurement not only depends on the current band combination but also other L1 parameters that may occupy some baseband resource (e.g. number of MIMO layers).

When multiple UEs requiring measurement gaps receive the same MBS service via group scheduling (i.e. PTM transmission though G-RNTI) arise the question of how to handle these measurement gaps. We can think of three different alternatives:

1.	Common gap without MBS transmission: the measurement gaps of all UEs are configured to overlap and MBS data transmission stops during that one gap.

2.	Common gap with MBS transmission: the measurement gaps of all UEs are configured to overlap but MBS data transmission does not stop during that one gap.

3.	Dedicated gap with MBS transmission: gaps are individually configured and MBS data transmission does not stop for each individual gap.

NOTE:	Dedicated gaps without transmission is obviously not an option as it would leave very little time available for MBS data transmission, if any.

The first alternative is feasible but unfair as the worst UE will govern the throughput and latency of all UEs. The second and third alternatives do not exhibit the same drawback but UEs having to apply measurement gaps will suffer losses unless retransmissions are separately scheduled. So let us investigate this point.

For the retransmissions of PDUs originally sent via group scheduling, and targeting multiple UEs (the ones having used a measurement gap), we believe that group scheduling should also be used. Indeed, the very same reasons and benefits justifying the use of group scheduling for the first transmission also apply to retransmissions as soon as multiple UEs need to receive that retransmission. This favours the 2nd alternative with retransmissions being typically needed for multiple UEs. Such retransmissions would naturally be blind PDCP retransmissions not relying on any UE feedback: the network only assumes that the gap was used and schedules retransmissions to compensate for the corresponding losses.

Proposal 1: PDCP retransmissions to compensate for missed PTM transmissions due to measurement gaps also relies on group scheduling.

This could be achieved by the configuration of a second G-RNTI, which would then schedule retransmissions of MBS transmission overlapping with a common measurement gap. Only UEs having used the gap would decode these retransmissions.

# 3	Conclusion

This contribution has discussed the handling of measurement gaps in the context of PTM transmissions for MBS and proposed the following:

Proposal 1: PDCP retransmissions to compensate for missed PTM transmissions due to measurement gaps also relies on group scheduling.