3GPP TSG-RAN WG2 Meeting #113 electronic	 R2-2100392

# Online, Jan 25nd  -  Feb 5th , 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.305 | CR | 0095 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the descriptions of RequestLocationInformation message in TS36.305 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_LTE_iPos_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to the IE A-GNSS-RequestLocationInformation in LPP RequestLocationInformation message in TS36.355, the positioning mode of UE-based and UE-assisted is indicated within the RequestLocationInformation message but not within the positioning instructions of A-GNSS. However, the current specification specifies that there include positioning mode of UE-based, UE-assisted and Standalone within the positioning instructions of A-GNSS, which is conflict with TS37.355. - According to the LPP RequestLocationInformation message in TS37.355, there is not include positioning instructions for WLAN, Bluethooth, TBS, Motion sensor and Barometric Pressure Sensor positioning methods. Besides, the positioning mode indicated within the RequestLocationInformation message do not include Standalone mode. However, the current specification specifies that there include Standalone positioning mode within the positioning instructions for WLAN, Bluethooth, TBS, Motion sensor and Barometric Pressure Sensor positioning methods, which is conflict with TS37.355. - A-GNSS positioning method supporte standlone mode as described in clause 8.1. However, there lacks of A-GNSS positioning methods in the general descriptions of positioning methods supported in standlone mode in clause 4.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For the location information transfer procedure of A-GNSS, remove the positioning mode from the description of the positioning instructions and clarify the corresponding positioning mode can be indicated within the RequestLocationInformation message. - For the location information transfer procedure of WLAN, Bluethooth, TBS, Motion sensor and Barometric Pressure Sensor positioning methods, delete the description of the positioning instructions and clarify the corresponding positioning mode can be indicated within the RequestLocationInformation message. - Add A-GNSS positioning method in the general descriptions of positioning methods supported in standlone mode in clause 4.3. Impact analysis Impacted 5G architecture options: NR SA, MR-DC Impacted functionality: Positioning location information transfer Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no interoperability issue forseen. - If the UE is implemented according to the CR and the network is not, there is no interoperability issue forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors described above still exist in the current specification, which may lead to confliction with TS37.355. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 8.1.3.3.1, 8.6.3.1, 8.7.3.1.1, 8.8.3.1.1, 8.9.3.1.1, 8.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 4.3	Standard UE Positioning Methods

The standard positioning methods supported for E-UTRAN access are:

-	network-assisted GNSS methods;

-	downlink positioning;

-	enhanced cell ID method;

-	uplink positioning;

-	WLAN method;

-	Bluetooth method;

-	Terrestrial Beacon System method;

-	Sensor based methods:

-	Barometric Pressure Sensor;

-	Motion sensor.

Hybrid positioning using multiple methods from the list of positioning methods above is also supported.

Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

These positioning methods may be supported in UE-based, UE-assisted/E-SMLC-based, eNB-assisted, and LMU-assisted/E-SMLC-based versions. Table 4.3-1 indicates which of these versions are supported in this version of the specification for the standardised positioning methods.

Table 4.3-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, E-SMLC-based | eNB- assisted | LMU-assisted/ E-SMLC-based | SUPL |
| --- | --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | No | Yes (UE-based and UE-assisted) |
| Downlink Note1 | No | Yes | No | No | Yes (UE-assisted) |
| E-CID | No | Yes | Yes | No | Yes (UE-assisted) |
| Uplink | No | No | No | Yes | No |
| Sensor | Yes | Yes | No | No | No |
| WLAN | Yes | Yes | No | No | Yes |
| Bluetooth | No | Yes | No | No | No |
| TBS Note 2 | Yes | Yes | No | No | Yes (MBS) |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only for TBS positioning based on MBS signals. |  |  |  |  |  |

A-GNSS, Sensor, WLAN, Bluetooth, and TBS positioning methods based on MBS signals are also supported in standalone mode, as described in the corresponding clauses.

NEXT CHANGE

##### 8.1.3.3.1	E-SMLC initiated Location Information Transfer Procedure

Figure 8.1.3.3.1-1 shows the Location Information Transfer operations for the network-assisted GNSS method when the procedure is initiated by the E-SMLC.

Figure 8.1.3.3.1-1: E-SMLC-initiated Location Information Transfer Procedure

(1)The E-SMLC sends a LPP Request Location Information message to the UE for invocation of A-GNSS positioning. This request includes an indication of the positioning mode (UE-based, UE-assisted) and positioning instructions such as the positioning methods (GPS, Galileo, GLONASS, BDS, etc. and possibly non-GNSS methods, such as downlink positioning or E-CID), specific UE measurements requested if any, such as fine time assistance measurements, velocity, carrier phase, multi-frequency measurements, and quality of service parameters (accuracy, response time).

(2)	The UE performs the requested measurements and possibly calculates its own location. The UE sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step 1 elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

NEXT CHANGE

#### 8.6.3.1	E-SMLC initiated Location Information Transfer Procedure

Figure 8.6.3.1-1 shows the Location Information Transfer operations when the procedure is initiated by the E-SMLC.

Figure 8.6.3.1-1: E-SMLC-initiated Location Information Transfer Procedure

(1)	The E-SMLC sends a LPP Request Location Information message to the UE for invocation of barometric pressure sensor positioning. This request includes an indication of the positioning mode (UE-assisted, UE-based), specific requested UE measurements if any, and quality of service parameters (accuracy, response time).

(2)	The UE performs the requested measurements and possibly calculates its own position. The UE sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step 1 elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

NEXT CHANGE

##### 8.7.3.1.1	E-SMLC initiated Location Information Transfer Procedure

Figure 8.7.3.1.1-1 shows the Location Information Transfer operations for the WLAN method when the procedure is initiated by the E-SMLC.

Figure 8.7.3.1.1-1: E-SMLC-initiated Location Information Transfer Procedure

(1)	The E-SMLC sends a LPP Request Location Information message to the UE for invocation of WLAN positioning. This request includes an indication of the positioning mode (UE-assisted, UE-based), specific requested UE measurements if any, and quality of service parameters (accuracy, response time).

(2)	The UE performs the requested measurements and possibly calculates its own location. The UE sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step 1 elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

NEXT CHANGE

##### 8.8.3.1.1	E-SMLC initiated Location Information Transfer Procedure

Figure 8.8.3.1.1-1 shows the Location Information Transfer operations for the Bluetooth method when the procedure is initiated by the E-SMLC.

Figure 8.8.3.1.1-1: E-SMLC-initiated Location Information Transfer Procedure

(1)	The E-SMLC sends a LPP Request Location Information message to the UE for invocation of Bluetooth positioning. This request includes an indication of the positioning mode (UE-assisted), specific requested UE measurements if any, and quality of service parameters (accuracy, response time).

(2)	The UE performs the requested measurements and possibly calculates its own location. The UE sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step 1 elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

NEXT CHANGE

##### 8.9.3.1.1	E-SMLC initiated Location Information Transfer Procedure

Figure 8.9.3.1.1-1 shows the Location Information Transfer operations for the TBS method when the procedure is initiated by the E-SMLC.

Figure 8.9.3.1.1-1: E-SMLC-initiated Location Information Transfer Procedure

(1)	The E-SMLC sends a LPP Request Location Information message to the UE for invocation of TBS positioning. This request includes an indication of the positioning mode (UE-assisted, UE-based), specific requested UE measurements if any, and quality of service parameters (accuracy, response time).

(2)	The UE performs the requested measurements and possibly calculates its own location. The UE sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step 1 elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

NEXT CHANGE

#### 8.10.3.1	E-SMLC initiated Location Information Transfer Procedure

Figure 8.10.3.1-1 shows the Location Information Transfer operations when the procedure is initiated by the E-SMLC.

Figure 8.10.3.1-1: E-SMLC-initiated Location Information Transfer Procedure

(1)	The E-SMLC sends a LPP Request Location Information message to the UE for invocation of motion sensor positioning. This request includes an indication of the positioning mode (UE-based, UE-assisted), specific requested UE measurements if any, and quality of service parameters (accuracy, response time).

(2)	The UE performs the requested measurements. The UE sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step 1 elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

END OF CHANGE