3GPP TSG RAN WG2 Meeting #113-e				R2-2100416

Online, Jan 25th – Feb 5th, 2021

Source 	: CAICT

Title 	: Considerations on MAC timers in NTN

Agenda Item	: 8.10.2.2

Document for	: Discussion / Decision

Introduction

After the last meeting, UL scheduling enhancements proposed in [1] for NTN were discussed through email. In this contribution, we will discuss more considerations on MAC timers about UL scheduling in NTN.

MAC timers about UL scheduling in NTN

- sr-ProhibitTimer

sr-ProhibitTimer was introduced to prevent UE from sending frequent SR. When the UE has sent a SR but has not be provided corresponding UL scheduling while sr-ProhibitTimer is running, no further SR needs to be transmitted. According to [3], the UE instruct the physical layer to signal the SR on one valid PUCCH resource for SR, increment SR_COUNTER by 1 and start the sr-ProhibitTimer at the same time. When sr-ProhibitTimer is running, the UE waits for UL scheduling for it. UE retransmit SR if sr-ProhibitTimer is expired and SR_COUNTER is no more than the configured sr-TransMax. With this sr-ProhibitTimer, gNB could balance PUCCH resource for SR and the UE traffic delay.

In NTN, the maximum propagation delay could be larger than ever and be different in various networks like GEO, LEO, MEO, and etc. In GEO, the maximum propagation delay would up to 541.46ms. Due to the long propagation delay, the moment when the gNB thinks the sr-ProhibitTimer is started and the moment when the timer is expired may be quite different from that at the UE side. This is illustrated in the Fig.1 below. At the UE side, PUCCH with SR is transmitted and the sr-ProhibitTimer starts at t0. The PUCCH reaches gNB after one-trip propagation delay. Since UL grant from the gNB to the UE would also experience the long propagation delay, gNB has to send the UL grant no later than t2 which is one-trip delay before t3 when the sr-ProhibitTimer expires at UE side. Otherwise, the UE may think it’s UL transmission is not served and additional SR transmission would be triggered.

Fig. 1  Current sr-ProhibitTimer procedure in NTN

In the current specification [4], sr-ProhibitTimer could be configured {ms1, ms2, ms4, ms8, ms16, ms32, ms64, ms128} which the maximum value is 128ms. The current supported value of sr-ProhibitTimer could not satisfy the requirements in NTN since it is possible little time left for gNB to provide UL scheduling for UE. Or even it is possible the gNB has not get the scheduling request at all while sr-ProhibitTimer has been expired at the UE side.

In the previous meetings, it was proposed that the value range of the sr-ProhibitTimer is extended in NTN and FFS additional values and method of extension. To our opinion, if additional values are supported by sr-ProhibitTimer, the signaling overhead would be increased greatly to support the various satellite network types, LEO, MEO, GEO and etc. In addition, if the maximum propagation delay varies with the variability of cell coverage/ beam coverage, the value of sr-ProhibitTimer should be reconfigured accordingly which make the signaling overhead much heavier and complexity to the scheduler.

Observation 1: If additional values are supported by sr-ProhibitTimer, signaling overhead would be increased much heavier due to various network types is to be supported and the maximum propagation delay may vary frequently.

As illustrated in the Fig.2 below, UE starts sr-ProhibitTimer Koffset slots after the UE transmits SR on one valid PUCCH resource in NTN. After PUCCH for SR arrivers at gNB, as long as UL scheduling is provided within the time corresponding to the configured sr-ProhibitTimer value, no more SR is needed. With this improvement, the value range of sr-ProhibitTimer can be not extended.

On the other hand, it is noticed that RAN1 has agreed to introduce Koffset to enhance several DL-UL timing relationships. Koffset would also be applied to some MAC timers like ra-ContentionResolutionTimer, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL. In principle, Koffset is used to handle the timing difference in the UE side and gNB side due to large propagation delay. The Koffset could also be used handle the issue of sr-ProhibitTimer state difference at the gNB side and UE side. Since Koffset has been introduced, there would be no additional signaling overhead needed and specification efforts could be marginal.

Fig. 2  Enhancements on sr-ProhibitTimer procedure in NTN

Observation 2: No additional signaling overhead needed and specification efforts could be marginal to apply Koffset to the sr-ProhibitTimer.

As a result, it is proposed UE starts sr-ProhibitTimer Koffset after the UE transmits SR on one valid PUCCH resource in NTN.

Proposal 1: In NTN, UE starts sr-ProhibitTimer Koffset after the UE transmits SR on one valid PUCCH resource.

- retxBSR-Timer

retxBSR-Timer is used to remind the gNB of providing corresponding UL resources for the UE within certain time interval. The MAC entity restarts retxBSR-Timer upon reception of a UL grant for one HARQ process for CG PUSCH. When retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, BSR would be triggered. gNB is expected to provide enough UL resources corresponding to the requirements indicated by BSR before the timer expires. UE decides the time when retxBSR-Timer (re)starts according to detection of UL grant. Considering long propagation delay in NTN, it is possible the moment when retxBSR-Timer starts and the moment when the timer expires at gNB side is different from that at UE side.

Correspondingly, similar as the sr-ProhibitTimer, it is proposed the UE (re)starts retxBSR-Timer Koffset after the UE receives a UL grant in NTN.

Proposal 2: In NTN, UE (re)starts retxBSR-Timer Koffset after the UE receives a UL grant.

- configuredGrantTimer

Similar MAC timer includes configuredGrantTimer. The timer is (re)started if an uplink grant correponding to one HARQ process which is configued for CG PUSCH is received, or UL transmission is performed according to CG PUSCH. At gNB side, however, when the timer (re)starts and when the time expires may quite different from that at the UE side. It is proposed the UE (re)starts configuredGrantTimer Koffset after the time when UE receives a UL grant correponding to one HARQ process which is configued for CG PUSCH, or when UE performs UL transmission according to CG PUSCH.

Proposal 3: In NTN, UE (re)starts configuredGrantTimer Koffset after the time when UE receives a UL grant corresponding to one HARQ process which is configured for CG PUSCH, or when UE performs UL transmission according to CG PUSCH.

Conclusion

In this contribution, we discussed more considerations on MAC timers in NTN. The following observations and proposals are reached:

Observation 1: If additional values are supported by sr-ProhibitTimer, signaling overhead would be increased much heavier due to various network types is to be supported and the maximum propagation delay may vary frequently.

Observation 2: No additional signaling overhead needed and specification efforts could be marginal to apply Koffset to the sr-ProhibitTimer.

Proposal 1: In NTN, UE starts sr-ProhibitTimer Koffset after the UE transmits SR on one valid PUCCH resource.

Proposal 2: In NTN, UE (re)starts retxBSR-Timer Koffset after the UE receives a UL grant.

Proposal 3: In NTN, UE (re)starts configuredGrantTimer Koffset after the time when UE receives a UL grant corresponding to one HARQ process which is configured for CG PUSCH, or when UE performs UL transmission according to CG PUSCH.

- Reference

- R2-2009064, “Enhancements on UL scheduling for NTN, Nomor Research GmbH”, Thales

- R2-2009109, “Discussion on other MAC issues in NTN”, OPPO

- 3GPP TS 38.321 V16.3.0, “Medium Access Control (MAC) protocol specification”

- 3GPP TS 38.331 V16.3.1, “Radio Resource Control (RRC) protocol specification”