3GPP TSG-RAN WG2 Meeting #113-e	   R2-2100428

Online, Jan 25th - Feb 5th, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the Paging Collision

Agenda item:		8.3.2

Document for: 	Discussion and Decision

- Introduction

About the paging collision, a Reply LS has been sent to SA2 including the feasibility and effectiveness analysis for each paging collision solutions.

| Solution | Advantage | Disadvantage | RAN spec impact |
| --- | --- | --- | --- |
| Solution 1 | Less signalling overhead over Uu and no increase in MT connection establishment | Without UE assistant info, a new assigned 5G-GUTI/alternative UE_ID/offset may still result in PO collisions Paging collision may occur after cell reselection in which UE needs to request new 5G-GUTI/alternative UE_ID/offset again. | No impact. |
| Solution 2a |  |  | Change the legacy way to calculate PF/PO. |
| Solution 2b |  |  |  |
| Solution 3 | Paging missing in condition of PO collision can be avoided | Could result in significant Uu signaling overhead. Causes increased idle mode battery consumption for all devices even when Paging Occasion collision is not happening. Increased MT connection establishment latency when PO collision occurs. | UE is required to at least monitor one PO in a single DRX among consecutive DRX cycles. |

In this paper, we will further analyze each paging collision issues.

- Discussion

About the paging collision, SA2 has discussed the below solutions.

- Solution 1: UE -requested 5G-GUTI reassignment for one USIM using the Mobility Registration Update. However, it should be noted the 5G-GUTI is systematically reassigned by the network during the Mobility Registration Update procedure (as of Rel-15) requires. Proposed for 5GS only.

- Solution 2: Changes related to the UE_ID (UE Identity Index) that is used for calculation of PF/PO only:

- 2a: Calculation of PF/PO by using an Alternative UE_ID. The UE ID sent in the paging message is not impacted by this Alternative ID that is only used for PO/PF calculations Proposed for both EPS and 5GS.

- 2b: Calculation of PF/PO by using a UE_ID which is derived from IMSI+offset value. The offset value is negotiated between UE and MME. Proposed for EPS only.

- 2c: Calculation of PF/PO based on MUSIM Assistance Information which can carry either a paging policy selector in RAN or an Alternative ID (like in solution above) or a pattern of availability (e.g. specific SFN Slots/ DRX cycles).

- Solution 3: Repeating paging in the RAN on consecutive POs. for MUSIM devices.

In which the solution 2b is for the LTE and has been confirmed in both the SA2 and Ran plenary meeting. For the NR, the solution 1/2a/2c/3 can be adopted. For the solution 3, in the last RAN2 meeting, companies have expressed strong concern on the resource wasting and UE power consumption, while the Solution 2a will introduce an alternative ID, which will improve the complexity for the UE, Ran and CN. Thus the solution 2a/3 shall be given the lowest priority.

Proposal 1: From Ran 2 side, the solution 2a/3 shall be given the lowest priority.

For the solution 1 and 2c, the main difference is that whether the MUSIM Assistance Information is needed. As analyzed in the last meeting, with the solution 1, a new assigned 5G-GUTI may still result in PO collisions, thus in the SA2, some companies prefer to indicate some assistance information. However, to solve this issue, besides to sending the assistance information to the Network, it can also be left to the network implementation for that the PO is periodically distributed and the possible paging cycle is specified to be {rf32, rf64, rf128, rf256}. Take the Fig 1 as an example, the network B can have the same (network B2) or different (network B1) paging cycle from the network A, once the collision happened, the network B can shift the PO to the position of original PO+16rf, then the AMF select a 5G-STMSI that can derive a PO next to the original PO+16rf. The AMF can select it based on the some paging configuration information from the Ran.

Fig 1: PO collision

Anyway, the Fig 1 just provide one implementation method, from this method, we can see that the Network can solve the re-collsion issue even without the assistance information from the UE.

Observation 1: The re-collision issue can be solved without assistance information from the UE.

Company may have another concern on neighbor cells, for example, UE moves among the cell A/B/C, even the new assigned 5G-STMSI can avoid the collision with the cell A, it may conflict with the cell B/C when the UE moves to the Cell B/C. For this issue, some companies suggest to give some assistance information based on the POs of cell A/B/C, then the AMF can select a better 5G-STMSI based on these assistance information. But we think the AMF can also store the 5G-STMSIs that has caused collision and make choice based on the stored information. Thus it’s still unnecessary for the UE to report the assistance information.

Proposal 2: The solution 1 can be taken as baseline. How to re-assign the 5G-STMSI can left to the network implementation.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: From Ran 2 side, the solution 2a/3 shall be given the lowest priority.

Observation 1: The re-collision issue can be solved without assistance information from the UE.

Proposal 2: The solution 1 can be taken as baseline. How to re-assign the 5G-STMSI can left to the network implementation.