3GPP TSG-RAN WG2 Meeting #113-e                             	R2-2100434

Online, 25th January – 5th February 2021

Agenda item:	8.3.2

Source:	Samsung

Title:	Paging Collision Avoidance for Multi-RAT MUSIM UE

Document for:	Discussion & Decision

# Introduction

In revised Rel-17 Multi SIM WI in RAN planery#90e [1], updated objective for paging collision is as follows:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2]

  - RAT Concurrency: Network A can be NR or LTE. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx.

This contribution discusses potential solution to address paging collision issue for Multi-RAT Multi-SIM UE.

# Discussion

RAN2 made following agreements in the previous RAN2#112-e meeting [2]

Agreements - From RAN2 point of view, Option 1, 2a, 2b, and 3 are feasible to solve the paging collision issue in 5GS. Each have different effectiveness (as per analysis during the email discussion). When indicating reply to SA2, indicate both feasibility as well as effectiveness. - Indicate to SA2 that RAN2 continues to further evaluate the pros and cons of options 1, 2a, 2b, 3. - Option 4 is still allowed (but RAN2 will not specify UE implementation). - Clarifying "No E-UTRA impact" can be done in RANP. - Option 2c can be evaluated later as it doesn't work alone. Agreements - Enhancement for 5GS should be prioritized since it can handle paging collision issue in both NR+NR and NR+LTE scenarios

SA2 also reached following interim conclusions for paging collision avoidance in EPS [3]

For paging reception in EPS when the paging collision is detected, the following principles are agreed: - Upon the UE detecting paging collisions between two networks, the UE initiates a TAU procedure to the MME of one network, to request an IMSI offset. - UE may provide an IMSI offset to MME during TAU procedure. - The MME returns an IMSI offset to the UE in the TAU Accept. - During CN paging delivery, the MME provides to the RAN the UE_ID which is derived based on the IMSI and the IMSI offset. RAN and UE use the UE ID as the IMSI to calculate the PF/PO

In general, MUSIM UE can be associated with Multi-RAT networks i.e. two SIMs can pertain to network combinations of NR+LTE, LTE+NR, NR+NR, LTE+LTE.

The MUSIM UE will calculate each paging occasion on two networks, respectively. It means that the UE is able to detect whether potential paging collision would happen. Network can also interpret possible paging conflict when it does not receive paging response from UE. However, as it is not aware or informed on the paging configuration used on other network for MUSIM UE, it cannot be sure if loss of paging response is due to paging conflict or some decoding issue at UE or unreachability of UE. So, it would be more logical for the UE to determine potential paging collision and trigger actions on potential paging collision avoidance.

Proposal 1: MUSIM UE determines potential paging collision on two networks and triggers actions on potential paging collision avoidance.

It is relevant question whether there is need to specify prioritization rule on network/RAT selection for paging collision avoidance. That is, on which network/RAT among LTE and NR, a Multi-RAT MUSIM UE would attempt to address paging collision avoidance? However, any such prioritization would complicate the specification. We think it can be rather left to UE implementation.

Proposal 2: It is left to UE implementation as to how it selects one of the two RATs/networks for paging collision avoidance.

In general, paging configuration parameters can be same or different on two networks for MUSIM UE e.g. USIM A (UE_ID, T, N, PF_offset) and USIM B (UE_ID’, T’, N’, PF_offset’) and UE can provide collision indication and possibly assistance information to the network e.g. required offset. Since Access Stratum (AS) is better equipped with paging configuration parameters information on both the networks, it seems logical that AS builds the assistance information for paging collision avoidance.

Proposal 3: MUSIM UE may provide assistance information to network to resolve paging collision since involved networks are not coordinated. Access Stratum in the MUSIM UE builds assistance information for paging collision avoidance.

Once the UE detects potential paging collision, relevant question is how this is signaled to network for any corrective action. There seem two possible approaches of (a) NAS based signaling and (b) AS based signaling. Choice is largely dependent on which network entity (e.g. RAN or CN) would address the paging collision resolution. As MUSIM UE is in the idle/inactive modes in both the networks, this implies that the involved network entity must preserve the UE context in the idle/inactive mode to apply the needed paging modification for every paging message. Since RAN does not hold the UE context in idle mode, there would be a new need for storage at RAN and brings in complexity and undesired impact e.g. addressing mobility of idle mode UE. Therefore, CN is better suited to address paging modification and NAS signaling mechanism should be preferred.

Observation 1: Maintaining UE context for idle mode UE at RAN to support paging collision avoidance is drastic change and have undesired impact.

Proposal 4: MUSIM UE utilizes NAS signaling to request potential paging collision avoidance to the network.

In line with previous RAN2 meeting agreements, we consider only solutions under option 1, 2a, 2b and 3. The solutions 1, 2a, and 2b are very much similar with regard to the final objective of altering the CN parameter for UE_ID which, in turn, affects the paging timing PF/PO when overlap situation is encountered.

With option 1 for UE-requested 5G-GUTI reassignment, collision probability with new GUTI assigned by network is expected to be very less. In general, CN is not aware about paging timings. UE may be required to again request for new GUTI reassignment in the case new 5G-GUTI does not resolve paging collision or a cell reselection happens. UE may also optionally provide assistance information to help CN. Further, given the paging collision probability is very less, this is the simplest solution and quite effective.

Option 2a is similar to option 1 but utilizes Alternative UE_ID for only for PF/PO timing calculations. It involves complexity for CN, RAN and UE with managing two identities for paging message and paging timing calculation. This seems a drastic change from legacy.

Option 2b with offset approach is like option 1 but essentially separates the paging timing considerations with ‘offset’ and provides a clear information to devise an updated UE_ID. However, the approach may require re-negotiation when UE moves to a new cell/area with different paging configuration, however, only when it causes a paging collision. For EPS, IMSI as UE_ID is permanent identity and therefore, solution like option 1 and option 2a are not applicable in EPS. Option 2b with offset approach is applicable and was originally intended for EPS only in SA TR [3]. Notably UE requests IMSI offset to MME and may optionally include an offset value as assistance information.

Option 3 for network repetition or retransmission, in our opinion, is not paging resource efficient. We think RAN2 should consider to improvise this aspect before considering this as a solution. Some of the possible directions can include

- Network paging retransmission is limited to only MUSIM UEs which indicate potential paging collision; and/or

- Unlike TR 23.761 [3], retransmission is not required in two consecutive POs in second attempt, rather paging is retransmitted only in a different PF/PO (termed as secondary PF/PO) to address any possible systematic collision. Secondary PF/PO can be specified.

Proposal 5: Changing UE_ID via NAS signalling is taken as a baseline for paging collision avoidance solution (for both EPS and 5GS).

Proposal 6: UE-requested 5G-GUTI reassignment (i.e. option 1) for 5GS and Offset based approach (i.e. option 2b) for EPS is selected for paging collision avoidance. Assistance information from UE is kept optional.

# Conclusion

RAN2 to discuss and agree on following observations and proposals:

Proposal 1: MUSIM UE determines potential paging collision on two networks and triggers actions on potential paging collision avoidance.

Proposal 2: It is left to UE implementation as to how it selects one of the two RATs/networks for paging collision avoidance.

Proposal 3: MUSIM UE may provide assistance information to network to resolve paging collision since involved networks are not coordinated. Access Stratum in the MUSIM UE builds assistance information for paging collision avoidance.

Observation 1: Maintaining UE context for idle mode UE at RAN to support paging collision avoidance is drastic change and have undesired impact.

Proposal 4: MUSIM UE utilizes NAS signaling to request potential paging collision avoidance to the network.

Proposal 5: Changing UE_ID via NAS signalling is taken as a baseline for paging collision avoidance solution (for both EPS and NR).

Proposal 6: UE-requested 5G-GUTI reassignment (i.e. option 1) for 5GS and Offset based approach (i.e. option 2 b) for EPS is selected for paging collision avoidance. Assistance information from UE is kept optional.

# References

[1] RP-202895, Revised WID: Support for Multi-SIM devices for LTE/NR

[2] Report of 3GPP TSG RAN2#112-e meeting (Draft_RAN2_112-e_Meeting Report_v2)

[3] TR 23.761 v1.2.0, Study on system enablers for devices having multiple Universal Subscriber Identity Modules