3GPP TSG-RAN WG2 Meeting #113-e    							     R2-2100462

e-Meeting, 25th Jan– 5th Feb, 2021							(Revision of R2-2009087)

Source: 	vivo, Guangdong Genius

Title:  	RRM Relaxation for Power Saving

Agenda Item:	8.12.3.2

Document for:	Discussion and Decision

Introduction

One of the objectives in the RedCap SID is following:

Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant) [RAN2, RAN1]: - Reduced PDCCH monitoring by smaller numbers of blind decodes and CCE limits [RAN1]. - Extended DRX for RRC Inactive and/or Idle [RAN2] - RRM relaxation for stationary devices [RAN2]

In this contribution, we discuss the potential directions for RRM relaxation enhancement for RedCap UE.

Discussion

According to the SID[1], three typical use cases of RedCap devices need to be considered:

Use case specific requirements: - Industrial wireless sensors: Reference use cases and requirements are described in TR 22.832 and TS 22.104: Communication service availability is 99.99% and end-to-end latency less than 100 ms. The reference bit rate is less than 2 Mbps (potentially asymmetric e.g. UL heavy traffic) for all use cases and the device is stationary. The battery should last at least few years. For safety related sensors, latency requirement is lower, 5-10 ms (TR 22.804) - Video Surveillance: As described in TR 22.804, reference economic video bitrate would be 2-4 Mbps, latency < 500 ms, reliability 99%-99.9%. High-end video e.g. for farming would require 7.5-25 Mbps. It is noted that traffic pattern is dominated by UL transmissions. - Wearables: Reference bitrate for smart wearable application can be 5-50 Mbps in DL and 2-5 Mbps in UL and peak bit rate of the device higher, up to 150 Mbps for downlink and up to 50 Mbps for uplink. Battery of the device should last multiple days (up to 1-2 weeks).

As highlighted above, both industrial wireless sensors and wearables require much longer battery lifetime than smart phones. On top of the power saving mechanisms have been specified for smart phone in Rel-16, further enhancement should be studied at least for industrial wireless sensors and wearables use cases.

Proposal 1: Further UE power saving and battery lifetime enhancement should focus on both industrial wireless sensors and wearables use cases.

According to the study on RRM relaxation in Rel-16 power saving WI, the directions for RRM relaxation enhancement are different for idle/inactive state and connected state, we will discuss the solutions for idle/inactive state and connected state separately in the following.

  - RRM Relaxation Enhancement for idle/inactive UE

According to our statistics for 4G smart phones and 4G smart watches, their time ratio of idle state is 80% and 90%, respectively. The difference in time ratio of idle state is because of the user habits and services types supported by different devices. We expect the time ratio of idle/inactive state for industrial wireless sensors are much higher than wearables since their batteries need to last at least a few years. Therefore, to improve the battery life for industrial wireless sensors and wearables, power saving enhancements for idle/inactive state is critical.

Observation1: To improve the battery life, power saving enhancements for idle/inactive state are critical, since the industrial wireless sensors and wearables are in idle/inactive state for more than 90% of the time.

In Rel-16, RRM relaxation for different scenarios were evaluated and discussed in the study phase. Finally, the RRM relaxation for neighboring cells (both inter-/intra-frequency) is specified for NR UEs which are not at cell edge and/or in low mobility state in Rel-16 Power Saving WI. However, no relaxation for serving cell RRM measurement was allowed. Actually, the RRM relaxation for serving cell has been evaluated and discussed in Rel-16 [2], but not been specified due to limited scope.

Observation 2: In Rel-16 NR, RRM relaxation for neighboring cells is already specified while for serving cell has not been specified yet due to limited scope.

In Rel-15, RRM relaxation for serving cells is specified for idle NB-IoT UEs, given regular RRM measurement for cell reselection is somehow considered as unnecessary for the devices which have limited mobility or stationary. With the enhancement, an idle NB-IoT UE is allowed to skip serving cell measurements and use WUS for synchronization up to at most 10.24 seconds, if it is configured with Wake-Up Signal (WUS), and the serving cell S criterion is met with at least 2 dB margin and the relaxed monitoring criteria for neighbor cells is fulfilled [3].

Observation 3: In Rel-15 NB-IoT, RRM relaxation for the serving cell has been introduced for limited mobility or stationary idle UE.

In many use cases of low-end RedCap (e.g. industrial wireless sensor), the UEs are stationary. In another RedCap use case (e.g. wearable), the UEs are in low mobility at least in some time duration such as when the user is at home at night or in the office during the day. In these use cases, the measurement relaxation for serving cell can be further considered for power saving as we have done for NB-IoT. Rel-16 RRM relaxation and the mechanism in NB-IoT can be considered as the baseline.

Some initial evaluation of serving cell RRM relaxation in idle mode can be found in Annex. The simulation results show it is possible for RedCap UEs to process SSBs in serving cell once per 5.12 seconds and about 10% power saving gain can be achieved (details can be found in Observation A@Annex).

Proposal 2: RRM relaxation for serving cell should be discussed and specified for idle/inactive industrial wireless sensors/wearables which are stationary or in low mobility in WI phase.

  - RRM Relaxation Enhancement for Connected UE

For RRC connected UEs, it is required for the UE to derive one L3 sample per 200ms at the maximum. It is obvious that such frequent measurement may not be always necessary for stationary or low mobility UEs, e.g. some RedCap sceanrios, like industry sensors. Power saving enhancement to reduce the measurement samples has been widely discussed in LTE NB-IoT design which is proofed to be effective. In the study phase of Rel-16 Power Saving, RRM relaxation in connected state has also been studied and evaluated. The details of the simulation assumption can be found in Annex [4]. The conclusion for power saving benefit due to RRM measurement relaxation in the TR [5] of Rel-16 Power Saving study phase is copied below for reference and the gain related to connected state is highlighted in yellow.

For adapting/relaxing RRM measurement in time domain - 11.1% - 26.6% and 7.4% - 17.8% power saving gains are shown for increasing measurement period 4 times and 2 times respectively for RRC CONNECTED state, 17.9%-19.7% and 0.89%-5.36% power saving gains are shown RRC IDLE/INACTIVE state by increasing measurement period for 1 SSB burst set for measurement and periodic activities and 2 or 3 SSB burst set for measurement and periodic activities respectively. - For stationary or low mobility (e.g., 3km/h) case, increasing measurement period has less impact (e.g., handover failure rate changes from 0% to 0.26% for 3km/h by extending 4 times measurement period) to the mobility performance compared to high mobility cases (e.g., handover failure rate changes from 0%-1% for 60km/h by extending 4 times measurement period). For adapting/relaxing RRM measurement for intra-frequency measurement by reducing the number of measured cells, - By assuming number of neighbouring cells to be measured is reduced, it is shown that about 4.7% - 7.1% power saving gain can be observed if reducing the number of measured cells for IDLE state. 1 source shows that about 1.8% - 21.3% power saving gain can be observed if reducing the number of measured cells for CONNECTED state. - In additional to that, by also assuming that UE can limit the processing for measurement within a constrained time period and/or with reduced complexity, 26.43% - 37.5% power saving gain is shown. - The corresponding performance impact was not reported and summarized For additional resource for RRM measurement - For RRC IDLE/INACTIVE, by using additional resources for RRM measurement, the UE power saving gain is 19%~38%. Analysis on network architecture impact, higher-layer/PHY-layer signaling overhead and network energy consumption is not provided. Two sources report 1.6% of SSB overhead for 160ms additional reference signals periodicity assuming that SSB burst set periodicity is 20ms. If static UE specific signals (e.g., CSI-RS) for connected mode UEs can be reused as additional resource for RRM, reference signal overhead may not increase. - For RRC CONNECTED, one source showed that, by having CSI-RS repetition for CSI-RS based RRM, the UE power saving gain is 11.8%~20.7% by assuming that CSI-RS overhead of baseline is identical to CSI-RS overhead with enhancement. The analysis on handover failure rate is not provided under the assumed overhead. For reducing the number of measured inter-frequency layers - 2 sources show that reducing the number of measured inter-frequency layers can provide 21%~38% power saving gain for RRC CONNECTED states, and 1 source shows 14%~35% power saving gain for RRC IDLE states by assuming reducing inter-frequency layers from 6 to 3. The mobility performance impact is not provided. It is noted that for RRM evaluation, PDCCH-only monitoring without data is assumed for RRC CONNECTED state.

Observation 4: The power saving benefit by reducing RRM measurement for connected state has been concluded in the TR of Rel-16 power saving SI.

In addition, during the email discussion in [6] after RAN2#112e meeting, neighbour cell RRM relaxation in RRC_CONNECTED was discussed. According to the discussion, majority companies think neighbour cell RRM relaxation in RRC_CONNECTED should be supported in the WI phase for fixed or immobile UE (supported by 14 companies out of 17 companies ) and for slightly moving UE (supported by 10 companies out of 17 companies). Hence, we suggest to recommendate the RRM relaxation in connected state for stationary/low mobility RedCap UEs in the WI decription for RedCap.

Proposal 3: RAN2 to recommendate the RRM relaxation in connected state for stationary/low mobility RedCap UEs in the WID of RedCap.

Conclusions

In this contribution, we discuss the potential directions for RRM relaxation enhancement for power saving in RedCap. Based on the discussion, we have the following observations and proposals:

Observation1: To improve the battery life, power saving enhancements for idle/inactive state are critical, since the industrial wireless sensors and wearables are in idle/inactive state for more than 90% of the time.

Observation 2: In Rel-16 NR, RRM relaxation for neighboring cells is already specified while for serving cell has not been specified yet due to limited scope.

Observation 3: In Rel-15 NB-IoT, RRM relaxation for the serving cell has been introduced for limited mobility or stationary idle UE.

Observation 4: The power saving benefit by reducing RRM measurement for connected state has been concluded in the TR of Rel-16 power saving SI.

Proposal 1: Further UE power saving and battery lifetime enhancement should focus on both industrial wireless sensors and wearables use cases.

Proposal 2: RRM relaxation for serving cell should be discussed and specified for idle/inactive industrial wireless sensors/wearables which are stationary or in low mobility in WI phase.

Proposal 3: RAN2 to recommendate the RRM relaxation in connected state for stationary/low mobility RedCap UEs in the WID of RedCap.

Reference

- RP-201386, Revised SID on Study on support of reduced capability NR devices，Ericsson

- R2-1904155,	Email report 105_56 - RRM related aspects for power saving	Ericsson	report	Rel-16	FS_NR_UE_pow_sav

- TS 36.133, Requirements for support of radio resource management

- R2-1905962, Evaluation on the mobility impact for RRM measurement relaxation, vivo

- TR 38.840, Study on UE power saving

- R2-2100569, Report of Email discussion[155][REDCAP] RRM relaxations, ZTE Corporation, Sanechips

Annex

5.1 Evaluation of serving cell RRM relaxation in idle mode

Our simulation assumptions and simulation cases are summarized in the following:

- Simulation Assumptions:

- Paging rate = wake-up signal/channel (WUS) paging rate = 10%, referring to that the probability for WUS indicating the UE to wake up and receive paging PDSCH is 10%.

- The time-domain gap of SSB relative to Paging Occasion (PO) is 10ms.

- The gap between WUS and SSB is 0 or 3ms.

- No time gap between WUS and SSB

- 𝑃WUS= 𝑃PO = 𝑃SSB =50 power units/slot, where 𝑃WUS, 𝑃PO and 𝑃SSB represent the relative power of WUS, PO, SSB respectively.

- Scaled to a 20MHz receiving bandwidth from the 100MHz power model in TR38.840

- TWUS= TSSB = 2ms, where TWUS and TSSB denote the duration of WUS and SSB.

- One SSB per DRX cycle (paging cycle) is processed in high SINR case

- Simulation cases:

- Rel-15 paging monitoring without WUS

- Case1: No serving cell RRM relaxation is assumed.

- Case2: 4x serving cell RRM relaxation, i.e. only one SSB is measured every four paging cycles.

- WUS for paging monitoring, no gap between WUS and SSB

- Case3: No serving cell RRM relaxation is assumed.

- Case4: 4x serving cell RRM relaxation, i.e. only one SSB is measured every four paging cycles.

- WUS for paging monitoring, the gap between WUS and SSB is 3ms

- Case5: No serving cell RRM relaxation is assumed.

- Case6: 4x serving cell RRM relaxation, i.e. only one SSB is measured every four paging cycles.

For Case 1, UE needs to receive SSB and monitor PO per paging cycle. For Case 2, with RRM relaxation, UE only perform SSB measurement every four paging cycles. As shown in Figure 1, in the last three paging cycles, power consumption is saved by skipping 75% of SSB processing and longer deep sleep duration can be obtained.

Case 3 and Case 4 take WUS into account, but there is no gap between WUS and SSB. Because the WUS paging rate is 10%, we only consider the case that WUS indicates UE not to wake up to receive Paging in the PO. With RRM relaxation, only one SSB is measured every four paging cycles for Case 4. Reduced SSB measurement leads to longer deep sleep.

Case 5 and Case 6 take WUS into account, and the gap between WUS and SSB is assumed as 3ms with considering the WUS processing time. The other assumptions are no changed compared to Case 3 and Case 4.

Figure 1 Ilustration for four evaluation cases

According to the simulation assumptions, the simulation results are summarized in Table 1. Case 2 can achieve 13.4% power saving gain over Case 1 thanks to the RRM relaxation. When WUS is configured and no gap between WUS and SSB, Case 4 reduces 3.6% power consumption over Case 3. Besides, when WUS is configured and the gap between WUS and SSB is 3ms, Case 6 reduces 8.3% power consumption over Case 5. Therefore, it is beneficial to support serving cell RRM relaxation for high SINR RedCap UE in idle mode.

Table 1 Simulation results of RRM relax in high SINR case

| Cases |  | Average relative power per slot | Power saving gain |
| --- | --- | --- | --- |
| w/o WUS | Case 1 | 1.6975 | 13.4% (case 2 over case 1) |
|  | Case 2 | 1.4709 |  |
| w/ WUS no gap between WUS and SSB | Case 3 | 1.5367 | 3.6% (case 4 over case 3) |
|  | Case 4 | 1.4814 |  |
| w/ WUS the gap between WUS and SSB is 3ms | Case 5 | 1.627 | 8.3% (case 6 over case 5) |
|  | Case 6 | 1.4914 |  |

Observation A: 3.6%~13.4% power saving gain can be observed if serving cell RRM relaxation is introduced for high SINR UE.

Compared with Case1, for case 2, UE does not need to receive SSB every paging cycle, and the power saving gain is achieved by skipping (N-1) SSB receptions every N paging cycle. Typically, UE receives SSB for synchronization to guarantee the reliable reception of paging PDCCH and PDSCH. However, the time drift caused by local oscillator is quite slow so that UE may still maintain good timing even if SSB receptions are skipped in several paging cycles.

Based on the time drift model as agreed in LTE eMTC/NB-IOT, we evaluated the time drift caused by oscillator after a number of DRX cycles. As shown in Table 2 that it is feasible for UE to process SSB every 5.12 seconds in order to maintaining good time synchronization, assuming 1/3 CP timing drift is acceptable.

Table 2 Time drift due to oscillator with 5ppm frequency error

| Sync periodicity(s) | 0.32 | 0.64 | 1.28 | 2.56 | 5.12 | 1/3 CP_length (30kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Time drift(s) | 2.56×10-9 | 1.02×10-8 | 4.10×10-8 | 1.64×10-7 | 6.55×10-7 | 7.81×10-7 |

To investigate the demodulation impacts due to 1/3 CP timing drift, we performed the link-level evaluation as the following.

In the evaluation, we assume the receiver intercepts the time domain signal 1/3 CP ahead of the OFDM signal, as shown in Figure 2. Thus, even if UE suffers from time drift up to 1/3 CP length, the starting time of intercepted signal is still within CP duration, UE can still decode the DL transmission, e.g. paging PDCCH and paging PDSCH. We evaluated the performance of PDCCH and PDSCH with different starting time of intercepted signal, the evaluation assumptions are given in Table A-1 and A-2 in appendix, and the evaluation results are shown in Figure 3 and Figure 4.The results prove that a 1/3 CP timing drift is acceptable for paging reception.

Figure 2 Illustration of time domain signal interception for FFT transform at receiver

Figure 3 Performance of PDCCH with different starting time of intercepted signal

Figure 4 Performance of PDSCH with different starting time of intercepted signal

Note that, since NB-IoT UEs are low cost devices, the time drift caused by oscillator using the model in NB-IoT can be considered as the worst cases for Redcap UEs. Even if considering the worst case, UE are still allowed to receive SSB for synchronization every 5.12s with limited performance loss. In addition, considering the RRM relaxation can be applied to high SINR UEs, the performance loss due to sub-optimal timing is negligible.

Observation B: It is feasible for RedCap UEs to process SSBs once per multiple paging cycles, assuming 1/3 CP timing drift tolerance and NB-IOT/eMTC timing drift model, it is possible for UE to process SSBs once per 5.12 seconds.

5.2 Evaluation of serving cell RRM relaxation in connected mode

In order to further study and confirm the above mobility impact for relaxed measurement period in time-domain on mobility performance from RAN2 perspective, system simulations are performed with different UE speeds and varied the measurement period.

The simulation scenario is dense urban deployment with 7 sites (3 cells per site, hexagonal grid) where each cell is covered by a sector antenna. Single frequency layer was assumed, thus no inter-frequency measurements were considered. More detailed simulation assumptions are listed in Annex. In these simulations, UE measures [1, 2, 5] samples during one measurement period and the measurement period is varied within 200, 400 and 800ms with UE speeds of 0 km/h,30km/h and 60km/h. That is to say, L3 filter period is from 200ms to 800ms. And L1/L3 sample interval is 40ms if 5 samples per measurement period of 200ms is assumed. Time to trigger value of 0ms, 320ms and 640ms were used respectively.

UE mobility/handover model and mobility performance metric follows TR36.839, which is the most detailed RAN2 specification for mobility performance evaluation. An important metric for mobility performance is handover failure rate, number of failed handovers divided by total number of handovers. In simulations the handover failure may occur, i.e., in a case where handover does not succeed due to UE not being able to send measurement report to network or UE is not being able to receive HO command or failure to access target cell. And the evaluation procedure also follows TR36.839, where the handover is initiated based on an A3 RSRP measurement event (A3 event is also the major event for handover triggering from practical deployment).

Detailed simulation assumptions are following:

| Parameter | Assumption |
| --- | --- |
| Cell layout | Hexagonal grid, 7 sites, 3 cells per site, with wrap-around Dense urban with one frequency layer(i.e. 4.0GHz) |
| Macro inter-site distance | 200 m |
| UE mobility | UE speed 3 and 30km/h UEs with random direction and straight movement |
| UE deployment | Uniform UE deployment with 20% UEs outdoor and 80% UEs indoor |
| BS antenna configuration | 1 sector antenna per sector Antenna downtilt 15 degrees |
| Macro BS Tx power | 44 dBm |
| Channel model | NR channel model |
| Pathloss model | Uma |
| Network load | 100% RB usage |
| Measurement interval | 200, 400, 800ms |
| L1 measurement filter samples | 5,2,1 |
| Layer3 Filter Parameter K | 1 |
| Handover criteria | Event A3, RSRP |
| A3 event threshold | 2 dB |
| A3 event hysteresis | 1 dB |
| A3 event time-to-trigger | 0 ms, 320 ms, 640ms |
| Radio Link Failure Qout SINR | -8 dB |
| Radio Link Failure Qin SINR | -6 dB |
| Radio Link Failure timer T310 | 1000 ms |

Handover failure rate = (number of handover failures) / (Total number of handover attempts)

Figure 5.2.1.3.1: A handover failure is declared when the criterion 1) is met in state 2. [TR36.839]

Figure 5.2.1.3.2: A handover failure is declared when the criterion 2) is met in state 2.