3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2100465

# Online, Jan 25 th - Feb 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0332 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to TS 38.300 for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Since the establishment of DRBs is optional, SDAP sublayer should be optional for IAB-MT, this was agreed at RAN2#110-e meeting. RAN2#110e agreement [048] Ph1-5: SDAP related features in TR 38.822 (5-1, 5-2) are optional for IAB-MT.. Therefore Layer 2 of IAB-MT should include MAC, RLC, BAP, PDCP and optionally SDAP, the current description for Layer 2 of IAB-MT in clause 6.1 is not accurate. - For UL, an IAB-node can perform transmission to either a parent IAB-node or IAB-donor. But the illustration in Figure 6.1-5 misses the IAB-donor case and should be updated. - The services and functions of the BAP sublayer described in clause 6.11.1 are not in accordance with TS 38.340, the polling signalling should also be added. |
| Summary of change: |  | - Revise the Layer 2 description for IAB-MT to optionally include SDAP sublayer in clause 6.1. - Update the illustration in Figure 6.1-5 to include IAB-donor case. - Add polling signalling function in clause 6.11.1. Impact analysis Impacted functionality: Layer 2 of IAB-MT Inter-operability: - If the IAB-MT is implemented according to this CR but the network is not, there is no inter-operability issue foreseen. - If the network is implemented according to this CR but the IAB-MT is not, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The contents between different specifications are not aligned, which may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 Overview 6.11.1 Services and Functions |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.1	Overview

The layer 2 of NR is split into the following sublayers: Medium Access Control (MAC), Radio Link Control (RLC), Packet Data Convergence Protocol (PDCP) and Service Data Adaptation Protocol (SDAP). The two figures below depict the Layer 2 architecture for downlink and uplink, where:

-	The physical layer offers to the MAC sublayer transport channels;

-	The MAC sublayer offers to the RLC sublayer logical channels;

-	The RLC sublayer offers to the PDCP sublayer RLC channels;

-	The PDCP sublayer offers to the SDAP sublayer radio bearers;

-	The SDAP sublayer offers to 5GC QoS flows;

-	Comp. refers to header compression and segm. to segmentation;

-	Control channels (BCCH, PCCH are not depicted for clarity).

NOTE:	The gNB may not be able to guarantee that a L2 buffer overflow will never occur. If such overflow occurs, the UE may discard packets in the L2 buffer.

Figure 6.1-1: Downlink Layer 2 Structure

Figure 6.1-2: Uplink Layer 2 Structure

Radio bearers are categorized into two groups: data radio bearers (DRB) for user plane data and signalling radio bearers (SRB) for control plane data.

For IAB, the Layer 2 of NR includes: MAC, RLC, Backhaul Adaptation Protocol (BAP), PDCP and optionally SDAP.

-	The BAP sublayer supports routing across the IAB topology and traffic mapping to BH RLC channels for enforcement of traffic prioritization and QoS.

Figures 6.1-3 below depicts the Layer-2 architecture for downlink on the IAB-donor. Figure 6.1-4 and 6.1-5 depict the Layer-2 architecture for downlink and uplink on the IAB-node, where the BAP sublayer offers routing functionality and mapping to BH RLC channels.

Figure 6.1-3: DL L2-structure for user plane at IAB-donor

Figure 6.1-4: DL L2-structure at IAB-node

Figure 6.1-5: UL L2-structure at IAB-node

NEXT CHANGE

6.11.1	Services and Functions

The main service and functions of the BAP sublayer include:

-	Transfer of data;

-	Routing of packets to next hop;

-	Determination of BAP destination and BAP path for packets from upper layers;

-	Determination of egress BH RLC channels for packets routed to next hop;

-	Differentiating traffic to be delivered to upper layers from traffic to be delivered to egress link;

-	Flow control feedback and polling signalling;

-	BH RLF indication.

END OF CHANGES