3GPP TSG-RAN WG2 #113e	R2-2100491

Electronic meeting, 25th Jan – 5th Feb, 2021

Agenda Item:	8.16.3

Source:	Ericsson

Title:	UE onboarding

Document for:	Discussion

# 1	Introduction

The Rel-16 Private Network (PRN) work item (WI) [1] gives basic functionality support for Non-Public Networks (NPNs). Two different deployment scenarios for NPNs have been specified in 5GS: Standalone NPNs (SNPNs) and Public Network Integrated NPNs (PNI-NPNs).

To further enhance NPNs, SA2 started an eNPN Study Item (SI), see SP-190453 (with updated work plan in SP-191376) [2] resulting in TR 23.700-07 [4]. As the SA2 eNPN work causes impact on RAN, the Rel-17 ePRN WID, RP-202363 [3], was approved at RAN#90e in December 2020 addressing three different objectives.

In this paper, we focus on covering the following objective as copied from the WID (only RAN2 impacts listed here):

- Support UE onboarding and provisioning for NPN including:

  - The UE onboarding relevant parameter broadcast from SIB [RAN2]

  - The associated cell selection/reselection, cell access control and the connected mode mobility support [RAN2/RAN3]

Key Issue (KI) #4 in SA2’s study [2], which is the one related to this objective, is divided in two components: 1) UE onboarding and 2) remote provisioning. The first component seeks to enable 3GPP connectivity for UEs that only have default credentials, so that these UEs can be provisioned with the necessary network credentials following second component’s solutions. RAN impacts are limited to the first component and thus, we here focus on UE onboarding.

# 2	Discussion

## 2.1	General description

The process of onboarding per se has different implications and meaning depending on the type of NPN deployments. In fact, as we will see below, one could argue that onboarding is ultimately focused exclusively on SNPNs. However, regardless of the deployment and irrespective of how the onboarding procedure is undertaken, it is worth mentioning that the NPN credentials that are later remotely provisioned to the UE (KI#4 second component) are then usually stored/saved by the UE. Hence, we consider it important to start by mentioning that onboarding is likely to be carried out only once in the lifetime of the UE. Therefore, this procedure should not be considered as time-critical in any way.

Onboarding is not a time-critical procedure.

Building on the previous discussion, we now discuss what onboarding means and how the principle works for both PNI-NPNs and SNPNs.

### 2.1.1	UE onboarding to a PNI-NPN

The following NOTE is captured in TR 23.700-07 [4], clause 5.4.1:

NOTE 1:	Provisioning of PLMN credentials is not in scope of this KI. A UE accessing a PLMN is assumed to have provisioned 3GPP credentials.

PNI-NPN deployments rely on PLMNs and can be achieved by e.g., allocating one or more network slices to the NPN. To prevent non intended UEs from accessing the NPN, Closed Access Groups (CAGs) are broadcast in SIB1 and identify a group of subscribers who are permitted to access one or more CAG cells associated to the CAG as described in TS 38.300 [3]. In turn, UEs can be configured with an “Allowed CAG list” which contains the identities of the CAGs that the UE is allowed to access within the PLMN. With this, a UE holding PLMN credentials can access the network and, in case of needing PNI-NPN credentials to, e.g. access a PNI-NPN slice, the subsequent provisioning is already enabled by current specification.

-	No enhancement for the UE onboarding (component 1 of KI#4) with PLMN credentials used for primary authentication and PLMN network selection are needed for the case of PNI-NPN credentials provisioning.

In fact, SA2’s study concludes that for KI#4 no enhancements are needed for PNI-NPN deployments, see TR 23.300-07 [4], clause 8.4.2:

Therefore, there is no need to study scenarios where the PLMN would be the subscription owner.

Moreover, the meaning of “onboarding” for PNI-NPNs is perhaps not clear and could also refer to alternative existing situations. For instance, Rel-16 allows manual CAG selection of a CAG that is not part of the allowed CAG list of the UE’s NAS configuration. Consequently, the subscription can be extended with new CAG IDs, which could also (arguably) be regarded as onboarding. Even more so, when an NPN is described by a specific network slice and a UE (with PLMN subscription) is also provisioned with other slices, legacy specification allows for the subscription to be extended with the NPN slice upon request. This could also be considered as onboarding. Therefore, given these ambiguities and the lack of impact foreseen for RAN, we do not need to consider UE onboarding in PNI-NPN deployments.

There is no need to add RAN support for onboarding in PNI-NPN deployments as the UE has provisioned PLMN credentials.

### 2.1.2	UE onboarding to an SNPN

An onboarding related use case for SNPNs could be, e.g. UE devices being delivered to a factory and needing to register to the factory’s SNPN. If the UEs only have some basic default credentials but lack network credentials, it is not possible for these to gain access to the SNPN by legacy procedures. In this regard, the goal is to allow the core network to identify and authenticate the UE by means of these default credentials.

TR 23.700-07 [4] describes the onboarding process for SNPNs as illustrated in Figure 1. Firstly, the UE should seek connectivity to an Onboarding Network (ON), to which it performs initial registration and then the latter, authenticates the UE by accessing a Default Credential Server (DCS), which could be, for example, the device manufacturer’s server. Secondly, the ON provisions the UE with the subscription data, which is provided by a Subscription Owner SNPN (SO-SNPN) through a Provisioning Server (PS). In general terms then, it is possible to say that ultimately the SO-SNPN corresponds to the actual NPN and the ON acts as a “bridge” providing (initial) connectivity to UEs with insufficient NPN credentials. The nodes in Figure 1 are considered logical entities and their actual physical location is not discussed in this contribution. Indeed, e.g. in a factory, the ON could belong to the SO-SNPN and in this case, there is no need to differentiate between the two

Figure 1: UE onboarding to an SNPN.

In this paper, we use the following abbreviations:

- ON = Onboarding Network

  - Can be either an SNPN (which is then shortened as O-SNPN) or a PLMN

- SO-SNPN = Subscription Owner SNPN

- PS = Provisioning Server

In this contribution, we focus on the O-SNPN case for UE onboarding to the SO-SNPN.

Before going into details how onboarding could be achieved, it should be pointed out that already during the SA2 study, similarities between onboarding and LTE’s Rel-16 Restricted Local Operator Services (RLOS) surfaced. RLOS allows unauthenticated UEs in limited service state to get access to restricted services. In this regard, AS/NAS security is not (necessarily) used when carrying out an Attach procedure of type RLOS, which would not be the case when performing onboarding. However, since it is also necessary for onboarding to give (limited) access to a UE which is not yet authenticated, we do believe that it is possible to use the AS level signalling principles of the LTE RLOS feature specification, .e.g. indications in SIB and RRC messages, as a baseline for developing a specific solution for the present objective.

Given the similarities with RLOS, the existing RRC signalling procedure specified for RLOS in LTE should be used as a basis for developing an onboarding solution.

Below we propose a way on how a UE can discover and select an ON before being provisioned with SO-SNPN credentials and other information that enable 3GPP connectivity.

## 2.2	UE onboarding support in an O-SNPN

### 2.2.1	Indicating support for onboarding

Since a UE delivered to an SNPN is expected to have minimum pre-configuration, it does not know which networks provide onboarding support. In this regard, TR 23.700-07 [4] concluded the following in clause 8.4.1:

-	The NG-RAN of the Onboarding network includes an indication for Onboarding enabled in the SIB (per O-SNPN, considering that the NG-RAN can be shared) so that the UE can discover and select an appropriate O-SNPN. […]

Thus, the indication should be per O-SNPN. Furthermore, as onboarding networks are not necessarily only used for onboarding purposes, but may also provide UEs with other services, it should be possible for the network to decide whether to enable/allow the onboarding feature.

Each O-SNPN broadcasts a 1-bit onboardingAllowed indication.

The onboardingAllowed indication could be transmitted in SIB1, an NPN-related SIB, i.e. SIB10, or even a new SIB if there is more information to be broadcast in relation to onboarding.

In order to decide in which SIB to broadcast the onboardingAllowed indication, different aspects need to be considered, e.g. as observed above, onboarding is not a time-critical procedure and thus, the indication does not need to be broadcast frequently. Hence, a first approach could be to broadcast the onboardingAllowed indication in SIB10, which is already defined for NPN-related purposes and contains the Human Readable Network Names (HRNNs) of the NPNs listed in SIB1. This would come in line with the fact that onboarding is not a time-critical procedure, since SIB10 is typically configured with a much lower frequency than SIB1. Yet, SIB10 is used for manual selection while onboarding is for automatic SNPN selection and the ON is basically used to provide connectivity to UEs with insufficient NPN credentials.

On the other hand, we consider that a single bit per SNPN is enough to allow networks to indicate their ability of allowing onboarding. In this sense, clause 8.4.1 also concludes the following:

-	[…] The UE may or may not be pre-configured with O-SNPN network selection information (e.g. O-SNPN network identifiers).

NOTE 2:	Whether the indication for Onboarding is sufficient or more SIB information is needed can be further discussed in the normative phase.

In this regard, since the onboarding process is not time-critical and probably occurs only once in the life of the UE or at least extremely seldom, we consider that specifying and broadcasting further onboarding-related information would not be justified. Consequently, if multiple ONs are available, it should be possible to proceed under a trial-and-error basis, leaving the selection order to UE implementation. Thus, if a first onboarding attempt would be rejected by the network, the UE would move to another ON to evaluate whether it can get access there.

Only the onboardingAllowed indication and no further onboarding information is broadcast by the RAN.

So, as long as only a single bit needs to be broadcast per O-SNPN, adding the indication in SIB1 would be acceptable, since this helps the UE to quickly decide whether it can select the cell or if it should continue with the cell selection procedure. Furthermore, this indication can be used to control access for onboarding requests, see next sections. Access control parameters are typically broadcast in SIB1.

Assuming that only the onboardingAllowed indication and no further onboarding information is broadcast, the bit should be included in SIB1.

### 2.2.2	RAN congestion control

A note was captured in the SA2 TR 23.700-07 [4] on the possibility of discussing with RAN WGs whether handling of RAN-level congestion is feasible. We consider this to be achievable using either the onboardingAllowed indication to control congestion or Unified Access Control (UAC) mechanism.

1) Using the onboardingAllowed indication to control congestion

One possible way of controlling RAN-level congestion would be to use the previously proposed onboardingAllowed indication. In this sense, the ON’s RAN could toggle the onboardingAllowed bit in the SIB to control congestion due to UE onboarding requests (see next section for onboarding requests) by controlling the access. This would imply the RAN to change the SIB whenever the congestion situation changes.

2) A new Access Class value can be defined for onboarding purposes

Another approach would be to introduce a new Access Class value, e.g. one of the reserved standardized access categories 11-31, for onboarding purposes, see TS 22.261 [5], clause 6.22, which would require specification work by SA1. While the toggling solution of the onboardingAllowed indication would bar all or none of the onboarding requests, the UAC mechanism allows for higher granularity by allowing the RAN to configure a barring factor and a barring time, which would however mean additional complexity in terms of specification work and also with regard to configuration as appropriate values need to be chosen.

UAC parameters are sent in SIB1 and may also, similar as the onboardingAllowed indication, need to be modified  whenever the congestion level in the cell changes. Furthermore, there is no SA1 requirement to introduce a new “Onboarding” Access Category, and as mentioned above, it is expected that the onboarding process will be carried out only once in the UE’s lifetime. Hence, such a level of granularity is arguably not justified.

Impact of SI changes when congestion situation changes in the ON

It can be assumed that an onboarding UE that finds a suitable onboarding network, i.e. after reading relevant system information, the UE would immediately initiate the onboarding procedure. This means that it would have the most recent system information before accessing the cell. If there is congestion on RAN level, only new UEs arriving in the onboarding network would need to evaluate the onboardingAllowed indication or the UAC parameters respectively, so that it is not necessary to send SIB change notifications when the previously mentioned parameters are modified.

It can be assumed that UEs would immediately initiate the onboarding procedure so that it is not necessary to inform all UEs about system information with regard to congestion control for onboarding.

In Table 1, we summarize and compare the above RAN congestion control mechanisms.

Table 1: Evaluation of RAN congestion control mechanisms.

|  | onboardingAllowed indication | New Access Category for onboarding |
| --- | --- | --- |
| General description | Affects all or none of the onboarding UEs | RAN can individually configure UAC parameters for onboarding, i.e. barring factor and time to control access probabilities |
| Specification impact | The purpose of the onboardingAllowed bit also includes congestion control | SA1 has to define a new UAC value for onboarding purposes; no corresponding SA1 requirements |
| Complexity | Simple solution | Higher complexity w.r.t. configuration. Granularity not justified given this is a once-in-a-lifetime procedure |
| SIB/SIB1 updates | onboardingAllowed indication is toggled depending on congestion situation (would not require SIB change notifications) | UAC parameter configuration for onboarding may need modification depending on congestion situation (would not require SIB change notifications) |

As shown above, both mechanisms can solve congestion problems in the network, i.e. either the onboardingAllowed bit can be toggled, or a new Access Class value for UAC can be introduced.

Congestion control can be achieved by simply toggling the onboardingAllowed indication or by introducing a new Access Class value for onboarding and using UAC.

However, the UAC solution requiring the introduction of a new Access Class value comes with a certain cost. It requires SA1 work and adds complexity with respect to granularity. Such granularity does not seem justified for a once-in-a-lifetime procedure.

Using UAC for onboarding requires SA1 work and adds some complexity with respect to granularity which is not justified for a once-in-a-lifetime procedure.

### 2.2.3		Onboarding request

Upon registration to an SNPN for Onboarding, the UE provides an indication at NAS level that the registration request is for onboarding […]

According to the conclusions captured in clause 8.4.1 of TR 23.700-07 [4], the following information is provided from the UE to the network for the purpose of onboarding:

However, before such a procedure can be initiated by the UE, the UE needs to ensure the network indicates that onboarding is allowed. Else, if onboarding is not supported or if not indicated as being allowed in the SIB, the UE is not allowed to send this type of request.

A UE can only request onboarding if the onboarding network indicates that onboarding is allowed.

Furthermore, the RAN of the onboarding network needs to select an AMF which supports UE onboarding. Yet, as the NAS level message indicating the onboarding request to the AMF is transparent to the RAN, an explicit onboarding indication needs to be provided from the UE to the RAN.

An explicit onboarding indication is helpful for the RAN to select an AMF which supports onboarding.

There are different approaches for a UE to indicate an onboarding request to the RAN. A first approach would be for the UE to indicate an onboarding request to the RAN in the RRCSetupRequest message (i.e., msg3). A second and more suitable approach, as explained below, mimics the RRC signalling procedure specified in LTE Rel-16 RLOS and includes the onboarding request in the RRCSetupComplete message (i.e., msg5). We now discuss how such a request would be carried out in each of the above options.

1) Onboarding request in msg3 (RRCSetupRequest)

To indicate onboarding request in msg3, a new EstablishmentCause value for this purpose would be needed. It should be noted that the msg3 size is limited, and thus, there is only a limited number of spare values reserved for the EstablishmentCause. This indication could allow the RAN to control congestion caused by onboarding requests after having received msg3. However, if congestion is caused by onboarding requests, the UE should not even be allowed to perform the random access procedure and be rejected only in msg4. If there is no congestion and since onboarding is not a time-critical service, there is no benefit to send the onboarding indication already in msg3, as such information is not needed for the RRCSetup (i.e. msg4). Thus, we think that using one of the reserved values of msg3 for onboarding purposes is not justified.

Using one of the reserved values as EstablishmentCause in msg3 is not justified for onboarding purposes.

2) Onboarding request in msg5 (RRCSetupComplete)

Similar to the RLOS indications in LTE Rel-16, the onboarding request can be indicated in msg5. It appears more logical to send the onboarding indication in msg5 which also carries the NAS level message containing the onboarding request (to be defined by CT1). For this, a new “onboardingRequest field would need to be defined on AS level.

The UE indicates the onboarding request in a new onboardingRequest field in the RRCSetupComplete message (msg5).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Onboarding is not a time-critical procedure.

Observation 2	There is no need to add RAN support for onboarding in PNI-NPN deployments as the UE has provisioned PLMN credentials.

Observation 3	It can be assumed that UEs would immediately initiate the onboarding procedure so that it is not necessary to inform all UEs about system information with regard to congestion control for onboarding.

Observation 4	Congestion control can be achieved by simply toggling the onboardingAllowed indication or by introducing a new Access Class value for onboarding and using UAC.

Observation 5	Using UAC for onboarding requires SA1 work and adds some complexity with respect to granularity which is not justified for a once-in-a-lifetime procedure.

Observation 6	An explicit onboarding indication is helpful for the RAN to select an AMF which supports onboarding.

Observation 7	Using one of the reserved values as EstablishmentCause in msg3 is not justified for onboarding purposes.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Given the similarities with RLOS, the existing RRC signalling procedure specified for RLOS in LTE should be used as a basis for developing an onboarding solution.

Proposal 2	Each O-SNPN broadcasts a 1-bit onboardingAllowed indication.

Proposal 3	Only the onboardingAllowed indication and no further onboarding information is broadcast by the RAN.

Proposal 4	Assuming that only the onboardingAllowed indication and no further onboarding information is broadcast, the bit should be included in SIB1.

Proposal 5	A UE can only request onboarding if the onboarding network indicates that onboarding is allowed.

Proposal 6	The UE indicates the onboarding request in a new onboardingRequest field in the RRCSetupComplete message (msg5).

# References

RP-191563, “Revised WID: Private Network Support for NG-RAN”, China Telecom, Nokia, Vodafone, RAN#84, 2019-06

RP-202363, “Revised WID: Enhancement of Private Network support for NG-RAN”, RAN#90e, 2020-12

TS 38.300, “NR and NG-RAN Overall description”, TSG RAN, V16.3.0, 2020-10

TR 23.700-07, “Study on enhanced support of non-public networks”, TSG SA, V1.2.0, 2020-11

TS 22.261, “Service requirements for the 5G system; Stage 1”, TSG SA, V17.4.0, 2020-09