3GPP TSG-RAN WG2 Meeting #113-e	R2-2100501

Online, January 25th – February 5th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2329 | rev | - | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the actions of sidelink measurement configuration in TS 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-1-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Uu interfac, if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, UE will remove the measurement reporting entry for this measId associated with the measObjectId from the VarMeasReportList and stop the corresponding timer as shown in following: The UE shall: 1> for each measObjectId included in the received measObjectToAddModList: 2> if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry: ..... 3> for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any: 4> remove the measurement reporting entry for this measId from the VarMeasReportList, if included; 4> stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId; ..... 2> else: 3> add a new entry for the received measObject to the measObjectList within VarMeasConfig. The Reason is after receiving and applying the measurement modification RRC signaling, UE should report the measurement results with new measurement object. And If measurement results with old measurement object is not removed, gNB can not distinguish which measurement result is associated with new measurement object. Similarly, In sidelink measurement object addition/modification, UE also needs to remove the measurement reporting entry for this sl-MeasId associated with sl-MeasObjectId from the VarMeasReportListSL, if an entry with the matching sl-MeasObjectId exists in the sl-MeasObjectList within the VarMeasConfigSL after receiving measurement configuration modification RRC sidelink signaling from peer UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions of removing measurement reporting entry from VarMeasReportListSL Impact analysis Impacted 5G architecture options: Standalone/Non-standalone Impacted functionality: Sidelink RRC Inter-operability: If the network implements the CR but the UE is not, , or vice-versa, there is no inter-operability issue. If the one UE implements the CR but the other does not, UE can not distinguish whether measurement result received from peer UE is associated to the new measurement object. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE can not distinguish whether measurement result received from peer UE is associated to the new measurement object. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.10.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

5.8.10.2.5	Sidelink measurement object addition/modification

The UE shall:

1>	for each sl-MeasObjectId included in the received sl-MeasObjectToAddModList:

2>	if an entry with the matching sl-MeasObjectId exists in the sl-MeasObjectList within the VarMeasConfigSL, for this entry:

3>	for each sl-MeasId associated with this sl-MeasObjectId included in the sl-MeasIdList within the VarMeasConfigSL, if any:

4>	remove the measurement reporting entry for this sl-MeasId from the VarMeasReportListSL, if included;

4>	stop the periodical reporting timer and reset the associated information (e.g. sl-TimeToTrigger) for this sl-MeasId;

3>	reconfigure the entry with the value received for this sl-MeasObject;

2>	else:

3>	add a new entry for the received sl-MeasObject to the sl-MeasObjectList within VarMeasConfigSL.

End of the change