3GPP TSG-RAN WG2 Meeting #113 Electronic 	R2-2100508

Elbonia, 25th of January – 5th of February 2021

Agenda item:	8.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Switching notification procedure for basic switching scenarios for Single RX UE

WID/SID:	LTE_NR_MUSIM-Core

Document for:	Discussion and Decision

# 1	Introduction

Rel-17 WI [1] for ‘support of Multi-SIM devices’ aims to study the system impacts when the UE communicates to more than one PLMN via multi-SIM support and specifies potential enhancements to avoid performance degradation in the MUSIM scenarios.

The objectives of the WI are:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2] - RAT Concurrency: Network A can be NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx. - Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2]: - RAT Concurrency: Network A is NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx - Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is VoLTE/VoNR [RAN2] - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx UE SIMs may belong to same or different operators. USIM can be a physical SIM or eSIM. Coordination with relevant WGs, such as SA2, should be considered where relevant. Specification change should focus on NR side for objective 1.

In this discussion paper we further discuss on the objective 2 and potential solutions relevant for the same.

# 2	Discussion

When MUSIM capable UE is registered at NTWK-A and NTWK-B, is in CONNECTED mode operation with NTWK-A, and in IDLE/INACTIVE mode in NTWK-B without capability to perform simultaneous RX and/or TX activities parallel to its connection, it may need to interrupt/gap its connection in NTWK-A for short/ medium length activities in NTWK-B or leave its connection for long activities in NTWK-B. While it is desired to have least possible impact on connected mode activities without a connection release it may be necessary to leave the connection in case the duration of absence is too long. Different solutions for different duration of activities in NTWK-B are needed for optimized UE performance and least NW impact. In RAN2-112e meeting it was agreed to further discuss on possible RRC level switching scenarios as referred below.

RAN2 will continue to discuss RRC-based switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED. There may be different mechanisms (short/long, leaving/returning, etc.).

In below sections we discuss the basic scenarios to be supported for single RX UE for objective 2 and the possible solutions. We further discuss the additional scenarios associated with switching notification in [4].

## 2.1 Switching for idle mode monitoring in NTWK-B

When MUSIM capable UE registered at NTWK-A and NTWK-B is in connected mode operation with NTWK-A, the UE may need to frequently switch to NTWK-B for idle mode operations. The idle mode activities which can be managed with static gaps are idle mode paging monitoring and serving cell measurements of idle mode. UE may need gaps for paging monitoring and serving cell measurements for every DRX cycle as specified in 38.133. Out of these two activities, the gaps required for paging monitoring are determined and fixed by the paging occasion of NTWK-B and this information is not known to NTWK-A, whereas, the gaps required for serving cell measurements per DRX cycle can be configured by NTWK-A depending on the traffic situation and distribution of scheduling gaps.

As the time required for the UE to listen to NTWK-B is limited for the above activities in the range of few sub-frames for set of radio frames, UE can inform the availability pattern /gap pattern statically at the time of RRC connection instead of notifying the switching for every occurrence.

In some existing UE implementations for MUSIM operation in LTE, the UE makes short absence for these situations without network knowledge. With NR system handling latency and reliability sensitive traffic flows this autonomous absence without network knowledge is not preferred.  Hence, indication of pattern of short absence for idle mode operation in other network will be required for NR operation. The indication can be included as part of UE capability reporting or, optionally, requested by the network if the network intends to avoid scheduling towards the UE during these gaps.

Observation 1: For some application scenarios of NR, the connected mode network needs to know the short absence of UE for idle mode operation on other USIM to minimise the impact of service interruption.

For the above scenarios, the UE may include the assistance information for gap configuration such as periodicity and offset within period in existing uplink RRC message instead of new RRC message for this purpose. Network can include the gap configuration via RRC Reconfiguration message.

Proposal 1: Periodic gap configuration for short leave scenarios related to idle mode monitoring should be supported. Existing RRC messages can be reused with additional parameters for this purpose. FFS Additional parameters.

In addition to the above idle mode operations, further idle mode monitoring may be required depending on radio condition and network changes. For example, if the serving cell measurements show degraded radio conditions, UE may need to start additional measurements on neighbouring cells for cell reselection and if cell reselection succeeds, the UE may need to synchronize to new cell and also acquire system information in the new cell. It is also possible that NTWK-B may change its system information and UE may need to monitor new system information by extending the downlink monitoring in the other network.

Cell Reselection and System Information monitoring in NTWK-B will require longer gaps than the gaps applicable for idle mode paging monitoring and serving cell measurements. For these situations switching indication about leaving NTWK-A along with the reason may be required. NTWK A may configure scheduling pause /gap for longer duration based on this notification to avoid losing of downlink messages.

The connected-idle mode operation with possible gaps for downlink monitoring is illustrated in the following figure. In this figure NTKW-1 refers to NTWK-A and NTWK-2 refers to NTWK-B.

Observation 2: Switching Notification without any state change at NTWK-A is required for idle mode mobility and system information change scenarios.

Proposal 2: Switching indication for short leave scenarios where the UE only needs to switch the downlink reception in other NTWK without uplink operation needs to be supported

## 2.3 Switching to other network for short signalling procedures

As part of idle mode operation with NTWK-B, UE may also need to establish RRC connection for some short signalling information in NTWK-B for Routing area update or RNA Update and short message services. For these scenarios, the UE will return to idle mode operation after short absence, but this signalling procedure requires UE also to trigger uplink message transmissions including RACH Access. UE with Single TX capability needs to suspend any uplink activity in other network for these scenarios.  Depending on the maximum time required to complete this signalling procedure, the connected mode operation with network operation can be maintained in connected mode without state transition.

Observation 3: Short signalling message procedures at NTWK-A can be completed without leaving the connected mode in NTWK-B in optimum scenarios.

Observation 4: The gap duration for short signalling procedure depends on the impact to the ongoing activities such as pending scheduling of connected mode operation.

Proposal 3: Switching indication for short leave scenarios where the UE is maintained in RRC-CONNECTED at NTWK-A should be supported.

## 2.4 Switching to NTWK-B for long time leave

When MUSIM UE is in CONNECTED-IDLE mode operation between NTWK-A and NTWK-B, the UE in IDLE mode may need to establish RRC connection in NTWK-B and indicate the leave notification to NTWK-A. For this long leave  notification, the signaling procedure can be either:

- procedure where the UE indicates the leave and waits for network response before leaving; or

- the UE may send RRC level indication as a procedure without network response.

For these two kinds of long leave indication, UE may additionally need to communicate at NAS level to inform the AMF about the leaving so that AMF can further buffer or pause the data transmission from AMF. In some scenarios, the UE may need to start the connection setup procedure with NTWK-B quickly due to latency requirements without waiting for NTWK-A response. For this scenario, leave indication without network response needs to be supported. The UE may need to leave the network right after sending the message containing RRC and NAS level Leave information and obtain lower layer acknowledgement itself.

Proposal 4: Leave indication for UE switching to NTWK-B with and without explicit response from NTWK-A should be both supported.

The decision for moving the UE to either RRC_IDLE or RRC_INACTIVE depends on the duration for which the UE is expected to be absent from NTWK-A. However, the time of absence may not be known at the time of leave. In such cases, whether the NTWK-A should move the UE to RRC_IDLE or RRC_INACTIVE is not known.

Observation 4: The duration of the UE’s leave from NTWK-A depends on the UEs activities in NTWK-B and as such might not be known in advance.

In case the leave indication without network response needs to be supported, the RRC state to which the UE should be moved at the time of leave can be pre-configured via RRC signaling.  The pre-configuration can also include the state transition information depending on the time of return. This pre-configured return behavior enables the UE to leave NTWK-A without waiting for the network response. NTWK-A may also configure the UE to remain in RRC_CONNECTED state using connected mode discontinuous RX, C-DRX, for a short duration before moving the UE to RRC_INACTIVE or directly to RRC_IDLE, depending on the configuration. With this pre-configuration, if the UE completes the NTWK-B operation in shorter duration it can resume its RRC connection after being in C-DRX.

Proposal 5: RAN2 discuss the RRC Configuration of the UE behavior when it switches from NTWK-A without waiting for network response.

For the long leave scenarios, AMF of NTWK-A also needs to know about the leaving event so that it can manage further signaling and pending transmission towards the UE. If the UE is configured with NAS level leave indication, the NAS level leave indication can also be included in the RRC message which indicates the leave at AS level. As per the conclusion for the key issue for coordinated leave in SA2, the interim conclusion indicates that NAS level Leave indication to be supported which may also require network response to the leave indication to configure some additional parameter related to idle mode monitoring after moving to IDLE state. RAN2 to further discuss about the RAN level impacts of supporting signaling procedure for NAS level leave indication including the impact of delay in switching to another network.

Proposal 6: RAN2 discuss the impacts of supporting NAS level Leave indication procedure and the applicable scenarios.

Proposal 7: Configuration of NAS level and RRC level leave indication procedure for long leave scenarios should be supported.

# 3	Conclusion

The following proposals and observations were made in this paper:

For short leave -idle mode activity in NTWK-A

Observation 1: For some application scenarios of NR, the connected mode network needs to know the short absence of UE for idle mode operation on other USIM to minimise the impact of service interruption.

Proposal 1: Periodic gap configuration for short leave scenarios related to idle mode monitoring should be supported. Existing RRC messages can be reused with additional parameters for this purpose. FFS Additional parameters.

Observation 2: Switching Notification without any state change at NTWK-A is required for idle mode mobility and system information change scenarios.

Proposal 2: Switching indication for short leave scenarios where the UE only needs to switch the downlink reception in other NTWK without uplink operation needs to be supported.

For short leave -idle mode activity in NTWK-A

Observation 3: Short signalling message procedures at NTWK-A can be completed without leaving the connected mode in NTWK-B in optimum scenarios.

Observation 4: The gap duration for short signalling procedure depends on the impact to the ongoing activities such as pending scheduling of connected mode operation.

Proposal 3: Switching indication for short leave scenarios where the UE is maintained in RRC-CONNECTED at NTWK-A should be supported.

For Long Leave scenarios

Proposal 4: Leave indication for UE switching to NTWK-B with and without explicit response from NTWK-A should be both supported.

Observation 4: The duration of the UE’s leave from NTWK-A depends on the UEs activities in NTWK-B and as such might not be known in advance.

Proposal 5: RAN2 discuss the RRC Configuration of the UE behavior when it switches from NTWK-A without waiting for network response.

Proposal 6: RAN2 discuss the impacts of supporting NAS level Leave indication procedure and the applicable scenarios.

Proposal 7: Configuration of NAS level and RRC level leave indication procedure for long leave scenarios should be supported.

# Annexure: RAN2 Agreements for Objective 2

- Agreements

Indicate to SA2 that the table 1 is a baseline on the discussion the expected time (in ms) required for UE to send a (NAS) busy indication to Network B.

From RAN2 point of view, it is feasible that the busy indication is sent as an RRC message with security for RRC_INACTIVE. FFS how this works.

RAN2 will continue to discuss RRC-based switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED. There may be different mechanisms (short/long, leaving/returning, etc.).

1a: The sub-Case 3-1 is supported in WI, i.e., the switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED  includes the case where Dual-RX/Single-TX UE is in RRC_CONNECTED state in NW A while performing only reception in NW B (i.e., in RRC_idle State and RRC inactive state).

1b: For Sub-Case 3-1, whether the Rx capability coordination between UE and NW is needed can be decided after the RRC-based switching/leaving and returning procedure is defined.

2: The Sub-Case 3-2, i.e. Dual-RX/Single-TX UE stays in RRC_CONNECTED mode in NW A while performing reception and transmission in NW B(in RRC_ CONNECTED or during RRC setup/resume period ), is not considered in the WI from RAN2 viewpoint. Scheduling gap is not excluded.

4: 	FFS: The Sub-Case 4-1, i.e. Dual-RX/Dual-TX UE stays in RRC_CONNECTED mode in NW A while performing both reception and transmission in NW B without changing into RRC_CONNECTED state in NW B, is not considered in the WI from RAN2 viewpoint.

5: 	FFS: The Sub-Case 4-2, i.e. Dual-RX/Dual-TX UE stays in RRC_CONNECTED state in NW A while performing both reception and transmission in RRC_ CONNECTED in NW B, is not considered in the WI from RAN2 viewpoint.

FFS if/how to ensure UE doesn't disconnect from RRC_CONNECTED during busy indication

Capability change is not precluded by proposals.

# References

- RP-193263 – WID For Support of Multi-SIM devices -Rel-17.

- TR 23.761 – Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM)

- RAN2-112e-Meeting-Report

- R2-210XXX – Additional Switching scenarios for MUSIM operation -RAN2-113-e , Nokia , Nokia Shanghai Bells.