3GPP TSG-RAN WG2 Meeting #113-e	R2- 2100536

e-Meeting, January 25th – February 5th, 2021                                 Revision of R2-2009231

Agenda Item:	8.15.2.1

Source:	Ericsson

Title:	General aspects for SL DRX

Document for:	Discussion, Decision

# 1	Introduction

In the WID on NR sidelink enhancement [1], RAN2 is tasked to specify mechanisms to support Sidelink DRX:

Sidelink DRX for broadcast, groupcast, and unicast [RAN2] - Define on- and off-durations in sidelink and specify the corresponding UE procedure - Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other - Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE.

In RAN2#112-e, the following agreements regarding high level aspects of SL DRX were made:

Agreements on SL DRX:

1: 	Sidelink DRX needs to support sidelink communications for both in and out of network’s coverage scenarios.

2:	RAN2 will prioritize normal use case without consideration of relay UE use case in Rel-17.

3:	Support SL DRX for all casting types.

4:	If a UE is in SL active time, UE should monitor PSCCH. FFS on PSSCH. FFS for sensing impacts.

5:	RAN2 is not going to introduce SL paging and SL PO for SL DRX.

6:	As baseline, for Sidelink DRX for SL unicast, it is proposed to inherit and use timers similar to what are used in Uu DRX. FFS for SL broadcast/groupcast. FFS on detailed timers.

7:	Working assumption: SL DRX should take PSCCH monitoring also for sensing (in addition to data reception) into account if SL DRX is used.

8:	Support of long DRX cycle for SL unicast should be assumed as a baseline. FFS on the need of short DRX cycle.

9:	Deprioritize SL WUS from RAN2 point of view in Rel-17.

In this contribution, we present our views on the above FFS and on other key aspects of the SL DRX.

# 2	Discussions

## 2.1	Design principles

There are some design principles that we believe RAN2 should agree upon before specifying the details of SL DRX, as follows:

- First, Uu DRX configurations should have higher priority than SL DRX configurations. That means, once a Uu DRX configuration is in place, a SL DRX configuration should adapt to Uu DRX configuration. In addition, a SL DRX configuration should not lead to modifications or override of the Uu DRX configuration. This is not to cause unwanted problems to the existing framework of Uu DRX.

- Second, it is important to keep all UEs in equal footing regarding SL DRX. This comes in different aspects. In the first aspect, a UE should not be allowed to enable or disable the DRX feature of another UE. Furthermore, a UE should not be allowed to terminate an ongoing DRX cycle or switch between DRX cycles of another UE. In the second aspect, a UE is not allowed to unilaterally change (a parameter of) a DRX configuration of another UE. Equivalently, a UE does not have to follow a DRX configuration selected by another UE. For example, to achieve the DRX alignment between UEs, it is reasonable to allow one UE to send some side information to assist another UE in determining a DRX configuration, but the latter UE does not have to accept all that is indicated by the former UE.

RAN2 adopts the following design principles for SL DRX:

- UE selects SL DRX within a set of DRX (pre)configurations.

- No UE can enable/disable the DRX feature nor terminate an ongoing DRX cycle in another UE.

- A UE is not allowed to unilaterally change a DRX configuration of another UE.

Another important aspect of DRX design is which node in the network can generate a SL DRX configuration. Obviously, a gNB can generate SL DRX configurations and configure them to the in-coverage UEs, just like Uu DRX configurations. An interesting question is whether a UE is allowed to generate DRX configurations on its own or to change some parameters of an existing configuration to make a new configuration. We see the benefit of allowing such flexibility (e.g., in DRX alignment between UEs) but we also foresee the complexity of supporting it (e.g., how to ensure that DRX configurations generated by different UEs do not overlap too much and cause high interference.). In general, we believe that UE-generated DRX configurations should not be allowed (i.e., the DRX configurations are (pre-)configured). This includes not allowing a UE to modify an existing SL DRX configuration to create a new one.

UEs are not allowed to generate DRX configurations themselves.

For UE in coverage, SL DRX configurations are configured by the gNB.

For UE out of coverage, SL DRX configurations are pre-configured.

## 2.2	SL DRX parameters

When it comes to DRX cycles, as agreed in RAN2#112-e, support of long DRX cycle for SL unicast should be assumed as a baseline. The benefits of short DRX cycles, however, is not clear for the SL. The reason is, on one hand short DRX cycle provide additional flexibility to balance between communication performance and power-saving. On the other hand, due to the distributed nature of SL, allowing both short and long DRX cycles will likely complicate the DRX alignment procedures between UEs. Therefore, we are sceptical about the overall benefits of supporting short DRX cycles and in our views, short DRX cycles should be down-prioritized in Rel-17.  This is also aligned with our view on the DRX design principles above.

The overall benefits of short DRX cycles are not clear in the SL.

Down-prioritize short DRX cycle for SL in Rel-17.

In RAN2#112-e, there is an FFS on whether UE should monitor PSSCH in addition to PSCCH during DRX active time. In our views, 2nd stage SCI on PSSCH carries the below information.

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	Cast type indicator – 2 bits as defined in Table 8.4.1.1-1.

Without this information, it is impossible to do meaningful monitoring and reception during Active Time.

Hence in SL active time, UE should monitor PSCCH and 2nd stage SCI in PSSCH.

In SL active time, UE monitors both PSCCH and 2nd stage SCI in PSSCH.

As agreed by RAN2#112-e, Sidelink DRX for SL unicast is to inherit and use timers similar to what are used in Uu DRX.  It is FFS for SL broadcast/groupcast and FFS on detailed timers.

Some companies have proposed to rely on specific resource pool configuration to implement DRX for broadcast and groupcast. The benefit is to avoid spec change. However, the UE would have to maintain multiple specific resource configurations in case there are multiple resource pools configured for broadcast and groupcast. This would sacrifice configuration flexibility with increased management complexity on resource pools. In addition, a resource pool may be shared among cast types (as in Rel-16).  In that case, it will be infeasible to enable DRX for broadcast or groupcast based a specific resource pool configuration in the resource pool. Therefore, RAN2 should aim for a unified DRX solution for SL DRX regardless of cast types. In other words, the timer based approach as in Uu DRX should be also adopted for broadcast and groupcast.

It is beneficial to adopt a unified DRX solution for SL DRX regardless of cast types.

Adopt the timer-based approach as in Uu DRX for SL broadcast/groupcast.

For SL DRX, at least three different categories of parameters are expected to be defined. i.e.,

- DRX parameters which are similar as Uu DRX.

- DRX parameters related to Mode 1 resource allocation

- DRX parameters related to SL characteristics which are different from Uu

Design of SL DRX need to comprise the three different categories

  - Category 1: parameters which are similar as Uu DRX

  - Category 2: parameters related to Mode 1 resource allocation

  - Category 3: parameters related to SL specific characteristics

For Category 1 parameters, basic parameters in Uu DRX including

- drx-onDurationTimerSL: the duration at the beginning of a SL DRX Cycle;

- drx-SlotOffsetSL: the delay before starting the drx-onDurationTimerSL;

- drx-InactivityTimerSL: the duration after the PSCCH/PSSCH occasion in which a PSCCH/PSSCH indicates a new SL transmission for the MAC entity;

- drx-LongCycleStartOffsetSL: the Long DRX cycle and drx-StartOffsetSL which defines the subframe where the Long and Short SL DRX Cycle starts;

can be directly reused for SL DRX.  In addition, since HARQ feedback and retransmission are introduced for SL unicast, the drx-RetransmissionTimer and drx-HARQ-RTT-Timer as in Uu DRX should be also reused for SL DRX. Similar as in Uu DRX, these two timers are not applicable to broadcast.

- drx-RetransmissionTimerSL (per SL HARQ process except for the broadcast process): the maximum duration until a SL retransmission is received;

- drx-HARQ-RTT-TimerSL (per SL HARQ process except for the broadcast process): the minimum duration before a SL assignment for HARQ retransmission is expected by the MAC entity;

Therefore, we propose RAN2 to agree on the below proposal.

As in Uu DRX configuration, adopt the basic parameters for SL DRX configuration including onDurationTimer, SlotOffset, InactivityTimer, LongCycleStartOffset, RetransmissionTimer (except for broadcast) and HARQ-RTT-Timer (except for broadcast).

For Category 2 parameters, in SL resource allocation Mode 1, gNB can assign SL grants to UE using DCI signalling addressed to SL RNTIs. This means that the UE needs to monitor PDCCH carrying SL grants in addition to PDCCH carrying UL grants or DL assignments. This would affect Uu DRX. As one solution, new timers related to Mode 1 SL grant monitoring can be introduced to Uu DRX configuration.

- drx-RetransmissionTimerSLmode1 (per SL HARQ process): the maximum duration until a grant for SL retransmission is received;

- drx-HARQ-RTT-TimerSLmode1 (per SL HARQ process): the minimum duration before a SL HARQ retransmission grant is expected by the MAC entity.

In order to support SL resource allocation Mode 1, adopt the parameters including RetransmissionTimerSLmode1 and HARQ-RTT-TimerSLmode1 in Uu DRX configuration.

For Category 3 parameters, the Sidelink Channel State Information (SL-CSI) reporting procedure is used to provide a peer UE with sidelink channel state information. For each triggered CSI report event, the CSI triggering UE can configure a latency requirement to the CSI reporting UE. The CSI reporting UE needs to report the CSI using a MAC CE within the latency requirements. It is beneficial to consider this aspect when designing SL DRX.  Therefore, we suggest adding the below parameters in SL DRX configuration.

- drx-CSIReportTimerSL: the maximum duration until a SL CSI report is received.

- drx-CSIReportRTTTimerSL: the minimum duration before a SL transmission grant on PSCCH for CSI report is expected by the MAC entity.

Adopt the two parameters including CSIReportTimer and CSIReportRTTTimer for SL DRX configuration. The corresponding start/stop procedure is FFS.

As an example, the related procedure may be defined as the below

- if a MAC PDU is transmitted in a sidelink grant and triggers CSI report from the peer UE:

  - start the drx-CSIReportRTTTimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PSSCH transmission;

    - stop drx-CSIReportTimerSL

- if drx-CSIReportRTTTimerSL expires:

  - start the drx-CSIReportTimerSL for the corresponding HARQ process in the first symbol after the expiry of drx-CSIReportRTTTimerSL.

- If a HARQ feedback is received for a PSSCH transmission:

  - stop drx-CSIReportTimerSL for the corresponding HARQ process

After first agreeing on the basic parameters for SL DRX, RAN2 can further discuss the detailed DRX procedures.

RAN2 further discusses the procedures after first agreeing on the basic parameters for SL DRX.

# 2.3	Multiple SL DRX configurations

In the previous sections we discussed the support of DRX for unicast/groupcast/broadcast and possible mechanisms to achieve Active time alignment. Now the key question is how the DRX configurations should be configured to best support the Active time alignment for all casting types. As discussed in Section 2, we are not in favor of a UE creating its own DRX configurations. Rather, the DRX configurations should be configured to the in-coverage UEs from the gNB and to out-of-coverage UEs via pre-configuration. The next questions would be how many DRX configurations a UE should support and how the UEs use the DRX configurations for unicast/groupcast/broadcast.

As for the first question, in our views, the maximum number of supported SL DRX configurations in a UE can be up to UE’s capability, but at least two types of SL DRX configurations should be supported. More specifically, we envisage that firstly all UEs in a resource pool should support a DRX configuration that is the same for everyone (so-called common DRX). Additionally, one or more DRX configurations which are configured for specific classes of traffic or UEs should be supported (so-called class-specific DRXs). Details for associating a specific DRX configuration with a class of traffic are up to discussion, for example, using some UE identifications or casting type.

It is beneficial to have a common DRX for all UEs in a resource pool, which can be used to bootstrap the DRX alignment between a pair of or a group of UEs.

Support two types of SL DRX configurations: common DRX and class specific DRX.

Given these two types of DRX configurations, the common DRX can act as a background configuration so that a pair of UEs (or a set of UEs) can utilize to conduct their DRX alignment process, and once the process is completed, switch to a relevant specific DRX. When the communication between the said UEs finishes, they can fall back to using the common DRX before switching to yet a new specific DRX if necessary. That means a UE has a single active DRX at a time (either the common DRX or a specific DRX). Another possibility is that a UE can be active simultaneously in both the common DRX and a specific DRX if the energy condition allows. Additionally, the common DRX can naturally be used for broadcast communication. In contrast, a class-specific DRX configuration can be triggered by a transmission or the reception of a packet belonging to the class or by a UE belong to the class, depending on which represents a class. By supporting these two types of DRX configurations, we have enough flexibility to support DRX alignment for all casting types and traffic types, both in-coverage and out-of-coverage conditions.

A common SL DRX configuration is common to all UEs in a resource pool regardless of cast types and service types (or QoS classes).

A class specific DRX configuration is configured to a group of UEs. There may be many different options.

In Option 1, a specific DRX configuration could be configured per unicast link (including source ID, and destination ID).

In Option 2, a specific DRX configuration could be configured per cast type (unicast or groupcast or broadcast)

In Option 3, a specific DRX configuration could be configured per service type (e.g., PSID/ITS-AID) or QoS class (e.g., PQI).

In order to give sufficient flexibility, in our views, all these three options need to be supported.

A class specific DRX configuration may be configured per unicast link, cast type or service type.

In case the UE is configured with multiple SL DRX configurations, there may be two possibilities. In the first possibility, a UE has a single active DRX at a time (either the common DRX or a specific DRX). Another possibility is that a UE can be active simultaneously in both the common DRX and a specific DRX if the energy condition allows. This can be further discussed by RAN2.

RAN2 further discusses whether the UE applies single active DRX configuration at a time or multiple active DRX configurations simultaneously.

# 5	Conclusion

In the previous sections we made the following observations:

Observation 1	The overall benefits of short DRX cycles are not clear in the SL.

Observation 2	It is beneficial to adopt a unified DRX solution for SL DRX regardless of cast types.

Observation 3	Design of SL DRX need to comprise the three different categories

	Category 1: parameters which are similar to Uu DRX

	Category 2: parameters related to Mode 1 resource allocation

	Category 3: parameters related to SL specific characteristics

Observation 4	It is beneficial to have a common DRX for all UEs in a resource pool, which can be used to bootstrap the DRX alignment between a pair of or a group of UEs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 adopts the following design principles for SL DRX:

	UE selects SL DRX within a set of DRX (pre)configurations.

	No UE can enable/disable the DRX feature nor terminate an ongoing DRX cycle in another UE.

	A UE is not allowed to unilaterally change a DRX configuration of another UE.

Proposal 2	UEs are not allowed to generate DRX configurations themselves.

Proposal 3	 For UE in coverage, SL DRX configurations are configured by the gNB.

Proposal 4	For UE out of coverage, SL DRX configurations are pre-configured.

Proposal 5	Down-prioritize short DRX cycle for SL in Rel-17.

Proposal 6	In SL active time, UE monitors both PSCCH and 2nd stage SCI in PSSCH.

Proposal 7	Adopt the timer-based approach as in Uu DRX for SL broadcast/groupcast.

Proposal 8	As in Uu DRX configuration, adopt the basic parameters for SL DRX configuration including onDurationTimer, SlotOffset, InactivityTimer, LongCycleStartOffset, RetransmissionTimer (except for broadcast) and HARQ-RTT-Timer (except for broadcast).

Proposal 9	In order to support SL resource allocation Mode 1, adopt the parameters including RetransmissionTimerSLmode1 and HARQ-RTT-TimerSLmode1 in Uu DRX configuration.

Proposal 10	Adopt the two parameters including CSIReportTimer and CSIReportRTTTimer for SL DRX configuration. The corresponding start/stop procedure is FFS.

Proposal 11	RAN2 further discusses the procedures after first agreeing on the basic parameters for SL DRX.

Proposal 12	Support two types of SL DRX configurations: common DRX and class specific DRX.

Proposal 13	A common SL DRX configuration is common to all UEs in a resource pool regardless of cast types and service types (or QoS classes).

Proposal 14	A class specific DRX configuration may be configured per unicast link, cast type or service type.

Proposal 15	RAN2 further discusses whether the UE applies single active DRX configuration at a time or multiple active DRX configurations simultaneously.

# References

RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics, 3GPP TSG RAN Meeting #90e, Electronic Meeting, December 7 - 11, 2020