3GPP TSG-RAN WG2 Meeting #113-e	R2-2100539

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.15.2.2

Source:	Ericsson

Title:	SL DRX alignment between two UEs

Document for:	Discussion, Decision

# 1	Introduction

In the WID on NR sidelink enhancement [1], RAN2 is tasked to specify mechanisms to support Sidelink DRX:

Sidelink DRX for broadcast, groupcast, and unicast [RAN2] - Define on- and off-durations in sidelink and specify the corresponding UE procedure - Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other - Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE.

Wherein, the second objective, i.e., alignment of SL DRX among UEs is one important aspect.

In RAN2#112-e, companies have started to discuss the two options, i.e., TX UE centric or RX UE centric. However, no agreements have been made yet. In this contribution, we further present our views.

# 2	Alignment between UEs

For a pair of UEs in a same communication, DRX alignment between two UEs makes sure that the two UEs are in Active state in the same time duration and communicate to each other. This alleviates the need for a UE to turn on its circuits outside its DRX Active time to be able to send messages to the other UE. Additionally, the communication between two UEs is bidirectional in many cases, making a perfect sense for DRX alignment. In case of groupcast and broadcast, aligning active time also means all intended receivers of a message to be active at the same time to receive the message sent from a transmitter UE.

At RAN2#112-e, for unicast, companies have discussed the two options, i.e., TX UE centric or either RX UE centric to some extent.

Before continuing discussions on the options, we would like to first clarify our views on the term “TX UE” and the term “RX UE”.

For a PC5 unicast link comprising two UEs, the term “TX UE” stands for the UE which initiates a PC5 link/SLRB establishment towards the other UE, while the other UE which receives the PC5 link/SLRB establishment is referred to as “RX UE”. Based on such definition, role of “TX UE” or “RX UE” will not change for a PC5 link/SL RB regardless which UE initiates a transmission on the link/SL RB.

For unicast, the term “TX UE” stands for the UE which initiates a PC5 link/SLRB establishment towards the other UE, while the other UE which receives the PC5 link/SLRB establishment is referred to as “RX UE”.

For TX UE centric option, a TX UE (or its serving gNB in case the TX UE is in coverage) sends a desired SL DRX configuration to a RX UE. The RX UE follows the received SL DRX configuration.

For RX UE centric option, a RX UE informs a TX UE of its DRX configuration. The TX UE follows the received DRX configuration from the RX UE and transmits data to the RX UE only when the RX UE is in active.

TX UE centric option has the benefit that it is aligned with Rel-16 signalling framework. However, it has the drawback that the RX UE may have to always keep in active time by following instructions from all TX UEs, and therefore have no power saving especially when the RX UE is connected to multiple TX UEs.

Meanwhile RX UE centric option can achieve better power saving for the RX UE. However, RX UE centric option would require bigger spec changes than TX UE centric option. In addition, longer delay may be caused for data transfer at the TX UE, since the TX UE may not be able to transmit immediately to the RX UE when there is data available.

In our views, TX UE centric option shall be adopted considering that spec changes need to be limited for Rel-17 due to limited work time.

For unicast, TX UE centric option shall be adopted as the baseline for SL DRX alignment between UEs.

In addition, it is beneficial to allow TX UE and RX UE to negotiate between each other in the procedure. In our views, both TX UE and RX are equally positioned in the procedure. A RX UE is allowed to accept or reject a recommended DRX configuration by a TX UE.  In this way, the drawback of TX UE centric option, i.e., RX UE may lose its power saving by compulsorily following instructions from TX UEs can be mitigated.

For unicast, introduce a negotiation mechanism on top of TX UE centric option.

To simplify potential design efforts, the negotiation may be a two-step process. In this way, RX UE can only simplify indicate the decision (i.e., acceptance or rejection) after reception of a suggested SL DRX configuration. It is also possible to introduce a more complex process such as a three-step or four-step process. In this way, RX UE can indicate more information/choices in the response message.  The UE pair is therefore able to perform further negotiation and make compromise. RAN2 can further study if a more complex negotiation procedure needs to be designed.

For unicast, a two-step negotiation process (i.e., RX UE simply accepts or rejects TX UE’s suggestion) should be adopted as the baseline.

For unicast, it is FFS on the necessity of a more complex negotiation procedure such as a three-step or four-step process (i.e., RX UE can indicate more information/choices in the response message for further negotiation).

During a negotiation process, the process stops if the UE pair has made the agreement between each other, the UE pair applies the agreed DRX configuration. The process may terminate e.g., due to limitation of the number of steps. In this case the UE pair cannot make an agreement on a DRX configuration, both UEs can fallback to a default/common DRX configuration instead.

In addition, for a PC5 link or a SL RB, the transmissions/receptions on the link/RB will be bidirectional. A DRX configuration for the link or the SL RB will be determined based on the type of the service carried on the link/RB. It is natural to apply the DRX configuration for the link/RB regardless of transmission direction.

For unicast, the UE pair (i.e., a TX UE and a RX UE) apply a DRX configuration when the UE pair has agreed on the DRX configuration regardless of transmission direction.

For unicast, the UE pair (i.e., a TX UE and a RX UE) falls back to a default/common DRX configuration if the UE pair cannot agree on a DRX configuration recommended by the TX UE.

For groupcast and broadcast, it is feasible to achieve alignment among UEs via a specific DRX configuration. The DRX configuration can be associated to the service type (e.g., PSID/ITS-AID) or QoS class (e.g., PQI).

For broadcast or groupcast, SL DRX is aligned among UEs via a service specific DRX configuration (e.g., associated to the service type (e.g., PSID/ITS-AID) or QoS class (e.g., PQI)).

# 5	Conclusion

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For unicast, the term “TX UE” stands for the UE which initiates a PC5 link/SLRB establishment towards the other UE, while the other UE which receives the PC5 link/SLRB establishment is referred to as “RX UE”.

Proposal 2	For unicast, TX UE centric option shall be adopted as the baseline for SL DRX alignment between UEs.

Proposal 3	For unicast, introduce a negotiation mechanism on top of TX UE centric option.

Proposal 4	For unicast, a two-step negotiation process (i.e., RX UE simply accepts or rejects TX UE’s suggestion) should be adopted as the baseline.

Proposal 5	For unicast, it is FFS on the necessity of a more complex negotiation procedure such as a three-step or four-step process (i.e., RX UE can indicate more information/choices in the response message for further negotiation).

Proposal 6	For unicast, the UE pair (i.e., a TX UE and a RX UE) apply a DRX configuration when the UE pair has agreed on the DRX configuration regardless of transmission direction.

Proposal 7	For unicast, the UE pair (i.e., a TX UE and a RX UE) falls back to a default/common DRX configuration if the UE pair cannot agree on a DRX configuration recommended by the TX UE.

Proposal 8	For broadcast or groupcast, SL DRX is aligned among UEs via a service specific DRX configuration (e.g., associated to the service type (e.g., PSID/ITS-AID) or QoS class (e.g., PQI)).

# References

RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics, 3GPP TSG RAN Meeting #90e, Electronic Meeting, December 7 - 11, 2020