3GPP TSG RAN WG2 Meeting #113-e	R2- 2100550

Online, January 25 – February 5, 2021

Agenda Item:	8.7.4

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Open Issues on NR Sidelink Relaying

Document for:	Discussion and Decision

# Introduction

A new study item for Rel-17 on NR sidelink relay has been approved at RAN#86 [1]. The objectives of the SI are as follows:

This study item targets to study single-hop NR sidelink-based relay.

- Study mechanism(s) with minimum specification impact to support the SA requirements for sidelink-based UE-to-network and UE-to-UE relay, focusing on the following aspects (if applicable)  for layer-3 relay and layer-2 relay [RAN2];

  - Relay (re-)selection criterion and procedure;

  - Relay/Remote UE authorization;

  - QoS for relaying functionality;

  - Service continuity;

  - Security of relayed connection after SA3 has provided its conclusions;

  - Impact on user plane protocol stack and control plane procedure, e.g., connection management of relayed connection;

- Study mechanism(s) to support upper layer operations of discovery model/procedure for sidelink relaying, assuming no new physical layer channel / signal [RAN2];

NOTE 1: The study shall take into account of further input from SA WGs, e.g., SA2 and SA3, for the bullets above (if applicable).

NOTE 2: It is assumed that UE-to-network relay and UE-to-UE relay use the same relaying solution.

NOTE 3: Forward compatibility for multi-hop relay support in a future release needs to be taken into account.

NOTE 4: For layer-2 UE-to-network relay, the architecture of end-to-end PDCP and hop-by-hop RLC, e.g., as recommended in TR 36.746, is taken as starting point.

In this document, open issues with respect to relay (re-)selection criterion and procedure, service continuity and resources for relay UEs will be discussed.

# Relay (Re-)Selection Criteria

As discussed in the email discussion [2], several criteria can be applied to trigger relay (re-)selection. Generally, the criteria can be classified into two main categories, namely Access Stratum (AS) criteria and higher layer criteria [2]. Both types might be applied in both L2 and L3 architectures [3,4]. One of the criteria is the RSRP of a candidate relay UE. The following subsections discuss consideration of RSRP as a relay (re-)selection.

## RSRP of Discovery Messages

Based on the RAN2-112-e agreement, the RSRP of discovery messages can be considered as one of the criteria for relay (re-)selection. However, the RSRP of discovery messages reflect the distance of a remote UE and a relay UE, but it does not guarantee the required QoS requirements of traffic in a remote UE. In case the discovery procedure uses dedicated resources (discovery resource pool) and not the common resource pool for transmission, a high RSRP of the discovery messages cannot be interpreted as high RSRP in the common resource pool for transmission. Although, it was agreed that a dedicated or common resource pool for discovery will be discussed in the WI phase, the agreement about considering the RSRP of the discovery messages as a relay (re-)selection criteria should be re-evaluated.

If the discovery procedure uses a dedicated resource pool (or a dedicated set of resources) and not the common resource pool for transmission, considering the RSRP of discovery messages might  not be appropriate for relay (re-)selection.

RAN2 to further study the cases where discovery procedure considers resources of the common and dedicated resource pools, when the RSRP of discovery messages is applied as a criteria for relay (re-)selection.

## RSRP of PC5 Unicast Link

Based on the RAN2-112-e agreement, the RSRP of an existing PC5 unicast link can be considered as a criteria for relay selection. The RSRP of messages in a PC5 unicast link is beneficial for the relay selection, if there is an on-going message exchange between the remote and relay UEs. In case where UEs do infrequently exchange any message over the PC5 link, the RSRP might be outdated.

The RSRP of PC5 unicast link might not be sufficient for relay (re-)selection, if the relay and remote UE are not continuously exchanging messages over the link.

RAN2 to study the case where remote and relay UEs are not continuously exchanging message and the RSRP of PC5 link is one of the criteria of relay (re-)selection.

# Service Continuity Considerations in Relay Re-selection

## Radio Link Failure case

One of the reasons to trigger the relay re-selection is a Radio Link Failure (RLF). In one case, the remote UE might recognize the RLF with a delay. In another case, a RLF might occur in the relay UE (e.g. between a relay UE and a destination UE in the UE-to-UE relay scenario) and the remote UE or gNB should be informed to start the relay re-selection procedure. In both cases, the service continuity might not be provided due to the possible delays in the relay re-selection.

In case of RLF, the service continuity might not be fulfilled due to the possible delays to trigger the relay re-selection procedure.

RAN2 to study the possible criteria to trigger relay re-selection in time to ensure service continuity.

## Candidate Relay Set in Remote UEs

The (re-)selection procedure might be time consuming to find an appropriate relay UE meeting all AS and higher layer criteria, whenever re-selection criteria are met. All UEs that meet  relay selection criteria can be listed by the gNB. This list can be sent to a remote UE along with other information, such as resource pool, thresholds for (re)-selection etc. The final relay (re-)selection can be left up to UE implementation. The candidate list assists a remote UE in a faster relay (re-)selection procedure. However, the list might be out-dated due to the dynamics of the networks and needs to be updated by gNB or a remote UE.

The relay re-selection procedure might lead to higher delay due to the involvement of the discovery procedure, performing measurements on PC5 and Uu interfaces and checking for the higher layer criteria.

A list of candidate relay UEs stored in a remote UE might accelerate the relay re-selection procedure.

The list of relay UEs, which meet the relay (re-)selection criteria, might be out-dated due to the dynamics of the network.

# Resources for Relay UEs

The following two types of resource pools might be studied to be considered by a relay UE for transmission of messages between two remote UEs or between a remote UE and gNB.

- Dedicated resource pool: as relaying imposes extra load on a relay UE, additional resources might be needed if the relayed data flow requires high reliability or low latency. A dedicated resource pool can be configured by gNB or pre-configured in a relay UE to be employed in the relaying of messages. The separation of resources for relay functionality can improve the reliability and latency.

- Common resource pool: a relay UE may use the common resource pool for relaying purpose. However, if the load of the relay node and the congestion of common resource pool are not considered in the relay (re-)selection procedure, the relay node might be unable to fulfil the QoS requirements of data flows.

By using the common resource pool for relaying, the relay UE might not be able to fulfil the QoS requirements.

Using a dedicated resource pool for relaying, the relay UE can differentiate messages for relaying from other messages.

RAN2 to study the feasibility of a dedicated resource pool for relaying.

# Conclusions

The following observations and proposals have been made in this document:

Observation 1:	If the discovery procedure uses a dedicated resource pool (or a dedicated set of resources) and not the common resource pool for transmission, considering the RSRP of discovery messages might  not be appropriate for relay (re-)selection.

Observation 2:	The RSRP of PC5 unicast link might not be sufficient for relay (re-)selection, if the relay and remote UE are not continuously exchanging messages over the link.

Observation 3:	In case of RLF, the service continuity might not be fulfilled due to the possible delays to trigger the relay re-selection procedure.

Observation 4:	The relay re-selection procedure might lead to higher delay due to the involvement of the discovery procedure, performing measurements on PC5 and Uu interfaces and checking for the higher layer criteria.

Observation 5:	The list of relay UEs, which meet the relay (re-)selection criteria, might be out-dated due to the dynamics of the network.

Observation 6:	By using the common resource pool for relaying, the relay UE might not be able to fulfil the QoS requirements.

Observation 7:	Using a dedicated resource pool for relaying, the relay UE can differentiate messages for relaying from other messages.

Proposal 1:	RAN2 to further study the cases where discovery procedure considers resources of the common and dedicated resource pools, when the RSRP of discovery messages is applied as a criteria for relay (re-)selection.

Proposal 2:	RAN2 to study the case where remote and relay UEs are not continuously exchanging message and the RSRP of PC5 link is one of the criteria of relay (re-)selection.

Proposal 3:	RAN2 to study the possible criteria to trigger relay re-selection in time to ensure service continuity.

Proposal 4:	A list of candidate relay UEs stored in a remote UE might accelerate the relay re-selection procedure.

Proposal 5:	RAN2 to study the feasibility of a dedicated resource pool for relaying.

# References

[1] RP-193253, “New SID on NR sidelink relay”, LG Electronics, 3GPP RAN#86, Dec 2019.

[2] [Post111-e][622][Relay] “Relay selection and reselection”.

[3] TS 23.303, “Technical Specification Group Services and System Aspects; Proximity-based services (ProSe); Stage 2, (Release 16)”, V16.0.0, (2020-07).

[4] TR 36.746, “Study on further enhancements to LTE Device to Device (D2D), User Equipment (UE) to network relays for Internet of Things (IoT) and wearables”, V15.1.1, (2018-04).