3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2100611

Elbonia, 25 January – 5 February 2021	R2-2010671

Agenda item:	8.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Re-routing enhancements in IAB

WID/SID:	NR_IAB_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses re-routing aspects for topology-adaptation enhancements.

# 2	Discussion

## 2.1	IAB node as dead end for downstream traffic

An IAB node can become a dead end for downstream traffic toward a given destination node if all downlink hops toward the destination are no longer available. In such a case, Rel-16 IAB does not allow re-routing of such data from the dead-end node, or re-transmitting and re-routing from an ancestor node of the dead-end node.

Observation 1:	Rel-16 IAB does not allow re-routing of downstream data having reached an IAB node with all downlink hops toward a given destination unavailable.

There are different options for guiding such traffic to its destination.

Option 1:	BAP-routing paths towards a descendant node with a parent node as next hop. As discussed in [3], this is a generalization of routing paths with both UL and DL hops, from its previously proposed version where upstream data is routed via dual-connected descendant nodes. In this case, such routing paths would remain as re-routing options at an IAB node that has run out of DL-only routes toward the destination.

Option 2:	uplink indication that certain BAP destination(s) have become unreachable. Reception of such an indication allows re-routing by the parent node, if it has alternative routes available, or else forwarding the indication further to grandparent node(s). This option has two possible variants:

2A:	indication not accompanying data. This could be a BAP control PDU listing BAP destinations that are unreachable from the node sending this indication. It would allow re-routed retransmission by the parent node, of data that the parent node has buffered possibly despite RLC ACKs received, as proposed in the next section. It minimizes the number of hops that the data needs to travel to the destination.

2B:	undeliverable-indication in the header of a BAP PDU returned to parent node. This variant, like option 1, involves the data being sent back and forth (from and to the parent node), but allows buffering of data at only one node at a time.

Proposal 1: 	RAN2 discuss re-routing of downstream data having reached an IAB node with all downlink hops toward a given destination unavailable, and select among options: 
1) BAP-routing paths with a parent node as next hop;
2A) uplink indication (not accompanying data) that certain destinations are unreachable;
2B) undeliverable-indication in the header of a BAP PDU returned to parent node.

## 2.2	Local rerouting enhancements in case of BH RLF

In Rel-16, when a BH RLF occurs, rerouting can be done locally at the IAB node if there is an alternative route to the same destination node. As illustrated in Fig. 1, in case Path#1 is not available due to the BH RLF between IAB#1 and IAB#2 (see Scenario a), IAB#1 may select Path#2 via IAB#4 for the BH data destined to Donor DU. However, if IAB#2 cannot deliver a packet to IAB#3 due to the BH RLF between IAB#2 and IAB#3 (see Scenario b), local rerouting is not feasible at IAB#2 as there is no alternative route to the same destination node.

Observation 2:	Local rerouting can be done locally at the IAB node if there is an alternative route to the same destination node.

Figure 1. Multi-hop IAB deployment with two paths to the destination (Upstream traffic)

Rel-16 also introduces the BH RLF indication for the failure handling when an IAB node fails to recover from a BH RLF. For instance, in Fig. 1, in case Path#1 is not available due to the BH RLF recovery failure between IAB#2 and IAB#3 (see Scenario b), BH RLF indication may be sent to IAB#1. If the rerouting is possible at IAB#1, IAB#1 may select Path#2 via IAB#4 for the BH data, which is destined to Donor DU, for the future routing. On the other hand, BAP PDUs which were successfully sent to IAB#2 but not to IAB#3 (due to a BH RLF in Path#1) cannot be rerouted by IAB#1 to IAB#4. That is because BAP entity may only reroute the BAP Data PDUs, which were not acknowledged by the lower layer, to an alternative path [1]. However, to enable improved local rerouting by a child IAB node in case of BH RLF at a parent node, the BAP PDUs can be stored by the BAP entity until the expiry of a BAP discard timer despite the received RLC ACKs. This way, the data packets can be rerouted by a child IAB node via an alternative path if a BH RLF indication is received while the PDUs are still not discarded. In this case, to avoid congestion and fulfil the QoS requirements, the BAP discard timer needs to be specified, which determines the maximum time interval that a PDU can be stored by the BAP entity. The BAP discard timer was also proposed at 3GPP RAN2#111e meeting [2].

Observation 3:	In case of BH RLF, BH RLF indication may be sent to the child nodes. Rerouting may be possible at a child IAB node if an alternative path exists when the BH RLF indication is received.

Observation 4:	Since the BAP entity may only reroute the BAP Data PDUs, which were not acknowledged by the lower layer, to an alternative path, it is not possible at the child IAB node to locally reroute the BAP PDUs which were successfully sent to the parent IAB node but not to the ancestor in case of a BH failure between the parent and ancestor nodes.

Proposal 2:	BAP PDUs are not discarded by the BAP entity until the expiry of a BAP discard timer despite the received RLC ACKs. In case of a received BH RLF indication, the buffered PDUs are rerouted by the child IAB node via an alternative path.

# 3	Conclusion

In this contribution we made the following observations and proposals:

Observation 1:	Rel-16 IAB does not allow re-routing of downstream data having reached an IAB node with all downlink hops toward a given destination unavailable.

Observation 2:	Local rerouting can be done locally at the IAB node if there is an alternative route to the same destination node.

Observation 3:	In case of BH RLF, BH RLF indication may be sent to the child nodes. Rerouting may be possible at a child IAB node if an alternative path exists when the BH RLF indication is received.

Observation 4:	Since the BAP entity may only reroute the BAP Data PDUs, which were not acknowledged by the lower layer, to an alternative path, it is not possible at the child IAB node to locally reroute the BAP PDUs which were successfully sent to the parent IAB node but not to the ancestor in case of a BH failure between the parent and ancestor nodes.

Proposal 1: 	RAN2 discuss re-routing of downstream data having reached an IAB node with all downlink hops toward a given destination unavailable, and select among options:

1) BAP-routing paths with a parent node as next hop;
2A) uplink indication (not accompanying data) that certain destinations are unreachable;
2B) undeliverable-indication in the header of a BAP PDU returned to parent node.

Proposal 2:	BAP PDUs are not discarded by the BAP entity until the expiry of a BAP discard timer despite the received RLC ACKs. In case of a received BH RLF indication, the buffered PDUs are rerouted by the child IAB node via an alternative path.

# 4	References

[1]	TS38.340 3GPP TS RAN NR; Backhaul Adaptation Protocol (BAP) specification (Release 16), V16.2.0.

[2]	R2-2007312 ZTE, Sanechips, Miscellaneous enhancements for IAB network, 3GPP RAN2#111e.

[3] R2-2010671, Nokia, Nokia Shanghai Bell, On topology-adaptation enhancements