3GPP TSG-RAN WG2 Meeting #113- electronic				R2-2100620

Electronic meeting, 25th Jan – 5th Feb 2021

Agenda item:		7.4.2

Source:			Intel Corporation, Huawei, HiSilicon

Title:		Support of NUL and SUL during DAPS handover

Document for:	 	Discussion and decision

# Introduction

In [1], RAN1 mentioned they have discussed support of NUL and SUL during DAPS handover and agreed to the following:

- In Rel-16, UE is not required to support both NUL and SUL in source cell during DAPS handover

- In Rel-16, UE is not required to support both NUL and SUL in target cell during DAPS handover

- Up to RAN2 for the solution to handle above cases

In this contribution, we expressed our view on this issue.

# Discussion

In RAN2#112e, RAN2 discussed the support of multi-RTP, CA and DC during DAPS handover, and concluded that

Network ensures that multi-TRP does not operate simultaneously with DAPS HO. This will typically require network to do RRC reconfiguration before sending DAPS HO command.

Network ensures that SCG and/or SCells are not configured when UE receives DAPS HO. This will typically require network to do RRC reconfiguration before sending DAPS HO command.

Corresponding CRs [2]-[5] have been agreed.

Based on RAN1 LS in [1], there could be two approaches in RAN2:

Approach 1: The NUL+ SUL cannot work simultaneously with DAPS, i.e. the SUL configured in source shall be released if NUL+SUL is configured before the handover command is sent to the UE, and NUL+SUL configuration cannot be configured in DAPS HO command.

Approach 2: The NUL+SUL is optional to the DAPS UE, i.e. introduce the capability on the support of NUL+SUL for DAPS;

However, according to RAN1 discussion in [6], RAN1 intention is to remove the configuration instead of introducing a new UE capability on this. The reason “not required” is used in the LS because company commented “not required” was used in the agreement made in RANP for mTRP+DAPS as “UE is not required to support simultaneous operation of multi-DCI/single-DCI mTRP and DAPS in Rel-16.”.

Note: For details of the extensive RAN1 discussion that took place between Oct 26 to Nov 5, please see R1-2009542 [6].

Therefore we propose

Proposal: NUL+SUL does not operate simultaneously with DAPS HO. This will typically require network to do RRC reconfiguration, i.e. the network shall release SUL configuration if NUL+SUL is configured in source before the DAPS HO command is sent to the UE, and the network cannot configure the NUL+SUL in DAPS HO command.

# Conclusion

Based on the discussion, we have following proposals:

Proposal: NUL+SUL does not operate simultaneously with DAPS HO. This will typically require network to do RRC reconfiguration, i.e. the network shall release SUL configuration if NUL+SUL is configured in source before the DAPS HO command is sent to the UE, and the network cannot configure the NUL+SUL in DAPS HO command.

The corresponding CRs are provided in [7]-[8].

# References

[1] R1-2009682 LS on support of NUL and SUL during DAPS handover, RAN1

[2] R2-2010747	Clarification on no support of CA or DC with DAPS		ZTE Corporation, Sanechips, Ericsson

[3] R2-2010748	Clarification on no support of CA, DC or multi-TRP with DAPS	ZTE Corporation, Sanechips, Ericsson

[4] R2-2010750	Clarification on no support of CA, DC or multi-TRP with DAPS	ZTE Corporation, Sanechips, Ericsson

[5] R2-2010749	Clarification on no support of CA or DC with DAPS		ZTE Corporation, Sanechips, Ericsson

[6] R1-2009542 NR mob-enh email discussion	Moderator (Intel Corporation)

[7] R2-2100627	38.331 CR on support of NUL and SUL during DAPS handover	Intel Corporation, Huawei, HiSilicon

[8] R2-2100628	38.300 CR on support of NUL and SUL during DAPS handover	Intel Corporation, Huawei, HiSilicon