3GPP TSG-RAN WG2 Meeting #113 electronic                                             　    　　　　　　 R2-2100647

Online, January 25th – Feburary 5th 2021

Source: 	KDDI Corporation

Title:	Considerations on Time Alignment Timer for SCG deactivation

Tdoc Type:	Discussion

Agenda Item:	8.2.2 Fast Scell activation

Document for:	Discussion and decision

Release:	Rel-17

Introduction

In the last two RAN2 meetings, a lot of time was spent discussing whether the RACH for SCG reactivation is needed and whether to keep the TA while SCG is deactivated. A consensus hasn’t been reached [1]. Some companies consider that keeping monitoring reference signals, such as SRS, to maintain the TA with SCG is energy consuming, while other companies consider launching RACH more time consuming to reactive the SCG, lowering the user experience for cases in which latency is the main performance indicator. In this paper we would like to discuss our views on the Time Alignment Timer (TAT) with SCG reactivation.

Discussion

In the current specification, a TAT is started or restarted once the UE receives a timing alignment command from the network. While the TAT is valid, the UE assumes it is synchronized with the network and assumes it is able to access the network without the need of a RACH. This same definition could be applied to SCG reactivation. If the connection to a SCG is deactivated, a reactivation command due to the uplink transmission could be received very shortly while the TAT is still running, if the UE is assumed still time aligned to the SCG, then it can access to the SCG directly for packet forwarding.

In the current specification, the UE action after receiving SCG deactivation is to stop transmitting SRS/UL-SCH/RACH, to stop reporting CSI and to stop monitoring PDCCH/PUCCH. There is no statement on what action should be taken for the TAT, so it should be clarified in this Rel-17 WI. Although the effect of eliminating the RACH while the TAT is running might only apply for the cases in which the SCG is in a deactivated state for a short duration of time, it is certain that fast access to the SCG requires the elimination of the RACH. Maintaining the TAT also doesn’t impose a lot of impacts on RAN2 and its application is not complex. The action of maintaining the TAT until it is expired after the SCG is deactivated should be explicitly included in the specification.

Proposal1: RAN2 agrees to assume that after SCG is deactivated, UL TA is still accurate until TA timer expires.

Proposal2: RAN2 agrees to keep the TA timer running when SCG is deactivated.

Proposal3: RAN2 agrees to resume normal SCG operation (resume UL transmission) without RACH if the SCG is activated while the TAT is still running.

Conclusion

We introduce the following proposals.

Proposal 1: RAN2 agrees to assume that after SCG is deactivated, UL TA is still accurate until TA timer expires.

Proposal2: RAN2 agrees to keep the TA timer running when SCG is deactivated.

Proposal3: RAN2 agrees to resume normal SCG operation (resume UL transmission) without RACH if the SCG is activated while the TAT is still running.

References

[1] R2-2100001 Report of 3GPP TSG RAN2#112-e meeting, Online