3GPP TSG-RAN WG2 Meeting #113 electronic    	                              R2-2100696

# Online, Jan 25 th - Feb 5th, 2021                                                    Revision of R2-2009678

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2348 | rev | - | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TS 38.331 on logged MDT configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A pair of CRs (R2-2009677 and R2-2009678) were submitted to RAN2#112-e meeting, trying to discuss the configuration effectiveness issue for logged MDT in both EUTRA and NR. However, those CRs were not discussed, the summary in R2-2011010 (Summary on 6.10.3 RRC Corrections) expressed some concerns on the changes to TS 36.331, stating that the impacted text was introduced from Rel-10, so the changes should be carefully checked. But the change to TS 38.331 on the configuration effectiveness should be considered. The reason for the support of the change is given below. According to TS 37.320, the old configuration shall be replaced whenever the configuration for logged MDT is renewed, regardless of the RAT on which the configuration was configured. #### 5.1.1.1 Measurement configuration A release operation for logged measurement configuration in the UE is realized only by configuration replacement when the configuration is overwritten or by configuration clearance in case a duration timer stopping or expiration condition is met.. Besides, the description in clauses 5.5a.1.2/5.5a.1.3 shows that the configuration initiated by NG-RAN (LoggedMeasurementConfiguration) can trigger the release operation of the previous logged measurement configuration, which, is also initiated by NG-RAN. This further confirms that the logged measurement configuration can be released upon the reception of a new configuration within the same RAT. #### 5.5a.1.2 Initiation NG-RAN initiates the logged measurement configuration procedure to UE in RRC_CONNECTED by sending the LoggedMeasurementConfiguration message. #### 5.5a.1.3 Reception of the LoggedMeasurementConfiguration by the UE Upon receiving the LoggedMeasurementConfiguration message the UE shall: 1> discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2; <*text omitted*>. However, in the TS 38.331 section 5.5a.2.2, the specification describes the configuration replacement in the following way, which means the UE shall release the old logged measurement configuration only when the newly received configuration is in another RAT. #### 5.5a.2.2 Initiation The UE shall initiate the procedure upon receiving a logged measurement configuration in another RAT. The UE shall also initiate the procedure upon power off or detach. The UE shall: 1> stop timer T330, if running; 1> if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.. In our understanding, the current description is not accurate as it adds an extra limitation to the release operation of logged configuration. |
| Summary of change: |  | Remove the ‘in another RAT’ in the section 5.5a.2.2. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: logged measurement configuration Inter-operability: - If UE implements according to the CR and the network does not, there is no impact forseen. - If the network implements according to the CR and the UE does not, UE won’t release the old logged measurement configuration when the newly received configuration is in the same RAT, which in turn disables the network’s ability of updating the logged measurement configuration within the same RAT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will not release the old logged measurement configuration when the newly received configuration is in the same RAT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5a.2.2 Initiation |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF THE CHANGE

5.5a.2.2	Initiation

The UE shall initiate the procedure upon receiving a logged measurement configuration. The UE shall also initiate the procedure upon power off or detach.

The UE shall:

1>	stop timer T330, if running;

1>	if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

END OF THE CHANGE