3GPP TSG-RAN WG2 Meeting #113 electronic    	                              R2-2100698

# Online, Jan 25 th - Feb 5th, 2021

Agenda Item:	8.13.2.2

Source: 	vivo

Title:         	Discussion on contents and signalling model of 2-step RACH report

Document for: 	Discussion and Decision

- Introduction

RAN2#112-e meeting [1] discussed the contents of 2-step RA and made the following agreements:

Confirm that the information included in Rel-16 RA report which also applied to 2-step RA at least contains: - Cell ID of the cell in which the RA is performed - RA purpose - Frequency information of the BWP where RA is performed - Frequency information of RA resources - Number of preambles sent on an SSB - Beam index - Contention detection per RA attempt - Beam quality indication. FFS on the details. At least following RACH frequency related information should be included in RACH report for optimization of 2-step RACH: - msgA-FrequencyStart-r17 - msgA-FrequencyStartCFRA-r17 - msgA-SubcarrierSpacing-r17 - msgA-SubcarrierSpacingCFRA-r17 - msgA-FDM-r17 - msgA-FDMCFRA-r17

Now that we have the above baseline for 2-step RA report content, we would like to further discuss the additional information which is also useful for SON optimization, besides, we also consider the signalling model of the 2-step RACH report therein.

- Discussion

  - Contents for 2-step RACH report

Beam quality indication

It was agreed that the beam quality indication should be included in the RA report, but the details such as whether this should be indicated per attempt or per procedure, and should be an explicit Boolean indication or a measured DL RSRP are FFS.

The purpose of recording the parameter, which indicates whether the RSRP of the downlink pathloss reference exceeds msgA-RSRP-ThresholdSSB, is for SON purposes. With this indication, the network is able to perform parameterization on the preconfigured threshold based on the received RACH reports, to further improve the successful RACH rate. Assume this parameter is logged per RACH procedure, thus for each RACH report there is at most one indication. Therefore the recorded measurements of the other non-indicated entries turn out to be meaningless. Network expects to receive RACH reports carrying the failure events, such as the failed RACH attempt in case the RSRP of the downlink pathloss reference is above msgA-RSRP- ThresholdSSB, so that the network is able to recognize some underlying issues with the received reports and achieve further optimization. For the non-indicated entries, however, since the network fails to know if DL beam quality is above or below the msgA-RSRP-ThresholdSSB, the reported entries will be ultimately ignored by the network as such information is ineffective.

- For the non-indicated entries, as the network fails to know whether DL beam quality is above or below the msgA-RSRP-ThresholdSSB, the reported entries will be ultimately ignored by the network as such information is pointless.

As shown in the above analysis, our preference for the granularity of the parameter, which indicates whether the DL beam quality is above or below the msgA-RSRP-ThresholdSSB-r16, is per random access attempt. If this proposal is agreeable, RAN2 also needs to reply to the RAN3 LS [2] to confirm that the granularity of the beam quality indication should be per-RA-attempt.

The beam quality should be indicated per random access attempt.

LS RAN3 to confirm that the granularity of the beam quality indication should be per-RA-attempt.

Actually, both an explicit Boolean indication or a measured DL RSRP can be used to represent the indication, but with the mind that to keep the simplicity and consistency of ASN.1 code, we should re-use the legacy field as much as possible. As the indication is used to indicate whether the DL beam (SSB) quality associated with the random access attempt was above or below the threshold msgA-RSRP-ThresholdSSB. Basically it is the same as the field dlRSRPAboveThreshold, therefore the legacy field dlRSRPAboveThreshold can be reused in the 2-step RA report with some modifications to the field description.

- The beam quality indication performs the same function as the field dlRSRPAboveThreshold does.

The legacy field dlRSRPAboveThreshold is re-used in the 2-step RA report with some modifications to the field description.

Fallback/switch indication

Fallback and switch mechanisms are not identical.

- Fallback mechanism: UE performs MsgA transmission while receives MsgB containing fallback RAR, subsequently UE falls back to 4-step RA by performing Msg3 transmission;

- Switch mechanism: UE performs MsgA transmission but receives no response from gNB, or receives fallback RAR but ultimately fails to receive msg4, then UE will try to perform 2-step RA again, once the UE achieves the maximum number of MsgA preamble transmissions (msgA-TransMax), it will switch to 4-step random access.

Since the parameter msgA-TransMax is configured by NW and is cell-specific (BWP-UplinkCommon-> MsgA-ConfigCommon-> RACH-ConfigCommonTwoStepRA -> msgA-TransMax), and the cell ID of the cell in which the RA is performed is reported, so that the NW can be aware of the configured msgA-TransMax once it receives the RA report. Consequently, NW is able to derive that whether the UE had switched to 4-step RA according to the PerRAInfo entries recorded within the report. For instance, NW only needs to compare the the number of PerRAInfo entries and the configured msgA-TransMax, if the former is not greater than the latter, then it means UE didn’t switch to 4-step RA during the whole procedure.

- NW is able to derive that whether the UE had switched to 4-step RA according to the PerRAInfo entries recorded within the report.

Thus UE does not need to include the switch indication in the 2-step RA report. However, there is no clue for NW to figure out if UE received the fallback RAR during the RA procedure, with the current agreed information in the report. Therefore:

- There is no clue for NW to figure out if UE received the fallback RAR during the RA procedure.

Switch indication is not needed for 2-step RA report, but fallback indication should be explicitly indicated per RA attempt.

Apart from what had been agreed in the last meeting, the transmission number of MsgA is meaningful for network parameters optimization.

The 2-step RACH configuration defines the maximum number of MsgA preamble transmissions (msgA-TransMax) that UE can perform. UE will switch to 4-step RA once the transmission number of MsgA is above msgA-TransMax. Suppose the transmission number of MsgA is included in RACH report. In that case, the network can analyze the situations where RACH is successfully done and further fine-tune the parameter msgA-TransMax to meet the real-time demands. For instance, suppose most of the RACH reports collected show that the real transmission number of MsgA equals to the pre-configured msgA-TransMax, then it possibly means the value of msgA-TransMax should be increased so that the UEs which fail to perform 2-step RACH due to the restriction of small msgA-TransMax can succeed next time (successful RACH rate increased accordingly).

The 2-step RA report should also include the following information per attempt:

  - the transmission number of MsgA;

  - fallback indication.

  - Signalling model for 2-step RACH report

The structure of the legacy RA report is shown below:

Figure 1. Signalling structure of the legacy RA report

There could be at most 8 RA reports recorded, currently the RA report only denotes the single RA type (4-step RA), and the detailed information about each of the random access attempts in the chronological order of the random access attempts are provided in perRAInfoList.

As 2-step RA could fall back/switch to 4-step RA within a single RA procedure, thus the RA attempts should be included in the same RA report. However, the frequency-related parameters for 4-step RA are different from the information for 2-step RA (e.g., msg1-FrequencyStart-r16 vs. msgA-FrequencyStart-r17), so with the introduction of 2-step RA report, we can either try to a) enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters, or to b) adopt a new field ra-InformationCommon-2step to include all 2-step specific information. To be more specific, the structure of these two signalling models are given below:

a) enhance the legacy field ra-InformationCommon b) adopt a new field ra-InformationCommon-2step

Figure 2. Signalling structure of 2-step RA report

For option a), once UE falls back/switch to 4-step RA, the 4-step related frequency parameters will be recorded and the index for PerRAInfo can be counted consistently; instead, option b) would create a new field ra-InformationCommon to collect the information for 4-step RA attempts, where the index for PerRAInfo starts from 0 again. In our understanding, both options can function well, therefore we propose:

RAN2 to choose one of the following options as the signalling model of 2-step RA report:

- enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters;

- adopt a new field ra-InformationCommon-2step to include all 2-step specific information.

- Conclusion

In this paper, the following observations and proposal are given:

Observation 1	For the non-indicated entries, as the network fails to know whether DL beam quality is above or below the msgA-RSRP-ThresholdSSB, the reported entries will be ultimately ignored by the network as such information is ineffective.

Observation 2	The beam quality indication performs the same function as the field dlRSRPAboveThreshold does.

Observation 3	NW is able to derive that whether the UE had switched to 4-step RA according to the PerRAInfo entries recorded within the report.

Observation 4	There is no clue for NW to figure out if UE received the fallback RAR during the RA procedur.

Proposal 1	The beam quality should be indicated per random access attempt.

Proposal 2	LS RAN3 to confirm that the granularity of the beam quality indication should be per-RA-attempt.

Proposal 3	The legacy field dlRSRPAboveThreshold is re-used in the 2-step RA report with some modifications to the field description.

Proposal 4	Switch indication is not needed for 2-step RA report, but fallback indication should be explicitly indicated per RA attempt.

Proposal 5	The 2-step RA report should also include the following information per attempt:

a. the transmission number of MsgA;

b. fallback indication.

Proposal 6	      RAN2 to choose one of the following options as the signalling model of 2-step RA report:

a) enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters;

b) adopt a new field ra-InformationCommon-2step to include all 2-step specific information.

- References

- Chairman notes, 3GPP WG2 Meeting #112-e, Electronic meeting, November 2nd - 13th, 2020.

- R3-205797, LS to RAN2 on RACH report for 2-step RACH, RAN3, to RAN2, RAN3#109-e meeting, August 17th – 28th, 2020.