3GPP TSG-RAN WG2 #113-e	R2-2100772

Electronic meeting, 25th Jan– 5th Feb, 2021

Agenda Item:	5.4.1.4

Source:	  NTTDOCOMO, INC.

Title:	Clarification on band combination selection over inter-node message

Document for:	Discussion and Decision

1	Introduction

When implementing the band combination selection in eNB/gNB, there is a possibility that EN-DC configuration fails. This paper discuss the issue and provides the related solution.

2	Discussion

Issue for band combination selection

In general, the determination of EN-DC band combination between eNB and gNB could be summarized as following:

Step1: eNB receives band combination capabilities (UE-MRDC-Capability, UE-EUTRA-Capability, UE-NR-Capability) from UE.

Step2: eNB transmits candidate band combinations (IE: allowedBC-ListMRDC) based on the capability received from UE to the gNB

Step3: gNB selects a band combination based on the allowedBC-ListMRDC.

Step4: eNB receives the selected band combination (IE: selectedBandCombination) from gNB.

Figure1 EN-DC band combination selection procedure

We found an issue that if eNB narrows down a list of band combination and transmits it to gNB, EN-DC configuration may fail due to no band combination available for gNB. In detail, for instance, assume UE notifies eNB of UE capability of BC#1 and BC#2 as shown in the Figure2. Since eNB has no reference to NR capability, it may narrow down a list of allowed band combination to BC#1 and transmits it to gNB. Assume gNB only support NR 100MHz operation, then there is no band combination available for gNB to select. Thus, EN-DC configuration failure occurs.

Observation1: If eNB narrows down a list of band combination and transmits it to gNB, EN-DC configuration may fail due to no band combination available for gNB.

Figure2 EN-DC band combination selection issue#1

Justification to discuss this issue in 3GPP

Some Companies may argue the issue raised in this paper is related to network implementation, and it is more suitable to discuss them out of 3GPP. In fact, the issues was already discussed out of 3GPP (in O-RAN). Companies in O-RAN concerned the original concept to create the signaling of allowedBC-ListMRDC is “MN-greedy” (i.e. MN does not need to adapt to SN for band combination selection), while the proposals in this paper are to make MN to adapt to SN (i.e. “SN-greedy” approach). Since there is no explicit description in allowedBC-ListMRDC field description in RRC spec, it is necessary to clarify “SN-greedy” approach is possible for band combination selection for allowedBC-ListMRDC so as to avoid EN-DC configuration failure. (Companies in O-RAN also request for clarification regarding it in 3GPP spec)

Observation2: Companies in O-RAN think the original concept to create the signaling of allowedBC-ListMRDC is “MN-greedy” (i.e. MN does not need to adapt to SN), while there is no explicit description regarding this concept in allowedBC-ListMRDC field description in RRC spec.

Proposal1: Clarify in allowedBC-ListMRDC field description that MN may increase the probability that the SN finds a suitable SCG configuration by including in this field all entries that comprise at least the PCell band of MN.

Proposal2: Agree companion CR [2].

3. Conclusion

Based on the discussion in the previous sections, we made the following proposals:

Observation1: If eNB narrows down a list of band combination and transmits it to gNB, EN-DC configuration may fail due to no band combination available for gNB.

Observation2: Companies in O-RAN think the original concept to create the signaling of allowedBC-ListMRDC is “MN-greedy” (i.e. MN does not need to adapt to SN), while there is no explicit description regarding this concept in allowedBC-ListMRDC field description in RRC spec.

Proposal1: Clarify in allowedBC-ListMRDC field description that MN may increase the probability that the SN finds a suitable SCG configuration by including in this field all entries that comprise at least the PCell band of MN.

Proposal2: Agree companion CR [2].

4. References

- TS 38.331 v16.2.0

- R2-2100773 	Clarification on band combination selection over inter-node RRC message 	NTT DOCOMO, INC.