3GPP TSG-RAN WG2 Meeting #113 electronic					         R2-2100838

E-Meeting, Jan 25 – Feb 5. 2021

Source: 	vivo

Title:  	Support SNPN with subscription or credentials by a separate entity

Agenda Item:	8.16.2

Document for:	Discussion and Decision

Introduction

R17 NPN is aim at to support enhanced non-public network (eNPN) for NG-RAN resulting from the SA2 study on enhanced support of NPN. One of RAN objective is to specify the corresponding RAN functionality where necessary as following:

- Support SNPN along with subscription / credentials owned by an entity separate from the SNPN including:

  - The broadcasting of information to enable SNPN selection for UEs with subscription/credentials owned by an entity separate from the SNPN [RAN2]

  - The associated cell selection/reselection and connected mode mobility support [RAN2/RAN3]

  - The necessary modifications over network interfaces (e.g. NG, Xn, F1, E1 etc) [RAN3]

In this contribution, we will analyze the possible RAN enhancement to support SNPN with subscription or credentials by a separate entity.

Discussion

Background

The key issue 1 "Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN" addressed in SA2 provides a solution on how UE without subscription of a SNPN to access this SNPN by using subscriptions or credentials owned by an entity separate from the SNPN.

An example of the Support of SNPN with subscription or credentials by a separate entity is shown as below. The assumption is that UE is not configured the subscription of SNPN A. When UE enters the coverage of SNPN A, the UE attempt to access the SNPN A by using subscriptions or credentials owned by an entity separate#1 from the SNPN A. As SNPN A and the separate entities#1 have achieved agreement that SNPN A can support UE to get network connection by using subscriptions or credentials owned by an entity separate#1, UE can access to SNPN A successfully and obtain services via SNPN A.

Broadcast parameter

In SA2, two scenarios have been discussed for the support of SNPN with subscription or credentials by a separate entity. Both two scenarios are supported.

- Scenario 1: UE without subscription of SNPN accesses the SNPN by using other SNPN subscription;

- Scenario 2: UE without subscription of SNPN accesses the SNPN by using PLMN subscription;

According to the TS 23.700-07, the following SIB enhancement shall be supported.

SIB will be enhanced as follows, for SNPN only: - Indication that "access using credentials from a separate entity is supported" - Optionally, supported Group IDs (GIDs) - Optionally, an indication whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN

In NR, different SIBs are used to support different function/feature (e.g. SIB1 is used for access control). It can be found that the information above are beneficial for SNPN selection. In order to assistant UE to identify if a network support performing access by using subscriptions or credentials owned by an entity separate from the SNPN quickly, it is reasonable that put them in SIB1.

Proposal 1: For each SNPN belonging to a cell, the following information are introduced in SIB1:

  - Indication that "access using credentials from a separate entity is supported";

  - Optionally, supported Group IDs (GIDs);

  - Optionally, an indication whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN;

The Group ID as a specific case of SNPN ID reusing SNPN ID encoding in TS 23.003 [15], where

-	Assignment mode 1 indicates self-managed Home SP Group ID values as the NID Value is chosen independently at deployment time.

-	Assignment mode 0 indicates Home SP Group ID is globally unique as the NID Value is globally unique. One possibility for ensuring uniqueness is to use IANA PEN as in TS 23.003 [15].

NOTE 2:	The number of supported Group IDs that can be broadcast will be determined by RAN2.

It is found that the Group ID is a specific case of SNPN ID, which has a size of 92 bits. In R16 NPN, RAN sharing between SNPNs are supported. Considering that the maximum number of shared networks indicated in SIB1 is 12, the number of supported Group IDs can be limited to reduce signaling overhead. It is suggested that up to 12 Group IDs can be broadcasted in a cell, which means that a SNPN belonging to a cell can at least support using the subscription or credentials of a separate entity.

- Proposal 2: Up to 12 Group IDs can be broadcasted in a cell.

Network selection

For Supporting SNPN with subscription or credentials by a separate entity, the following conclusion has been achieved by SA2. In order to performing SNPN selection, the AS of UE in SNPN Access mode shall   reports the indication that access using credentials from a separate entity is supported and the supported group ID together with the corresponding available SNPN identifiers to NAS.

- For a UE configured to use SNPNs, automatic SNPN selection is performed in the following order: - UE selects and attempts to register with the SNPN it was last registered with (if available). - UE selects and attempts to register with the available SNPN identified by a PLMN ID and NID for which the UE has SUPI and credentials (i.e. as in Rel-16). - UE selects an available and allowable SNPN, which broadcasts "access using credentials from a separate entity is supported" indication and an SNPN ID contained in the user-controlled list (if available) - UE selects an available and allowable SNPN which broadcasts "access using credentials from a separate entity is supported" indication and an SNPN ID contained in the separate entity-controlled list (if available) - UE selects an available and allowable SNPN which broadcasts "access using credentials from a separate entity is supported" indication and a GID contained in the separate entity-controlled list (if available) - If the UE has not found an available and allowable SNPN based on the above, the UE selects an available and allowable SNPN which broadcasts an indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN: - If the UE detects a plurality of such SNPNs, the order in which the UE selects and attempts a registration with an SNPN is implementation specific

Proposal 3: For each available SNPN, AS reports the following information (if any) together with the SNPN identifier to NAS:

  - Indication that "access using credentials from a separate entity is supported";

  - Optionally, supported Group IDs (GIDs);

  - Optionally, an indication whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN;

Cell selection/reselection

In TS 38.304, the definition of suitable cell for UE in SNPN Access Mode is shown as following

For UE operating in SNPN Access Mode, a cell is considered as suitable if the following conditions are fulfilled: - The cell is part of either the selected SNPN or the registered SNPN of the UE; - The cell selection criteria are fulfilled, see clause 5.2.3.2;

For Supporting SNPN with subscription or credentials by a separate entity, the definition of suitable cell for UE in SNPN Access Mode shall be modified. Based on highlighted text in 2.3, the following description on the suitable cell definition for UE operating in SNPN Access Mode is suggested.

For UE operating in SNPN Access Mode, a cell is considered as suitable if either the following conditions are fulfilled: - The cell is part of a SNPN for which the indication that access using credentials from a separate entity is supported" is broadcast by the cell, and for that SNPN either: - The SNPN ID of that SNPN is contained in the user-controlled list or separate entity-controlled list in the UE; - a GID supported by that SNPN is contained in the separate entity-controlled list in the UE; - The cell is part of a SNPN for which the indication that that SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN is broadcast by the cell

Proposal 4: For UE operating in SNPN Access Mode, a cell is considered as suitable if either the following conditions are fulfilled:

- The cell is part of a SNPN for which the indication that access using credentials from a separate entity is supported" is broadcast by the cell, and for that SNPN either:

-	The SNPN ID of that SNPN is contained in the user-controlled list or separate entity-controlled list in the UE;

-	a GID supported by that SNPN is contained in the separate entity-controlled list in the UE;

- The cell is part of a SNPN for which the indication that that SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN is broadcast by the cell.

Conclusions

Based on the analysis given above, we have the following Observations and Proposals:

Proposal 1: For each SNPN belonging to a cell, the following information are introduced in SIB1 :

  - Indication that "access using credentials from a separate entity is supported";

  - Optionally, supported Group IDs (GIDs);

  - Optionally, an indication whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN;

- Proposal 2: Up to 12 Group IDs can be broadcasted in a cell.

Proposal 3: For each available SNPN, AS reports the following information (if any) together with the SNPN identifier to NAS:

  - Indication that "access using credentials from a separate entity is supported";

  - Optionally, supported Group IDs (GIDs);

  - Optionally, an indication whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN;

Proposal 4: For UE operating in SNPN Access Mode, a cell is considered as suitable if either the following conditions are fulfilled:

- The cell is part of a SNPN for which the indication that access using credentials from a separate entity is supported" is broadcast by the cell, and for that SNPN either:

-	The SNPN ID of that SNPN is contained in the user-controlled list or separate entity-controlled list in the UE;

-	a GID supported by that SNPN is contained in the separate entity-controlled list in the UE;

- The cell is part of a SNPN for which the indication that that SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN is broadcast by the cell.

Reference

- RP-202363, “Enhancement of Private Network Support for NG-RAN”.

- 3GPP TR 23.700-07, “Study on enhanced support of non-public networks”