3GPP TSG-RAN WG2 Meeting #113 electronic							  R2-2100847

Online, Jan 25 – Feb 5, 2021

Agenda Item:	8.2.3

Source: 	OPPO

Title:  	Discussion on conditional PSCell addition

Document for:	Discussion and decision

- Introduction

In the last RAN2#112e meeting, regarding CPA, agreements have been achieved, as illustrated as follows:

In MN initiated inter-SN CPC and CPA, the MN is not required to indicate the execution condition(s) to other involved entities (e.g. target SN, source SN).

For CPA and MN initiated Inter-SN CPC, the MN generates and transmits the conditional configuration message (i.e. RRCReconfiguration/RRCConnectionReconfiguration message) to the UE.  The RRCReconfiguration provided by the candidate PSCell(s) is encapsulated in the final conditional reconfiguration message to the UE. The MN is not allowed to alter the RRCReconfiguration provided by the candidate PSCell(s).

In this contribution, we will focus our analysis on CPA.

- Discussion on conditional PSCell addition

2.1 the procedure of PSCell addition

According to the last RAN2 agreement, in the CPA, the MN is not required to indicate the execution conditions to other involved entities (e.g., target SN, source SN). In addition, the MN should generate and transmit the conditional configuration message consisting of the execution conditions and the RRCReconfiguration provided by the candidate PSCells to the UE. As a result, a possible implementation is shown as follows:

Figure 1: the proposed control-plane signalling procedure for conditional PSCell addition

In the figure 1, MN determines the candidate SN based on the measurement results provided by the UE, and then forwards the conditional SN addition Request towards the candidate SN. Similar with the legacy SN addition Request, in the conditional SN addition Request, MN will provide the requested PDU Sessions/QoS Flows information, SCG configuration information, latest measurement results for SN to choose and configure the SCG cells, and the request of the radio resources to be allocated for split SRB operation, all the needed security information to enable SRB3 to be setup, Xn-U UL TNL address information for MN terminated bearers, a list of available DRB IDs for SN terminated bearers.

Observation 1: Since the execution condition is not needed to be indicated to the candidate SN, similar contents of legacy SN addition Request could be reused for the conditional SN addition Request.

Subsequently, conditional SN Addition Request ACK msg consisting of the new SCG radio resource configuration (including the PSCell, other SCG SCells and the corresponding radio resources), Xn-U TNL address information for the respective DRB, Xn-U UL TNL address information for SN terminated bearers, Xn-U DL TNL address information for MN terminated bearers, NG-U DL TNL address information and security algorithm for the SN terminated bearers should be feedbacked to the MN.

Observation 2: similar contents of legacy SN addition Request ACK message could be reused for the conditional SN addition Request ACK message.

Then, according to the agreements achieved in the last RAN2 #112e meeting, the RRCReconfiguration message consisting of the execution condition set per target PSCell and the configuration provided by the candidate SN Addition Request ACK msg could be sent by the UE in the step 3 shown in the figure 1. To confirm the reception of the RRCReconfiguration message, UE needs to send back the RRCReconfigurationComplete message towards the MN.

Proposal 1: RAN2 to agree that UE needs to send back the RRCReconfigurationComplete message towards the MN to confirm the reception of the RRCReconfiguration message consisting of the execution condition and the configuration of the target PSCells.

After reception of such RRCReconfiguration message, UE starts to evaluate whether or not the measurement results of the target PSCells meet the condition. Once the condition is met, the UE will perform RACH access towards the corresponding target PSCell. If the RACH procedure is completed, UE needs to send a RRCReconfigurationComplete message towards the SN. If the SRB3 has been established, UE could choose to send the RRCReconfigurationComplete msg towards the SN directly. Otherwise, UE should send the ULInformationTransferMRDC msg including a container with RRCReconfigurationComplete msg inside towards the MN, and then the MN will forward the RRCReconfigurationComplete msg towards the SN.

Proposal 2: RAN2 to agree that, upon completion of random access procedure towards candidate SN, UE shall transmit RRCReconfigurationComplete msg towards the SN via SRB3 if SRB3 is available, or to be included in a container in a UL RRC message (e.g., ULInformationTransferMRDC msg) towards MN via SRB1. In the second case, the MN should forward the RRCReconfigurationComplete msg towards the SN.

2.2 left FFSs

In the last RAN2#112e meeting, some FFSs are inherited from the RAN2#108 Reno meeting, indicated as follows:

FFS how many candidate cells (UE and network impacts should be clarified). FFS whether the number of candidate cells for CPAC different from that of CHO.

FFS on UE capability for triggering quantities.

In our opinion, these two FFS could be referred to the discussion of CHO in R16. Since there is no obvious reason why the maximum number of candidate cells to be configured for CPA should be different from what for CHO, the number should be defined as 8. For the second FFS, also similar with CHO, there is no need to introduce separate capability IE. The UE supporting the feature of CPA should in default has the capability of monitoring two triggering quantities simultaneously.

Proposal 3: RAN2 to agree that the maximum number of candidate cells to be configured for CPA and the way of definition of the UE capability for triggering quantities for CPC should be aligned with the CHO.

- Conclusions

Based on the discussion above, following observations and proposals are made:

Observation 1: Since the execution condition is not needed to be indicated to the candidate SN, similar contents of legacy SN addition Request could be reused for the conditional SN addition Request.

Observation 2: similar contents of legacy SN addition Request ACK message could be reused for the conditional SN addition Request ACK message.

Proposal 1: RAN2 to agree that UE needs to send back the RRCReconfigurationComplete message towards the MN to confirm the reception of the RRCReconfiguration message consisting of the execution condition and the configuration of the target PSCells.

Proposal 2: RAN2 to agree that, upon completion of random access procedure towards candidate SN, UE shall transmit RRCReconfigurationComplete towards the SN via SRB3 if SRB3 is available, or to be included in a container in a UL RRC message (e.g., ULInformationTransferMRDC msg) towards MN via SRB1. In the second case, the MN should forward the RRCReconfigurationComplete towards the SN.

Proposal 3: RAN2 to agree that the maximum number of candidate cells to be configured for CPA and the way of definition of the UE capability for triggering quantities for CPC should be aligned with the CHO.