3GPP TSG-RAN WG2 Meeting #113-e	R2-2100857

e-Meeting, 25 Jan - 05 Feb 2021

Agenda item:	8.5.4

Source:	Apple

Title:	Reliability enhancements for CG/SPS

Document for:	Discussion / Decision

# Introduction

The use of survival time was initially discussed during the RAN2#112e and RAN2 reached following agreement [2].

The discussion of RAN enhancements based on new QoS parameters is expected to continue in RAN2#113e. This contribution analyses QoS aspects related to survival time and concludes to propose MAC enhancements to increase message protection through pre-defined config of higher reliability during critical transmission periods.

# Background

## The concept of survival time

TS 22.104 [3] and TR 22.832 [4] contain an overview of the concepts behind survival time. TS 22.104 defines survival time as “the time that an application consuming a communication service may continue without an anticipated message“.

Survival time in TR 22.832

- Survival time indicates to the communication service the time available to recover from message delivery failures. The survival time is expressed as a time period which, especially with cyclic traffic, accommodates the maximum number of consecutive incorrectly received or lost messages that can be tolerated without causing an application layer failure.

- For applications that have survival time equal to zero, any loss of packet triggers this unavailability, while for applications with non-zero survival time only two or more consecutive packet losses will trigger unavailability (depending on the agreed traffic periodicity and length of the survival time).

Definition of communication service availability (per TS 22.261 [5])

- Communication service availability: percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area.

  - NOTE 3: The end point in "end-to-end" is assumed to be the communication service interface.

  - NOTE 4: The communication service is considered unavailable if it does not meet the pertinent QoS requirements. If availability is one of these requirements, the following rule applies: the system is considered unavailable if an expected message is not received within a specified time, which, at minimum, is the sum of maximum allowed end-to-end latency and survival time.

Figure 1: Example with lost messages from 22.104, annex C.3

Figure 1 shows the relation between logical communication link, communication service and application state, refer to TS 22.104, Figure C.3-1. At the application layer, at least one message is expected within the interval described by the sum of maximum end-to-end latency and survival time.

TR 22.832 assumes that if packet errors are detected but survival time has not yet expired, steps could be taken to ensure delivery of subsequent packets within survival time. One potential use of survival time could be to adjust the packet error rate (PER) if survival time is in jeopardy. This may translate into following simple condition:

if( (packet error detected) && (survival timer running) ) {trigger a suitable method for error protection;}

We think that the period of survival time and application recovery should be guarded by start/stop events. Depending on the situation different protection mechanisms may be used to increase reliability. We assume that the network can select suitable protection mechanisms based on UE capabilities.

Proposal 1: The period of survival time and potentially applicable application recovery should be guarded by start/stop events.

Proposal 2: The error protection method during survival time and application recovery (if any) is configurable by the network based on UE capabilities.

## Lower layer protection

IIoT/URLLC applications for deterministic communication typically rely on configured grants (CG) in uplink and semi-persistent scheduling (SPS) in downlink. Since the latency is very short for many IIoT applications, RLC acknowledged mode may not be enabled. Message repetition or ARQ through RLC AM may be too slow for URLLC applications with very low latency requirements. Under such circumstances, there is a need for better protection and recovery on MAC/PHY level, e.g., through HARQ, where a higher degree of reliability may be required for a period of time. The aim is to avoid losing the communication service by adding extra protection for a) ongoing messages that are in-flight or b) new messages, e.g., during survival time.

This paper assumes that the UE detects the start/end of survival time through a suitable method, either by network configuration or within the UE itself, which can result into a survival time start event and a survival time end event.

Figure 2: Example scenario to avoid

Depending on the length of survival time, the end-to-end latency and achievable HARQ ReTx timing, additional reliability may be required for certain messages, like the last application layer messages or any pending messages in the pipeline. This leads to the question how CG/SPS can be enhanced to provide an extra level of configurability and reliability.

Observation 1: Enhancements are required to provide an extra level of configurability and reliability for CG and SPS.

# Discussion

For many industrial applications the number of application layer messages per transfer interval is small (e.g., a single message). Likewise, the message size is often small. TS 22.104 provides the service performance requirements. When survival time is zero a single message loss leads to application downtime. Other IIoT applications may have short survival times like one transfer interval where the application transfers into a down state after two missed packets. In such cases the transmission must succeed without retransmission from application layer, thus sufficient protection is important on lower layers. There may as well be cases where, at the start of survival time or after the initial message when survival time is zero, a UE has still unacknowledged (or even NACKed) data in the HARQ buffer.

## HARQ retransmission enhancements

In order to limit over-provisioning of radio resources but increase reliability when required, one option is to provide additional redundancy / better reliability for the HARQ ReTx only. For as long as InitialTx succeeds with normal settings, no over-provisioning of the reliability target is needed.

The current CG design relies on dynamic grants for HARQ ReTx. This provides only limited means for reliability adjustments, since DG parameters are based on a common PUSCH-config and cannot be changed. Refer to 38.214 and 38.212 for the treatment of CS-RNTI with NDI=1.

For uplink, TS 38.214 (version g30) states: “For the PUSCH retransmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, the parameters in pusch-Config are applied for the PUSCH transmission except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex described in Clause 7.1 of [6, TS 38.213], mcs-Table, mcs-TableTransformPrecoder described in Clause 6.1.4.1 and transformPrecoder described in Clause 6.1.3.“

For downlink, TS 38.214 (version g30) states: “When receiving PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, if the UE is configured with pdsch-AggregationFactor in pdsch-config, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots.”

One method to overcome these limitations is to allow HARQ InitialTx and HARQ ReTx using an extended or a different CG/PHY config for an enhanced level of protection. Unacknowledged MAC PDUs sitting in the HARQ buffer can be retransmitted with more reliability, more repetitions, differently spaced in time and frequency, etc. — while the TBS remains the same.

Proposal 3: A CG/SPS config can utilize an enhanced configuration to allow an increased level of protection during HARQ retransmissions.

The retransmission enhancements may be used during survival time only (reactive mode). As an alternative, the enhancements may be used for any HARQ retransmission, regardless of survival time (proactive mode). A new UE capability can be added for this function.

The following two figures show an example how these CG/SPS enhancements could be used to ensure message protection during critical periods, using the approach in proposal 3. For the sake of simplicity, the PDB (and end-to-end latency) are assumed to be less than or equal to the transfer interval. Figure 3 and 4 show that the number of TB repetitions K (repK) is increased during HARQ retransmission, but other parameters could be altered during the retransmission as well.

Figure 3: HARQ ReTx during survival time

Figure 3 shows a case where the retransmission configuration is altered for a CG during survival time only. Figure 4 shows a case where the retransmission configuration is altered for any HARQ ReTx when the survival time is zero.

Figure 4: HARQ ReTx when survival time is zero

Another option to achieve a higher level of protection during survival time would be to allow HARQ ReTx in configured grant resources where certain CG parameters can be modified during retransmission. As a result, the reliability of HARQ ReTx can be selectively adjusted according to the requirements of the service.

A number of updates to the operation of configured grants are conceivable, too.

- An option for a CG to occur one-time. This may be implemented by making the ’periodicity’ parameter in configuredGrantConfig optional. In other words, such a CG does not have a periodicity.

- An option for a CG to end after a predefined number or periodicities, e.g., once started, the CG ends automatically after x-number of transmit occasions. This may require a new parameter in configuredGrantConfig.

Proposal 4: Utilize HARQ ReTx in configured grant resources where certain CG/SPS parameters can be modified during retransmission.

The following section 3.1.1 lists two options to specify the HARQ retransmission enhancements in proposal 3 and 4.

### Option 1: HARQ ReTx protection via selected CG/SPS parameter configuration

A straight-forward option would be to supply the configuredGrantConfig for a specific feature or functionality with additional parameters valid during HARQ ReTx phases only. Thereby the same feature can use different parameter values during InitialTx and ReTx/survival time. For example, to trigger a different number of repetitions during HARQ InitialTx and HARQ ReTx, an additional repK parameter could be defined (let’s call it repK-ReTx) that applies under conditions such as HARQ retransmission in survival time. Suitable other parameters could be defined in a similar way (e.g., periodicity-ReTx, timeDomainOffset-ReTx, pusch-RepTypeIndicator-ReTx, etc.).

One important thing to note is that the TBS needs to stay the same, since the MAC PDU is already in the HARQ buffer and the UE should not need to recreate the MAC PDU. The retransmission invokes the same HARQ process such that the CG keeps using the same HARQ ID and the same CG index.

This enhancement option could be used when only a subset of CG parameters needs to be adjusted. The annex in section 6.1.1.1 shows a potential implementation of this approach.

The typical case would be a HARQ retransmission of a configured grant through a dynamic grant. Here the CG ReTx is triggered by a DCI addressed to CS-RNTI with NDI=1 for a HARQ ID on a given CC/cell. The CG-PUSCH is retransmitted one time. Conditions to enable the new retransmission parameter (e.g., in survival time) can be specified.

Furthermore, a HARQ ReTx might be configured to automatically utilize CG resources (proposal 4) through one of the two alternative options below. The validity period of the new ReTx parameters might end at the expiry of the configuredGrantTimer.

- Alternative 1: HARQ ReTx starts at the next regular CG occasion.

- Alternative 2: HARQ ReTx does not need to wait until the next CG occasion. Early start by a different CG offset and periodicity.

However, unless Rel-17 allows for an explicit CG HARQ NACK (which is not currently the case), alternatives 1 and 2 might seem less favorable.

Proposal 5: Consider HARQ retransmission protection via selected CG/SPS parameter configuration as option 1.

A similar ‘selected parameter config’ can be applied to DL SPS as well. The need for recovery during survival time is more pronounced in UL though. For the DL, the network has more control over scheduling and transmission of data. The annex in section 6.1.1.2 contains a potential implementation of this approach for the DL.

Proposal 6: RAN2 to discuss if downlink changes are needed for SPS.

### Option 2: HARQ ReTx protection via complete CG/SPS configuration

When multiple parameters are meant to change, a separate (retransmission) CG/SPS config could be used as a secondary configuredGrantConfig that is valid during retransmission only (i.e., a shadow or overlay to configuredGrantConfig).

The annex in section 6.1.2.1 shows a potential implementation of this approach.

The configuredGrantRetransmissionConfig IE uses the exact same CG indices (configuredGrantConfigIndex, configuredGrantConfigIndexMAC) and HARQ process IDs (nrofHARQ-Processes) as ConfiguredGrantConfig, but with a different set of parameters. Strictly delta config: The configuredGrantRetransmissionConfig may contain just the delta to configuredGrantConfig. The configuredGrantRetransmissionConfig is valid during HARQ retransmission periods only.

Regardless of HARQ retransmission in dynamic grant (DG) or configured grant (CG) resources, the configuredGrantRetransmissionConfig may be valid for single or multiple CG transmit occasions.

- Single transmit: This is like a one-time CG with no periodicity. Within the base configuredGrantConfig, parameter ‘periodicity’ can be made optional. Both ‘periodicity’ and ’periodicityExt-r16’ are not present in ConfiguredGrantConfig. This may the usual use-case, typically used on DGs.

- Multi transmit occasion: based on periodicity+offset and a new parameter to indicate the number of CG transmissions.

Note that even when a retransmission is triggered by a CS-RNTI with NDI=1 over a DG, as an option, a CG could still use multiple transmit occasions. Typical use-case: configuredGrantRetransmissionConfig is triggered by a DCI addressed to CS-RNTI with NDI=1 and HARQ ID on a given CC/cell. (Enabled by L1 config.)

Proposal 7: Consider HARQ retransmission protection via complete CG/SPS config as enhancement option 2.

## Enhancements for both InitialTx and ReTx

Similar to option 2 described in section 3.1.2 and as a further extension, an alternative configuredGrantConfig could be defined along with the existing configuredGrantConfig. The alternative configuredGrantConfig could be used not only for retransmissions (as in option 2) but for initial transmissions as well. This represents a slightly more general approach, and it could be defined how the alternative configuredGrantConfig is applied and when it is valid.

To include the HARQ InitialTx in the extra protection may be useful when the latency is very short such that there is no time for HARQ ReTx or when an extra level of protection is required due to other factors. It may as well be useful in scenarios where message retransmission by upper layers (e.g., RLC AM) is applicable.

Figure 5 shows an example where the first transfer interval uses the original configuredGrantConfig and the second transfer interval uses an alternative configuredGantConfig mapped to the same CG. The alternative configuredGrantConfig has additional redundancy with a higher number of TB repetitions. Other parameters could be altered as required.

Figure 5: Example with modified InitialTx

The alternative configuredGrantConfig provides a variant of a CG config for an existing CG, that is, only one of the two CG configs can be used at a time. An alternative CG config could be dynamically enabled/disabled.

The annex in section 6.1.3.1 shows a potential implementation of this approach.

The configuredGrantAlternativeConfig IE uses the exact same CG indices (configuredGrantConfigIndex, configuredGrantConfigIndexMAC) and HARQ process IDs (nrofHARQ-Processes) as ConfiguredGrantConfig but with a different set of parameters. The configuredAlternativeConfig may contain just the delta to configuredGrantConfig on a subset of CGs, or it may be a full new config (including different TBS).

- The ‘full new config’ option might be used for InitialTx, for example, when new data arrives during survival time and a new TBS or a sufficiently different CG configuration is desired. The ‘delta’ option might be suitable for HARQ ReTx, utilizing the same TBS. However, both options may be used in all combinations.

- The configuredGrantConfig is the base list. ConfiguredGrantAlternativeConfig only contains CG entries that require different treatment, as not all logical channels may require multiple CG configurations. In other words, if a particular CG index is present in one CG config only then it shall only be present in the configuredGrantConfig.

- In case of a HARQ ReTx, the same HARQ buffer can be used — same as with options 1/2.

Since the CG index connects (overlays) different CG configs for the same CG, all LCP restrictions (allowedCG-List-r16) apply in the same way to both sets of CG configs. Therefore, there is no change to LCP restrictions needed.

The configuredGrantAlternativeConfig can be used when enabled, for example, during “survival time” on a specific LCH or when enabled by a “start event”. Applicability of configuredGrantAlternativeConfig could be based on network config, for example, including the following options: InitialTx-and-ReTx, InitialTx-only, ReTx-only. This might be controlled on the DCI or with an explicit RRC parameter. Whether ReTx is over CG or over DG could be configured as well. A new UE capability can be defined for this function.

Figure 6 below shows another example of a use-case. Only one LCH and one CG is shown. The first transfer interval uses the base CG config, and the alternative CG config is used during survival time. Each arrow represents either a single packet (at higher layers) or a TB (at MAC/PHY). For the transfer interval in survival time, the first CG bundle fails, even though this transmission is already using the alternative CG config (with TB repetition), because radio conditions are really bad. But with the next occasion of the CG bundle (for the same alternative CG config), in this case triggered by RLC ReTx, the transmission succeeds. Many other scenarios are possible.

Figure 6: Example with modified InitialTx and RLC AM

The usage of configuredGrantAlternativeConfig is subject to a set of separate conditions. When conditions apply the UE can use configuredGrantAlternativeConfig autonomously.

Proposal 8: RAN2 to consider the introduction of enhancements for protection of both InitialTx and ReTx via an alternative CG/SPS config.

## LCP restrictions

Another area for potential enhancements is the use of LCP restrictions. A logical channel (LCH), during survival time, could be temporarily mapped to a different set of CG resources configured with higher reliability. The set of CG resources used for this purpose could be collected in a separate list. As the sole purpose of such a CG is to provide additional redundancy during specific intervals only, it could be called a ‘dormant’ CG or allowedDormantCG-List.

A dormant CG is a normal CG that is defined through configuredGrantConfig. Just like a normal CG, it needs to be activated before use, however, the UE does not use it for most of the time. The dormant CG remains dormant until a failure or an event occurs, only then the UE is allowed to use it (autonomously — or as an option, after NW config).

For a given LCH, a sub-set of CGs can be mapped to the allowedCG-List and another sub-set of CGs (that is, different CGs) can be mapped to the allowedDormantCG-List. The dormant CG list is meant to be used under special circumstances only. For example, during survival time or for temporary recovery under bad radio conditions, at pre-defined conditions. This allows for fast adjustment without the need to configure/update additional CGs. CGs mapped to the allowedDormantCG-List may have higher reliability, shorter periodicity, different number of TB repetitions, etc.

One may argue that a similar behavior might as well be achieved with a CG type2 that gets activated when needed, however, a type 2 CG still requires MAC signalling. Such signalling might be too slow for a successful recovery during survival time. In contrast, a dormant CG can be used right away. The complexity of this approach, including HARQ buffer handling, may require further analysis.

The annex in section 6.1.4 shows a potential implementation of this approach.

A dormant CG is already activated beforehand (e.g., as a type1 CG via RRC), but it is not used until the new/dormant LCP restriction is active. Conditions to activate the new/dormant LCP restriction can be pre-defined, for events such as:

- survival time start event; or

- RRC configuration; or

- other conditions: FFS

A dormant LCP restriction can be deactivated through a set of pre-defined events, such as:

- survival time end event (or expiry of the survival timer, if configured); or

- RRC configuration; or

- Max number of CG transmit occasions reached; or

- other conditions: FFS

Furthermore, as with previous enhancement options, some CGs may be limited to a maximum number of CG transmit occasions. With that, the CG has a validity period in units of periodicity. When the configured max number of consecutive transmit occasions is exhausted, the CG becomes dormant again.

Proposal 9: RAN2 to evaluate whether additional LCP restrictions can be introduced.

The LCP restriction for dormant CGs can be configured to be activated based on pre-defined conditions.

- automatic activation/deactivation; or

- permanent config

Note that this enhancement option covers only the uplink but not the downlink.

# Conclusions

This paper discusses QoS aspects related to survival time and  proposes MAC enhancements to increase reliability during critical transmission periods. We have the following observations and proposals:

Proposal 1: The period of survival time and potentially applicable application recovery should be guarded by start/stop events.

Proposal 2: The error protection method during survival time and application recovery (if any) is configurable by the network based on UE capabilities.

Observation 1: Enhancements are required to provide an extra level of configurability and reliability for CG and SPS.

Proposal 3: A CG/SPS config can utilize an enhanced configuration to allow an increased level of protection during HARQ retransmissions.

Proposal 4: Utilize HARQ ReTx in configured grant resources where certain CG/SPS parameters can be modified during retransmission.

Proposal 5: Consider HARQ retransmission protection via selected CG/SPS parameter configuration as option 1.

Proposal 6: RAN2 to discuss if downlink changes are needed for SPS.

Proposal 7: Consider HARQ retransmission protection via complete CG/SPS config as enhancement option 2.

Proposal 8: RAN2 to consider the introduction of enhancements for protection of both InitialTx and ReTx via an alternative CG/SPS config.

Proposal 9: RAN2 to evaluate whether additional LCP restrictions can be introduced.

# References

[1] RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR

[2] R2-2010838, Reply LS on Use of Survival Time for Deterministic Applications in 5GS

[3] TS 22.104: Service requirements for cyber-physical control applications in vertical domains

[4] TR 22.832: Study on enhancements for cyber-physical control applications in vertical domains

[5] TS 22.261: Service requirements for the 5G system

# Annex with text proposals

This section contains initial details for the sketch of enhancements in the different options being proposed. At this stage the indicated changes are only for TS 38.331, with the intention to show the principle. Some of the TPs will need to be accompanied with further changes in RAN1 and MAC.

## Radio resource control information elements

### HARQ ReTx protection via selected parameter configuration

#### Uplink

Unchanged parts omitted

…

Figure 7: HARQ ReTx parameter in uplink

repK-ReTx

Number of repetitions K during HARQ retransmission.

Unchanged parts omitted

#### Downlink

Unchanged parts omitted

…

Figure 8: HARQ ReTx parameter in downlink

pdsch-AggregationFactorReTx

Number of repetitions for SPS PDSCH during HARQ retransmission. When the field is absent, the UE applies PDSCH aggregation factor defined in pdsch-AggregationFactor.

Unchanged parts omitted

### HARQ ReTx protection via retransmission CG/SPS configuration

#### Uplink

Unchanged parts omitted

…

Figure 9: HARQ ReTx in uplink (alternative)

configuredGrantRetransmissionConfig

Configuration for a Configured-Grant of type1 or type2 during HARQ retransmission in survival time. The configuredGrantRetransmissionConfig uses the exact same CG indices (configuredGrantConfigIndex, configuredGrantConfigIndexMAC) and HARQ process IDs (nrofHARQ-Processes) as ConfiguredGrantConfig.

Unchanged parts omitted

#### Downlink

Unchanged parts omitted

…

Figure 10: HARQ ReTx in downlink (alternative)

sps-RetransmissionConfig

UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP that can be configured for HARQ retransmission. The sps-RetransmissionConfig uses the exact same SPS indices and HARQ process IDs as sps-Config.

Unchanged parts omitted

### HARQ InitialTx and ReTx protection via alternative CG/SPS configuration

#### Uplink

Usage of configuredGrantAlternativeConfig can be subject to a set of separate conditions. When conditions apply the UE may use configuredGrantAlternativeConfig autonomously.

Unchanged parts omitted

…

Figure 11: Alternative configuredGrantConfig

configuredGrantAlternativeConfig

Configuration for a Configured-Grant of type1 or type2 in survival time. The configuredGrantAlternativeConfig is associated with ConfiguredGrantConfig and uses the exact same CG indices (configuredGrantConfigIndex, configuredGrantConfigIndexMAC) as ConfiguredGrantConfig.

Unchanged parts omitted

#### Downlink

Unchanged parts omitted

…

Figure 12: Alternative SPS Config

sps-AlternativeConfig

UE specific alternative SPS (Semi-Persistent Scheduling) configuration for one BWP that can be configured. The sps-AlternativeConfig uses the exact same SPS indices as sps-Config.

Unchanged parts omitted

### LCP restrictions

The LCP restriction for dormant CGs can be configured to be activated based on pre-defined conditions.

- automatic activation/deactivation; or

- permanent config

Unchanged parts omitted

…

Figure 13: New LCP restriction for dormant CGs

allowedDormantCG-List

This restriction applies only when the UL grant is a dormant configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configurations. Corresponds to "allowedDormantCG-List" as specified in TS 38.321 [3].

Unchanged parts omitted