3GPP TSG-RAN WG2 Meeting #113-e            R2-2100876 resubmission of R2-2009473

Electronic, Jan 25 - Feb 05, 2020

Agenda item:	8.8.2

Source:	Apple

Title:	Discussion on slice based cell selection and re-selection

Document for:	Discussion

# 1	Introduction

In RAN slicing SI, the main objectives related to RAN2 are listed as below from SID [1], with a quite tight timeline expecting to be completed in Dec. 2020.

- Study mechanisms to enable UE fast access to the cell supporting the intended slice, including [RAN2]

- Slice based cell reselection under network control

- Slice based RACH configuration or access barring

Note: whether the existing mechanism can meet this scenario or requirement can be studied.

Note: This study item should take SA2 output on slicing enhancement into consideration if RAN impacts are identified.

In this paper, we discussed the aspects related to slice based cell selection and reselection.

# 2   Discussion

Topic 1: Slice based dedicated priority mechanism

In RAN2#111 meeting, RAN2 made the following decision regarding dedicated priority mechanism.

Identify the problem with existing mechanisms with dedicated priority and study if some enhancements are needed

This topic was intensively discussed in email discussion [2] and [3], and a list of issues were mentioned. In this contribution, we would like to emphasize one specific issue. In details, dedicated priority configuration is only valid in a small area, once UE moves out of this area, the configuration maintained at UE (T320 still running) is not correct any more. Current UE operation is the dedicated priority configuration overrides broadcast signaling when T320 is running. The issue can get quite severe since slice-frequency mapping is not consistent in a large geographical area. For example:

- For eMBB, both F1 and F2 are used in area 1, while only F2 is available in area 2.

- The frequency priority for one slice could be different in different areas: Still using eMBB as example, F1 is prioritized in area 1 while F2 is prioritized in area 2.

- One slice is not available in certain area: Suppose UE is interested in URLLC slice, if UE is provided with “F2>F1” in area 1 in dedicated priority, and then moves to area 2, UE would perform access on F2. But actually F2 does not support URLLC in area 2. NW will reject the PDU session and the S-NSSAI for URLLC.

Observation 1: Current dedicated priority mechanism does not work properly since the dedicated priority configuration is only valid in a small area, and UE may move out of the area when T320 is still running.

If RAN2 would like to solve the issue, a similar solution like validity area RAN2 introduced for DC/CA early implementation can be considered.

Proposal 1: RAN2 to discuss whether the validity issue in dedicated priority mechanism should be solved.

Topic 2: Slice based cell selection and reselection based on broadcast signaling

In order to facilitate a fast access to the right cell supporting the intended slice, NW should provide the slice related information to UE via broadcast signaling. In this aspect, the assistant information could comprise of the following.

1) Slice types supported by current cell

This is the very basic scheme to allow the NW to unambiguously indicate to the devices the slice types the cell supports, based on which UE can prioritize certain cells over others in line with its interests.

2) Slice type specific cell selection and re-selection parameters

Given the wide types of devices which are expected to derive service from the 5G RAN, it becomes imperative that the device service requirements are factored in while selecting/re-selecting to a cell. For example, a low power cellular IoT device will have a different maximum output transmit power, receive level sensitivity, path loss criteria compared to a smartphone, which in turn would have a different criterion. The motivation is to allow devices with different device characteristics to be distinguished and provided an optimal camping criteria. It is proposed that NW advertises a set of cell specific selection/re-selection parameter, e.g., q-RxLevMin, q-QualMin, s-NonIntraSearchP, s-NonIntraSearchQ specific to individual slice types supported by that cell in SIB.

3) Slice types supported by neighbor cells

In order to ensure continuity of services as part of idle/inactive mode mobility, it’s recommended that UE prioritizes cell re-selection to those cells (intra or inter frequency) that supports services UE is interested in. Accordingly, the NW can provide the slice types supported by neighbor cells in SIB.

Proposal 2: Suggest to discuss that NW to broadcast slice type related information such as slice types supported by current cell and neighbor cells, slice type specific cell selection and re-selection parameters.

# 3	Conclusions

Based on the discussion above, we have the following proposals.

Observation 1: Current dedicated priority mechanism does not work properly since the dedicated priority configuration is only valid in a small area, and UE may move out of the area when T320 is still running.

Proposal 1: RAN2 to discuss whether the validity issue in dedicated priority mechanism should be solved.

Proposal 2: Suggest to discuss that NW to broadcast slice type related information such as slice types supported by current cell and neighbor cells, slice type specific cell selection and re-selection parameters.

# 4	References

- RP-193254    SID-Study on enhancement of RAN Slicing      CMCC, Verizon

- R2-2010366 Report of [Post111-e][916][Slicing] RAN slicing study questions (CMCC)

- R2-21xxxxx Report of [Post112-e][253][RAN slicing] Prioritized solutions for RAN slicing (CMCC)