3GPP TSG RAN WG2 Meeting #113-e                	R2-2100912

# Online meeting, 25 January-5 February 2021

Agenda item:		8.9.2

Source:			Sony

Title:		Discussion on TRS/CSI-RS configuration of idle/inactive-mode UEs

Document for:	 	Discussion and decision

# Introduction

A Work Item on UE power saving enhancements was agreed at RAN plenary #86 [1]. One of the objectives in the WID is the following;

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

Related to the objective 1b above, RAN1 made an agreement in RAN1#103-e meeting on the high level principle of signalling method for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE (s) in Rel-17 UE power saving enhancements [2].

Agreements:

- SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

  - Up to RAN2 to decide which SIB is to be used.

  - Whether or not to additionally support other high-layer signaling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2

The agreement is summarized in an LS with an action for RAN2 to take the signalling method for the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s) into account in the corresponding RAN2 study.

In this contribution, we discuss the signaling method for the TRS/CSI-RS configuration for idle/inactive mode UEs.

# Discussion

RAN1 agreed to use SIB to convey the configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs. In the legacy NR, the following SIBs have been introduced:

| System Information | Content |
| --- | --- |
| MIB | SFN, cell barring info, SIB 1 reception information |
| SIB 1 | Cell identity, Serving cell information (BWP, RACH, Paging info), TAC, RANAC, Other system information (OSI) scheduling information, UE timers, etc |
| SIB 2 | General cell reselection information, intra-frequency cell reselection information |
| SIB 3 | Cell specific information for intra-frequency cell reselection |
| SIB 4 | Inter-frequency cell reselection information |
| SIB 5 | Inter-system cell reselection (e.g. to LTE) information |
| SIB 6 | ETWS primary notification |
| SIB 7 | ETWS secondary notification |
| SIB 8 | CMAS notification |
| SIB 9 | Timing information (UTC, GPS, …) |

MIB and SIB1 are known as Minimum System Information. MIB is periodically transmitted by gNB in PBCH. SIB1 is also periodically transmitted by gNB as it contains essential cell-specific information. Upon acquiring MIB and SIB1, the UE is allowed to camp on a cell.

The other SIBs are known as on-demand SI because gNB transmits these SIBs when UE explicitly request for it. The UE is not required to acquire these SIBs in order to camp on a cell. As shown in Table 1, each SIB has a dedicated purpose. For example, SIB 6 is about ETWS and SIB 8 only contain CMAS notification.

Observation 1 – The existing legacy SIBs have been introduced in which each SIB carries a specific purpose/content.

The legacy TRS configuration for connected mode UEs is configured via RRC messaging. The TRS configuration contains mandatory fields, such as:

- TCI states configuration where the QCL relation between TRS and SSB is provided.

- CSI-RS configuration where the resource allocation configuration of individual CSI-RS is provided.

- CSI-RS resource-set configuration where the assignment/allocation of CSI-RS as TRS is provided.

There are also optional parameters in the legacy TRS configuration. The details of parameters and the required bits for the above mandatory fields (and excluding optional parameters) are given in Appendix A.

We observed that the estimated minimum configuration size of TRS configuration for each spatial filter is around 27 bytes with the following break-down:

- TCI states: 		4 bytes

- CSI-RS config: 		20 bytes

- CSI-RS Resource set config: 3 bytes

Depending on the SSB configuration, a gNB with 64 SSBs requires a TRS configuration size of 1728 bytes. Furthermore, a gNB may support multiple TRS configurations as there can be many connected UEs. Each or some of the connected UEs may have its own TRS configuration. During idle/inactive mode, a UE can be configured with one of those multiple TRS configurations. We consider that the configuration of potential TRS/CSI-RS for idle/inactive mode is provided by dedicated RRC signaling. This is in a similar fashion to conveying the TRS for connected mode UEs in legacy NR. In legacy NR, when the UE switches to idle mode, the UE is expected to remove the connected mode configuration. Particularly in this case, we expect the UE to keep the TRS configuration so that the UE can use it while the UE is in idle/inactive mode. There is also another case where the UE in idle/inactive mode uses the new TRS configuration. The new TRS configuration may not the same as the TRS configuration used when the UE was in connected mode.

Observation 2; The UE may use the same or a different TRS configuration in Idle/Inactive mode as used in Connected mode.

Observation 3: TRS/CSI-RS configuration size with mandatory parameters is not small and it can have a size of up to 1728 bytes.

Considering the TRS/CSI-RS configuration is relatively large, and the SIB is designed for a certain purpose then we propose a new SIB that contain TRS/CSI-RS configuration. Furthermore, the legacy TRS/CSI-RS configuration is a UE-specific signaling. In principle, a cell can accommodate multiple TRS/CSI-RS for different connected mode UEs. The new SIB should also consider supporting multiple TRS/CSI-RS configuration.

Proposal 1 – Introduce a new SIB that contains TRS/CSI-RS configuration

The SIB for TRS/CSI-RS configuration is categorized as on-demand SI in which it will be sent to the UE once the gNB has received SI request. This is particularly useful for a UE which moves to a new cell. Once the UE enters a new cell and still operating in idle mode, the UE can request the system information carrying TRS/CSI-RS configuration.

There is also another scenario where a connected mode UE in a cell changes its state to idle/inactive mode. This particular UE has received TRS/CSI-RS configuration and uses it when the UE is in connected mode. It is mostly expected that that UE may re-use the same TRS/CSI-RS configuration in idle/inactive state. Hence, it may not be efficient if this particular UE needs to acquire SI containing an identical TRS/CSI-RS configuration for idle/inactive. A more compact information can be indicated to that UE or even just an indication to re-use the TRS configuration used when in Connected mode.

Observation 4 – A UE that moves from connected to idle/inactive mode can in principle re-use the TRS/CSI-RS that has been used during connected mode.

Proposal 2 – Introduce a compact TRS/CSI-RS configuration or indication for idle/inactive mode UE that has previously been in connected mode in the same cell.

# Conclusion

In this contribution, the following observations and proposals are made;

Observation 1 – The existing legacy SIBs have been introduced in which each SIB carries a specific purpose/content.

Observation 2; The UE may use the same or a different TRS configuration in Idle/Inactive mode as used in Connected mode.

Observation 3: TRS/CSI-RS configuration size with mandatory parameters is not small and it can have a size of up to 1728 bytes.

Observation 4 – A UE that moves from connected to idle/inactive mode can in principle re-use the TRS/CSI-RS that has been used during connected mode.

Proposal 1 – Introduce a new SIB that contains TRS/CSI-RS configuration

Proposal 2 – Introduce a compact TRS/CSI-RS configuration or indication for idle/inactive mode UE that has previously been in connected mode in the same cell.

# Reference

- RP-193239, New WID: UE Power Saving Enhancements.

- R1-2009848 - LS on signalling method for TRS/CSI-RS occasion(s) for idle/inactive UE(s)