3GPP TSG-RAN WG2 Meeting #113 electronic	                         R2-2100920

Online, January, 2021

Agenda item:	8.5.3

Source:	III

Title:	CG Harmonization for NR-U and IIoT/URLLC in Unlicensed Controlled Environments

Document for:	Discussion

- Introduction

In 3GPP Rel-17 NR IIoT/URLLC, “the uplink enhancements for URLLC in unlicensed controlled environments (UCE)” had been accepted to be the working item. The objects focus on [1]:

- Specify support for UE-initiated COT for FBE with minimum specification effort

- Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

To harmonize UL configured-grant enhancements for NR-U and IIoT/URLLC in unlicensed controlled environments, there are some issues need to clarify. In this contribution, we will share our views on “Configured grant autonomously re-transmit issue on URLLC in UCE” and “Uplink transmission mode switch for URLLC in UCE”.

- Discussion

  - Configured grant autonomously re-transmit issue on URLLC in UCE

In RAN2# 112-e meeting, the following issue on URLLC in UCE has been pointed out [2]:

“If a configured grant is deprioritized and/or gNB didn’t get it (e.g. LBT failure and/or tx failure) then we should be able to autonomously re-transmit it.  FFS how to achieve it (using existing mechanisms should be considered as baseline)”.

To solve this issue, we need to further consider “the cg-Retransmission timer is configured or not”. In the RAN2# 112-e meeting, the “cg-Retransmission timer can be configured optionally for shared spectrum” has been accepted. If the cg-Retransmission timer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection and HARQ processes sharing between multiple CGs are allowed. Otherwise, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection and HARQ processes sharing between multiple CGs are not allowed.

With this condition, we indicate the following cases to figure out the issue:

- The cg-retransmission timer is configured

  - The configured grant conflict is happening and gNB get it

The higher priority configured grant will transmit successfully at this current time, as for the deprioritized configured grant, we recommend it use the legacy autonomousTx mechanism.

  - The configured grant conflict is not happening but LBT failure and/or tx failure is happening

The cg-retransmission timer is configured, used the legacy autonomousReTx is our recommendation.

  - The configured grant conflict is happening and gNB didn’t get it

In this situation, the high priority configured grant will transmit failure at the current time. Thus, we recommend the high priority configured grant using the legacy autonomousReTx mechanism. As for the deprioritized configured grant, we recommend it using the legacy autonomousTx mechanism.

|  | LBT success | LBT/Tx failure |
| --- | --- | --- |
| Configured grant conflict is not happened |  | autonomousReTx |
| Configured grant conflict is happened | autonomousTx | High priority: autonomousReTx Low priority: autonomousTx |

Observation#1: When the cg-retransmission timer is configured, for the “configured grant conflict is happening and gNB didn’t get it” issue, the high priority configured grant can use the legacy autonomousReTx mechanism, the deprioritized configured grant can use the legacy autonomousTx mechanism.

- The cg-retransmission timer is not configured

  - The configured grant conflict is happened and gNB get it

The higher priority configured grant will transmit successfully at the current time, as for the deprioritized configured grant, we recommend it using the legacy autonomousTx mechanism.

  - The configured grant conflict is not happening but LBT failure and/or tx failure is happening

The cg-retransmission timer is not configured, the HARQ processes sharing between multiple CGs are not allowed, the autonomousReTx cannot be used in this situation. Therefore, we think this LBT failure and/or tx failure configured grant can use the next resource with the same configured grant configuration to transmit same as legacy autonomousTx mechanism.

  - The configured grant conflict is happening and gNB didn’t get it

When the cg-retransmission timer is not configured, the HARQ processes sharing between multiple CGs are not allowed, if the high priority configured grant using the next resource with the same configured grant configuration to retransmit, and the low priority configured grant also using the legacy autonomousTx mechanism, then they will collision keep happening. For this condition, a new mechanism to solve this issue is necessary.

|  | LBT success | LBT/Tx failure |
| --- | --- | --- |
| Configured grant conflict is not happened |  | Use the next resource with the same CG configuration to transmit |
| Configured grant conflict is happened | autonomousTx | New mechanism is necessary |

Observation#2: When the cg-retransmission timer is not configured, the “configured grant conflict is happening and gNB didn’t get it” issue, a new mechanism for solving the “configured grant conflict is happening and gNB didn’t get it” issue is necessary

  - Transmission mode switch for URLLC in UCE

In previous RAN2# meeting, it is decided when the cg-retransmission timer is configured, NR-U CG mode is in used, when the cg-retransmission timer is not configured, URLLC CG mode is in used.

In Rel-16, the URLLC CG mode is used in licensed spectrum to solve the latency issue, the NR-U CG mode is used in unlicensed spectrum to solve the reliability issue. However, in UCE, the LBT/Tx failure may happened at any moment (e.g. unexpected interference), when the current communication quality become worse, the URLLC CG mode could not be used, the transmission fails will let the resource utilization become lower. Moreover, when the current communication quality is good, the NR-U CG mode’s feedback mechanism will cause the resource redundancy and let the resource utilization become lower.

Thus, the uplink transmission mode switching between NR-U and URLLC in unlicensed controlled environments should be considered.

Proposal#2: Consider the uplink transmission mode switching between NR-U and URLLC in unlicensed controlled environments

- Conclusion

For URLLC/IIoT in unlicensed controlled environments, there are some discussed in this contribution with the following observation made and proposal proposed:

Observation#1: When the cg-retransmission timer is configured, for the “configured grant conflict is happening and gNB didn’t get it” issue, the high priority configured grant can use the legacy autonomousReTx mechanism, the deprioritized configured grant can use the legacy autonomousTx mechanism.

Observation#2: When the cg-retransmission timer is not configured, the “configured grant conflict is happening and gNB didn’t get it” issue, a new mechanism for solving the “configured grant conflict is happening and gNB didn’t get it” issue is necessary

Proposal#1: Consider the uplink transmission mode switching between NR-U and URLLC in unlicensed controlled environments.

References

[1] RP-201310, Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NRWID, 3GPP TSG RAN Meeting #88e, Electronic meeting, June 29 – July 3, 2020;

[2] R2-2100001, Report of 3GPP TSG RAN WG2 meeting #112-e Online, 3GPP TSG RAN WG2 Meeting #112e, Electronic meeting, 2 - 13 November, 2020