3GPP TSG-RAN2 Meeting #113-e	R2-2100965

# Online, 25th Jan 2021 - 5th Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4556 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on paging narrowband selection-Option 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2021-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the current specification, UE supporting GWUS would selects narrowbands only among the paging narrowbands that are configured with GWUS, and GWUS are not supported for eMTC UEs in RRC_INACTIVE. But for eMTC UE in RRC_INACTIVE, when eNB receives CN initiated paging for an eMTC UE, it cannot differentiate whether the UE is in RRC_IDLE or RRC_INACTIVE state. Therefore, there will exist ambiguity for eNB’s selection on paging narrowbands, e.g., eNB either assume that the UE is in RRC_IDLE or assume that the UE is in RRC_INACTIVE and GWUS will not be used. Accordingly, eMTC UE in RRC_INACTIVE needs to monitor two narrowbands for paging This is obviously undesired. To solve this issue, it has been agreed that eMTC UE always selects paging narrowbands provided in system information, regardless whether GWUS are configured or not. So the groupNarrowBandList IE will not be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that the groupNarrowBandList IE should not be configured. Impact Analysis Impacted functionality: The changes only impacts GWUS use for eMTC UEs. Inter-operability: If the UE is implementing according to this CR and the network is not, or vise versa, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eMTC UE supporting GWUS may select a paging narrowband not configured with GWUS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.304 CR0819 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

6.3.2	Radio resource control information elements

//SKIP THE UNRELATED PART//

#### –	GWUS-Config

The IE GWUS-Config is used to specify the Group WUS configuration. For the UEs supporting GWUS, E-UTRAN uses GWUS to indicate that the UE shall attempt to receive paging in that cell, see TS 36.304 [4].

GWUS-Config information element

-- ASN1START

GWUS-Config-r16 ::=				SEQUENCE {

groupAlternation-r16			ENUMERATED {true}				OPTIONAL,	-- Need OR

commonSequence-r16				ENUMERATED {g0, g126}			OPTIONAL,	-- Need OR

timeParameters-r16				GWUS-TimeParameters-r16			OPTIONAL,	-- Cond NoWUSr15

resourceConfigDRX-r16			GWUS-ResourceConfig-r16,

resourceConfig-eDRX-Short-r16	GWUS-ResourceConfig-r16			OPTIONAL,	-- Need OP

resourceConfig-eDRX-Long-r16	GWUS-ResourceConfig-r16			OPTIONAL,	-- Cond TimeOffset

probThreshList-r16				GWUS-ProbThreshList-r16		OPTIONAL, 	-- Cond ProbabilityBased

groupNarrowBandList-r16			GWUS-GroupNarrowBandList-r16	OPTIONAL -- Need OR

}

GWUS-TimeParameters-r16 ::=		SEQUENCE {

maxDurationFactor-r16			ENUMERATED {one32th, one16th, one8th, one4th},

numPOs-r16						ENUMERATED {n1, n2, n4, spare1}			DEFAULT n1,

timeOffsetDRX-r16				ENUMERATED {ms40, ms80, ms160, ms240},

timeOffset-eDRX-Short-r16		ENUMERATED {ms40, ms80, ms160, ms240},

timeOffset-eDRX-Long-r16		ENUMERATED {ms1000, ms2000}				OPTIONAL,	-- Need OP

numDRX-CyclesRelaxed-r16		ENUMERATED {n1, n2, n4, n8}				OPTIONAL,	-- Need OR

powerBoost-r16					ENUMERATED {dB0, dB1dot8, dB3, dB4dot8}	OPTIONAL,	-- Need OR

...

}

GWUS-ResourceConfig-r16 ::=		SEQUENCE {

resourceMappingPattern-r16		CHOICE {

resourceLocationWithWUS			ENUMERATED {primary, secondary, primary3FDM},

resourceLocationWithoutWUS		ENUMERATED {n0, n2}

},

numGroupsList-r16				GWUS-NumGroupsList-r16			OPTIONAL,	-- Need OP

groupsForServiceList-r16		GWUS-GroupsForServiceList-r16	OPTIONAL	-- Cond ProbabilityBased

}

GWUS-GroupsForServiceList-r16 ::=	SEQUENCE (SIZE (1..maxGWUS-ProbThresholds-r16)) OF INTEGER (1..maxGWUS-Groups-1-r16)

GWUS-GroupNarrowBandList-r16 ::=	SEQUENCE (SIZE (1..maxAvailNarrowBands-r13)) OF BOOLEAN

GWUS-NumGroupsList-r16 ::= 		SEQUENCE (SIZE (1..maxGWUS-Resources-r16)) OF GWUS-NumGroups-r16

GWUS-ProbThreshList-r16 ::=		SEQUENCE (SIZE (1..maxGWUS-ProbThresholds-r16)) OF GWUS-PagingProbThresh-r16

GWUS-NumGroups-r16 ::=			ENUMERATED {n1, n2, n4, n8}

GWUS-PagingProbThresh-r16 ::=	ENUMERATED {p20, p30, p40, p50, p60, p70, p80, p90}

-- ASN1STOP

GWUS-Config field descriptions

commonSequence Presence of the field indicates common WUS sequence is configured. Value g0 indicates common WUS sequence for the shared WUS resource corresponds to g = 0, and value g126 indicates common WUS sequence for the shared WUS resource corresponds to g = 126, see TS 36.211 [21].

groupAlternation Presence of the field enables WUS group alternation between the two or more WUS resources for the gap type, see TS 36.304 [4].

groupNarrowBandList List indicating which paging narrowbands support group WUS see TS 36.304 [4]. First entry in the list indicates WUS support for first paging narrowband, second entry in the list indicates WUS support for second paging narrowband, and so on. If E-UTRAN includes groupNarrowBandList, the number of entries is equal to the value of paging-narrowBands. If this list is absent, group WUS is supported on all paging narrowbands. In this version, this field should not be configured.

groupsForServiceList Number of WUS groups for each paging probability group see TS 36.304 [4]. The first entry corresponds to the first probability group, the second entry corresponds to the second paging probability group, and so on. Total number of WUS groups in this list cannot be more than the total number of WUS groups in numGroupsList. If E-UTRAN includes groupsForServiceList, it includes the same number of entries and listed in the same order as in probThreshList.

numGroupsList List of WUS groups for each WUS resource see TS 36.304 [4]. First entry corresponds to the first resource, second entry corresponds to the second resource, and so on. numGroupsList is mandatory present in resourceConfigDRX. If numGroupsList is not present in resourceConfig-eDRX-Short, parameter for DRX WUS resource applies for short eDRX WUS resource. If numGroupsList is not present in resourceConfig-eDRX-Long, parameter for short eDRX WUS resource applies for long eDRX WUS resource.

probThreshList Paging probability thresholds corresponding to the paging probability groups, see TS 36.304 [4]. Value p20 corresponds to 20%, value p30 corresponds to 30%, and so on.

resourceConfigDRX, resourceConfig-eDRX-Short, resourceConfig-eDRX-Long WUS resource configured for each gap type see TS 36.304 [4]. If resourceConfig-eDRX-Short is not present, DRX WUS parameters apply for short eDRX WUS resource. If resourceConfig-eDRX-Long is not present, short eDRX WUS parameters apply for long eDRX WUS resource.

resourceMappingPattern Identifies the WUS resource mapping to time/frequency as defined in TS 36.304 [4]. If wus-Config-r15 is present in SystemInformationBlockType2, the field is set to value resourceLocationWithWUS; otherwise the field is set to value resourceLocationWithoutWUS.

timeParameters Time domain WUS configuration information. For individual field descriptions, see WUS-Config. If the field is absent, the parameters in wus-Config apply.

| Conditional presence | Explanation |
| --- | --- |
| NoWUSr15 | The field is mandatory present if wus-Config-r15 is not present in SystemInformationBlockType2; otherwise the field is not present. |
| ProbabilityBased | The field is mandatory present if paging probability based WUS group selection is configured; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TimeOffset | The field is optionally present, Need OP, if timeOffset-eDRX-Long is present in timeParameters; otherwise the field is not present, and the UE shall delete any existing value for this field. |

<End of the change>