3GPP TSG-RAN WG2 #113e	R2-2100970

Electronic meeting, 25th January – 5th February 2021

Agenda Item:	5.4.3

Source:	Ericsson

Title:	Dummify the capability bit v2x-EUTRA

Document for:	Discussion, Decision

# 1	Introduction

In the last RAN2#112e meeting, there was a discussion on how to handle the capability v2x-EUTRA that was introduced in Rel-15, but that it was agreed to not be used for NR sidelink when this was specified during Rel-16.

R2-2009832	UE capability for EUTRA V2X in DC	vivo	discussion

R2-2010336	Correction on the capability bit v2x-EUTRA of option-1	vivo	CR	Rel-15	38.306	15.11.0	0444	-	F	NR_newRAT-Core

R2-2010337	Correction on the capability bit v2x-EUTRA of option-2	vivo	CR	Rel-15	38.306	15.11.0	0445	-	F	NR_newRAT-Core

R2-2010338	Correction on the capability bit v2x-EUTRA of option-3	vivo	CR	Rel-15	38.331	15.11.0	2206	-	F	NR_newRAT-Core

[OPPO, Ericsson, CATT, Huawei]: Do not think option1 and option2 can solve the issue, but we’re also ok to leave it as it is. [Ericsson]: If we want to change this bit, it needs to take some offline checking with capability guys since it was decided in the main session long time ago.

=> Noted

In this contribution, we address this open issue by providing some motivation on whether this capability should be dummied and not used anymore.

# 2	Discussion

In the RAN2#110e and RAN2#111e meetings, it was discussed whether it was necessary to introduce UE capabilities parameters to indicate whether UE supports the PC5 BC for the MR-DC scenarios. The outcome has been that no capability signalling will be introduced for this case in Rel-16 (since also RAN4 did not agree to support this use case i.e., SL configured with MR-DC).

From RAN2#110e

RAN2 can attempt to introduce signaling for PC5 BC for (NG)EN/NE-DC scenario in CR implementation. If not so complicated, we introduce the signaling but otherwise we do not introduce it. Note with introduction of signaling, it does not mean RAN4 should introduce the corresponding BC now.

From RAN2#111e

A parameter to indicate whether UE supports PC5 BC when NR Uu BC is configured as NR-DC is not supported in this release.

The agreements taken basically means that the UE should not report any sidelink capabilities when is configured with MR-DC. According to this, the capability v2x-EUTRA introduced during Rel-15 in the context of EN-DC it becomes meaningless.

| v2x-EUTRA Indicates whether the UE supports EUTRA V2X according to UE-EUTRA-Capabilityas defined in TS 36.331 [5], independent of the configured EN-DC band combination. This field is only applied to EN-DC. In UE-NR-Capability, this field is not used, and UE does not include the field. | UE | No | Yes | No |
| --- | --- | --- | --- | --- |

RAN2 agreed in Rel-16 that the UE does not report any PC5 capability when this is configured with MR-DC.

According to the RAN2 agreements in Rel-16, the capability bit v2x-EUTRA introduced in Rel-15 has not meaning and is not used.

According to this, and also taking into account that RAN4 do not intend to support sidelink with MR-DC not even in Rel-17, a straightforward and a clean solution would be to dummy the capability bit v2x-EUTRA in TS 38.331 and TS 38.306 since this is not used in Rel-15 and Rel-16 and will not be used also in Rel-17. Thus we propose:

RAN2 to dummy the capability bit v2x-EUTRA in TS 38.331 and TS 38.306.

RAN2 to agree on the CRs in [1] and [2].

# Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 agreed in Rel-16 that the UE does not report any PC5 capability when this is configured with MR-DC.

Observation 2	According to the RAN2 agreements in Rel-16, the capability bit v2x-EUTRA introduced in Rel-15 has not meaning and is not used.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to dummy the capability bit v2x-EUTRA in TS 38.331 and TS 38.306.

Proposal 2	RAN2 to agree on the CRs in [1] and [2].

# References

R2-2100971, Dummy the capability bit v2x-EUTRA, Ericsson, RAN2#113e, January 2021

R2-2100972, Dummy the capability bit v2x-EUTRA, Ericsson, RAN2#113e, January 2021