3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2101127

Online, Jan 25 – Feb 5, 2021

Agenda item:		8.10.3.2

Source:		Lenovo, Motorola Mobility

Title:		Ephemeris provision for satellite and HAP constellation

Document for:		Discussion

- Introduction

The previous RAN2 meeting made the following agreements regarding ephemeris for NTN:

- Satellite/HAPS ephemeris based cell selection and reselection should be defined for NTN (FFS what the term satellite/HAPS ephemeris actually means). FFS when this ephemeris based cell selection / reselection can be used. FFS whether UE location (and/or other information) based cell selection and reselection should be introduced for NTN

- The satellite ephemeris should be provided to UE, at least for Satellite/HAPS ephemeris based cell selection and reselection (FFS what the term satellite/HAPS ephemeris actually means).

In this contribution we further discuss ephemeris provision considering satellite/HAP constellation (i.e. a group of satellites or HAPs).

- Discussion

The ephemeris data is used to indicate the trajectories and/or position coordinates of satellites or HAPs, so that the UE can perform TA pre-compensation, location/time-based handover and cell reselection. In the previous discussions for NTN, two typical formats of ephemeris were considered:

- Option A) Orbital parameters including orbital plane and satellite level parameters;

- Option B) ECEF coordinates to represent the position (x, y, z), time, velocity, etc.

The discussion for ephemeris format(s) is in the email discussion [Post112-e][153][NTN] and both options have pros and cons. The following discussion give further view from the constellation perspective regardless of which format(s) is finally accepted.

A typical form of Option A) as recommended in TR38.821 includes 5 orbital plane parameters and 2 satellite level parameters (Table 1), which needs to be represented by at least 7 double-precision floating point numbers plus some overhead (7*8=56Byte for a single satellite). With increased accuracy and applicability to HAPs, Option B) requires larger data size and more frequent update.

Table 1: Essential Elements of Ephemeris

| Orbital plane parameters | a | Square root of semi major axis（semi-major axis） |
| --- | --- | --- |
|  | e | Eccentricity（eccentricity） |
|  | i0 | Inclination angle at reference time（inclination） |
|  | Ω0 | Longitude of ascending node of orbit plane（right ascension of the ascending node） |
|  | ω | Argument of perigee（argument of periapsis） |
| Satellite level parameters | M0 | Mean anomaly at reference time（true anomaly and a reference point in time） |
|  | t0e | Ephemeris reference time（the epoch） |

Currently from RAN2 perspective, ephemeris is mainly used for:

- Timing relation: including TA estimation and pre-compensation for random access, and offsetting or extension for UP timers and windows;

- Mobility management: including cell selection/reselection in IDLE/INACTIVE and measurements/handover in CONNECTED.

For timing relation at least the ephemeris of the serving satellite/HAP is needed for the UE to estimate the TA, while for mobility management the ephemeris of neighbouring satellite/HAP is more important. E.g. a UE in IDLE/INACTIVE needs to know when and/or where the next cell will be available for location/time-based (which may be more accurate than RSRP/RSRQ-based) neighbouring cell measurement or cell reselection, and a UE in CONNECTED also needs such information for location/time-based (which may be more accurate than RSRP-based) measurement report triggering or conditional handover execution. In this case the ephemeris provision for satellite/HAP constellation (i.e. a group of satellites or HAPs) has to be considered.

Considering that both options require 56Byte or larger data size for one satellite/HAP, it is obvious that the scale of ephemeris for satellite/HAP constellation will lead to more overhead for broadcasting or signalling, as well as more UE power consumption for reception and decoding. Considering that satellites/HAPs belonging to the same operator typically share a common orbital plane or altitude, these ephemeris elements as the common part can be provided together for a satellite/HAP constellation.

Observation 1: At least for mobility management, the common part of ephemeris (e.g. orbit plane parameters for satellites or altitude for HAPs) can be provisioned together for a satellite/HAP constellation to reduce overhead and power consumption.

Proposal 1: For a satellite/HAP constellation, ephemeris provisioning of the common part should be supported.

Except for the shared orbit or altitude, another character of NTN is that satellites/HAPs typically operate with organized and fixed intervals of distance, time or angle. So it is sufficient to provision the ephemeris data of one satellite/HAP and the relativity of others instead of provisioning full ephemeris for all. For instance, a serving satellite can broadcast its own ephemeris data (e.g. M0_1, t0e_1M0_1M0_1), and the difference values of ephemeris data of its neighbour satellite(s) (e.g. ΔM0_1_2=M0_2-M0_1, Δt0e_1_2=t0e_2-t0e_1) which is of smaller size than the ephemeris data of its neighbour satellite(s) (e.g. M0_2, t0e_2). These ephemeris elements as the non-common part can be provided with compressed size for a satellite/HAP constellation.

Observation 2: The non-common part of the ephemeris data (e.g. the satellite level parameters for satellites or horizontal plane coordinates for HAPs) for a satellite/HAP constellation can be further compressed considering their relevance.

Proposal 2: For a satellite/HAP constellation, ephemeris provisioning of the non-common part in a relative manner (i.e. the relativity of ephemeris between satellites/HAPs) should be supported.

Furthermore, it is unnecessary for a UE to receive and decode the ephemeris of a satellite/HAP constellation every time it accesses one satellite/HAP within it, unless the ephemeris is updated. It means that a UE can skip obtaining the same ephemeris of a satellite/HAP constellation if already available. This can be easily implemented by making the UE aware of the constellation (e.g. a value tag). And the value tag can also be changed upon ephemeris update to indicate UE for updating.

Observation 3: It is unnecessary for a UE to obtain the same ephemeris of a satellite/HAP constellation if not updated.

Proposal 3: The UE can skip obtaining or decoding the same ephemeris of a satellite/HAP constellation.

- Conclusion

In this contribution, ephemeris provisioning for satellite/HAP constellation and possible enhancements are discussed. The following observation are given:

Observation 1: At least for mobility management, the common part of ephemeris (e.g. orbit plane parameters for satellites or altitude for HAPs) can be provisioned together for a satellite/HAP constellation to reduce overhead and power consumption.

Observation 2: The non-common part of the ephemeris data (e.g. the satellite level parameters for satellites or horizontal plane coordinates for HAPs) for a satellite/HAP constellation can be further compressed considering their relevance.

Observation 3: It is unnecessary for a UE to obtain the same ephemeris of a satellite/HAP constellation if not updated.

And we propose:

Proposal 1: For a satellite/HAP constellation, ephemeris provisioning of the common part should be supported.

Proposal 2: For a satellite/HAP constellation, ephemeris provisioning of the non-common part in a relative manner (i.e. the relativity of ephemeris between satellites/HAPs) should be supported.

Proposal 3: The UE can skip obtaining or decoding the same ephemeris of a satellite/HAP constellation.