3GPP TSG-RAN WG2 Meeting #113-e	R2-2101208

E-meeting, January 25 – February 5, 2021

Agenda Item:	8.4.3

Source: 	Canon Research Centre France

Title:	Discussion on RLF indication enhancement and local routing for R17-IAB

Document for:	Discussion

# Introduction

In the previous RAN2#112e meeting [1], the following agreements have been made:

Consider enhancements to topology adaptation that improve:

- Robustness, e.g., to rapid shadowing,

- service-interruption,

- load balancing among different IAB-nodes, IAB-donor-DUs and IAB-donor-CUs, and

- reduction in signaling load.

RAN2 to discuss enhancements to RLF indication/handling with the focus on the reduction of service interruption after BH RLF.

RAN2 to discuss local rerouting, including the benefits over central route determination, and on how topology-wide objectives can be addressed.

In this paper, we discuss how to enhance the RLF indication content and how to propagate it along the wireless backhaul. The goal is to allow an IAB-node to take the appropriate decision for local re-routing, in accordance with its BAP configuration established by the IAB-donor-CU, in order to reduce service interruption due to the RLF.

# Discussion

## Enhancement of RLF indication content

According to Rel-16, when experiencing BH RLF, an IAB-node may select another BH link based on routing entries with the same destination BAP address, i.e., by disregarding the BAP path ID. Also, in case the IAB-MT detects a BH RLF recovery failure for a BH link with a parent node, and the IAB-node is no longer able to provide BH connection to the IAB-donor, then the IAB-DU sends a BH RLF indication to its child IAB-node(s). The agreed type of RLF indication among the 5 types identified during the Rel-16 discussions corresponds to Type 4 (and to some extent also to Type 4x).

As proposed by several companies in [3], other types of RLF indication may be introduced in Rel-17: Type 1/Type 2 to indicate a BH RLF has occurred but is not yet recovered, and Type 3 to indicate BH RLF recovery. Receiving RLF indication Type 1, 2, or 4 at a child IAB-node can trigger the rerouting of packets to be transmitted in the upstream direction, assuming the child IAB-node has an alternative path configured by the Donor-CU. The benefit is to reduce service interruption due to the RLF. Then, receiving RLF indication Type 3 would enable a return to the initial routing.

Observation 1: In Rel-16 specifications, the control BAP PDU for RLF indication conveys the D/C bit and the 4-bit PDU type as well as 3 empty bits reserved for future use.

There may be some cases where an IAB-node with dual-connectivity uses both links for load balancing purpose. Indeed, if the capacity of one link is not sufficient to accommodate the total throughput, then some packets associated with one or several BAP path ID(s) would be transmitted through the first parent, while other packets associated with different BAP path ID(s) would be transmitted through the second parent.

If a BH RLF occurs for one of the parent links, the IAB-node may try to reroute the packets through the alternative link. In this respect, sending a BH RLF indication to the child IAB-node(s) would be useful to trigger some rerouting by the child IAB-node(s).

However, upon reception of such RLF indication from a parent IAB-node, a child IAB-node may try to reroute all the packets initially intended for this parent IAB-node through an alternative BH link, which may create a risk of congestion on this alternative link.

For this purpose, in order to help a child IAB-node to take the right decision for rerouting, it is proposed that the BH RLF indication format is enhanced so that it carries a list of the BAP path ID(s) impacted by the RLF. A child IAB-node receiving this enhanced RLF indication (Type 1, 2, or 4) may thus be able to reroute only the packets associated with the BAP path ID belonging to the received list.

Proposal 1: A BH RLF indication may convey a list of BAP path ID(s) or BAP Routing ID(s) impacted by the RLF.

Given the example of Figure 1 below illustrating a configuration of upstream BAP paths for 2 IAB-nodes (7 and 8), if a BH RLF occurs between IAB-node 4 and IAB-node 1, the IAB-node 4 may send a BH RLF indication including the list [path ID1, path ID4] to its child IAB-nodes 7 and 8. As a consequence, IAB-node 8 may reroute packets associated with path ID4 through a different IAB-node, i.e. IAB-node 5. In case of RLF recovery, IAB-node 4 may send a BH RLF indication Type 3 including the list [path ID1, path ID4] to indicate those paths are available again.

Figure 1 : Upstream paths configuration example

From the above example, it can be noticed that IAB-node 8 has three configured paths but has no information on which of these three paths is the best for rerouting.

Observation 2: An IAB-node may not take the best decision for local rerouting because of a lack of knowledge on the actual quality of its several alternative transmission paths.

It is then proposed that an IAB-node may send some information on the quality level of each BAP path in its BH Routing Configuration to its child IAB-node(s). The quality level may be related to the radio link conditions (e.g. SINR) and/or to the congestion status of the link towards the parent IAB-node. The quality level information may be sent only when there is a significant change for one path.

The quality level for several paths may be carried by a BH RLF indication message (Type 1, 2, 3, or 4). It may also be transmitted through another type of BH RLF indication message (e.g. Type 5: BH Path Quality level indication) or through a new type of indication message.

Proposal 2a: Some information related to the quality level associated with all or part of the BAP paths or the BAP Routing IDs handled by an IAB-node may be transmitted to the child IAB-node(s) through a BH RLF indication message.

Proposal 2b: The quality level associated with a BAP path or a BAP Routing ID may refer to the radio link conditions (e.g. SINR) and/or the congestion level of an associated BH link.

Proposal 2c: Information related to the quality level associated with all or part of the BAP paths or the BAP Routing IDs handled by an IAB-node may be transmitted to the child IAB-node(s) through a new type of BH RLF indication, e.g. Type 5, or through a dedicated BAP control PDU, e.g. Radio Link Quality (RLQ) indication.

With the example of Figure 1, IAB-node 4 may transmit a BH RLF indication including the quality level of paths ID1, ID2, ID4, and ID5 to IAB-node 7 and IAB-node 8. The quality level of paths ID2 and ID5 depends on the status of the BH link between IAB-node 4 and IAB-node 3. The quality level of paths ID1 and ID4 depends on the status of the BH link between IAB-node 4 and IAB-node 1. Assuming a BH RLF impacting paths ID1 and 4, IAB-node 8 may compare the quality level of paths ID4, ID5, and ID3 (which it may have previously received from IAB-node 5) and decide to reroute the packets associated with path ID4 to the best path between paths ID5 and ID3, i.e. the path having the highest quality level.

## Downstream forwarding of RLF indication

When the IAB-node receiving a BH RLF indication (Type 1, 2 or 4) does not have any alternative path, there is no possibility for the IAB-node to perform any rerouting in the upstream direction. In such a case, the IAB-node should forward the RLF indication to its own child IAB-node(s) which may have dual connectivity for local rerouting. This proposal was already expressed in [4].

Proposal 3a: Upon reception of a BH RLF indication from a parent IAB-node, an IAB node without any alternative path should forward the RLF indication to its own child IAB node(s).

Going further, it may be advantageous that a BH RLF indication is forwarded to the child IAB-node that generated the packets to reroute (e.g. an access IAB-node for a UE). Indeed, such an IAB-node should have the largest number of possible alternative paths to send its packets (provided those paths have been configured by the IAB-donor-CU). Moreover, selecting a new path ID from the IAB-node generating the packets would avoid any additional processing at intermediate relay IAB-nodes (e.g. on-the-fly rerouting). However, the IAB-node needs some information on the quality level of each path in order to select the best one.

Proposal 3b: A BH RLF indication including BAP path ID(s) or BAP Routing ID(s) (with or without the quality level) should be forwarded by an IAB-node to its child IAB node(s).

## Upstream forwarding of RLF indication

Up to now, we have considered the downstream transmission and forwarding of RLF indication with the purpose of rerouting in a descendant IAB-node. This is useful to reduce service interruption of upstream traffic waiting for a BAP configuration update from the IAB-donor-CU. However, as RLF indication is not transmitted to a parent IAB-node, packets rerouting for downstream traffic may not be triggered, with a risk of congestion and then service interruption, before a new BAP configuration is applied by the IAB-donor-CU.

Figure 2 : Downstream paths configuration example

With the example of Figure 2 above illustrating a configuration of downstream BAP paths for IAB-node 8, if a BH RLF occurs between IAB-node 4 and IAB-node 8, the IAB-node 1 is not aware of this RLF even though it could reroute packets through IAB-node 5 and path ID6.

Observation 3: Packet rerouting in downstream direction following a BH RLF may require a BAP configuration update from the IAB-donor-CU.

Then, it is proposed that an IAB-node receiving a BH RLF indication including the quality level of BAP Routing ID(s) forwards this RLF indication in the upstream direction when possible. The objective is to reach a parent IAB-node capable of re-routing data packets, and ultimately to reach the IAB-donor, firstly to reroute packets, and secondly to update the BAP configuration of IAB-nodes if necessary. In this respect, an IAB-node receiving the BH RLF indication checks its BAP routing tables to identify the next hop(s), i.e. the egress link(s), associated with the Routing ID having the IAB-donor address as destination. The RLF indication is then forwarded over the identified egress link(s) but different from the link on which the RLF indication was received.

Proposal 4: A BH RLF indication including BAP Routing ID(s) (with or without the quality level) should be forwarded by an IAB-node through the egress link(s) associated with the destination BAP address of the IAB-donor, except for the egress link on which the RLF indication was received.

## RLF indication update

When forwarding a BH RLF indication including the quality level associated with BAP path(s) or BAP Routing ID(s), an IAB-node may update the quality level according to its own estimation. If the quality level estimated by the IAB-node for a given BAP path or BAP Routing ID is lower than the quality level indicated in the received RLF indication, then the IAB-node may update this quality level when forwarding the RLF indication. If the quality level estimated by the IAB-node for a given BAP path is better than or equal to the quality level indicated in the received RLF indication, then the IAB-node does not update the RLF indication.

Proposal 5: When forwarding a BH RLF indication including the quality level associated with BAP path(s) or BAP Routing ID(s), an IAB-node may update the quality level associated with a BAP path or a BAP Routing ID, which is estimated as lower than the quality level indicated in the received RLF indication.

## Routing ID update

According to Release 16, when rerouting is decided and applied by an IAB-node, the rerouted packets will take a path that does not correspond to the Routing ID in their BAP header (PATH field). The IAB-node receiving the rerouted packets may not have the corresponding Routing ID as an entry in its routing table. Although the IAB-node can route the packets according to the destination BAP address only, it is preferable, for an optimized routing, that the Routing ID of the packets matches with the path that is actually taken. For this purpose, the IAB-node that performs rerouting should update the BAP path ID in the header of the rerouted packets.

In particular, in order to avoid any Routing ID update at intermediate IAB-nodes, an access IAB-node (for upstream traffic) or the IAB-donor-DU (for downstream traffic) should update the BAP path ID in the header of the rerouted packets.

Proposal 6: An IAB-node performing local rerouting should update the BAP path ID in the header of the rerouted packets to reflect the new path taken by the packets.

# Conclusion

In this contribution, we discuss possible enhancements about RLF indication to not only notify a BH RLF but also to transmit information related to the quality level of BAP paths. Such information should be propagated to the IAB-nodes capable of rerouting.

Observation 1: In Rel-16 specifications, the control BAP PDU for RLF indication conveys the D/C bit and the 4-bit PDU type as well as 3 empty bits reserved for future use.

Observation 2: An IAB-node may not take the best decision for local rerouting because of a lack of knowledge on the actual quality of its several alternative transmission paths.

Observation 3: Packet rerouting in downstream direction following a BH RLF may require a BAP configuration update from the IAB-donor-CU.

Proposal 1: A BH RLF indication may convey a list of BAP path ID(s) or BAP Routing ID(s) impacted by the RLF.

Proposal 2a: Some information related to the quality level associated with all or part of the BAP paths or the BAP Routing IDs handled by an IAB-node may be transmitted to the child IAB-node(s) through a BH RLF indication message.

Proposal 2b: The quality level associated with a BAP path or a BAP Routing ID may refer to the radio link conditions (e.g. SINR) and/or the congestion level of an associated BH link.

Proposal 2c: Information related to the quality level associated with all or part of the BAP paths or the BAP Routing IDs handled by an IAB-node may be transmitted to the child IAB-node(s) through a new type of BH RLF indication, e.g. Type 5, or through a dedicated BAP control PDU, e.g. Radio Link Quality (RLQ) indication.

Proposal 3a: Upon reception of a BH RLF indication from a parent IAB-node, an IAB node without any alternative path should forward the RLF indication to its own child IAB node(s).

Proposal 3b: A BH RLF indication including BAP path ID(s) or BAP Routing ID(s) (with or without the quality level) should be forwarded by an IAB-node to its child IAB node(s).

Proposal 4: A BH RLF indication including BAP path ID(s) (with or without the quality level) should be forwarded by an IAB-node through the egress link(s) associated with the destination BAP address of the IAB-donor, except for the egress link on which the RLF indication was received.

Proposal 5: When forwarding a BH RLF indication including the quality level associated with BAP path(s) or BAP Routing ID(s), an IAB-node may update the quality level associated with a BAP path or a BAP Routing ID, which is estimated as lower than the quality level indicated in the received RLF indication.

Proposal 6: An IAB-node performing local rerouting should update the BAP path ID in the header of the rerouted packets to reflect the new path taken by the packets.

# Reference

[1] Chairman notes of RAN2#112e meeting.

[2] RP-201293, New WID on Enhancements to Integrated Access and Backhaul. Qualcomm.

[3] R2-2009292, [Post111-e][903][eIAB] Topology adaptation enhancements RAN2 scope. Qualcomm.

[4] R2-2004593, Some left over issues on IAB BH RLF handling. NEC