3GPP TSG-RAN WG2 Meeting #113-e	R2-2101227

Electronic, 25nd Jan– 05th Feb, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on positioning latency

Agenda Item:	8.11.2.1

Document for:	Discussion and decision

Introduction

In the last RAN2 meeting [1], the enhancement on NR positioning was discussed. During the meeting, the following agreements have been made:

Agreements:

1: For latency analysis of Rel.16 solutions, RAN2 only consider the latency of positioning procedure, i.e. step 5 in MO-LR/step 12 in MT-LR (involving RRC, LPP, NRPPa, MAC).  A note is added to the TR and in our response to RAN1 to clarify this is what we covered.  We can clarify to RAN1 that more time would be needed for an end-to-end analysis.

2: For latency analysis of Rel.16 solutions, RAN2 only consider the latency caused by UE, gNB, AMF and LMF.

3: For latency analysis of Rel.16 solutions, RAN2 consider both UE-based and UE-assisted.

There was another email thread to finalize the TP on latency evaluation.

[Post112-e][616][POS] TP for latency analysis results (Intel)

Scope: Capture the latency analysis results in a TP, taking into account any input from RAN1/RAN3/SA2.

Intended outcome: Endorsable TP

Deadline:  Long

In this paper, we will further discuss the positioning latency issues from RAN2 point of view.

Latency analysis for typical scenarios

General statement

During RAN2#112-e, the methodology that the rapporteur handled the latency discussion was controversial that prioritized worst case scenario.

In our understanding, the latency result based on this worst case scenario may not provide meaningful results for a typical deployment, which may even be misleading to the external readers of the TR.

In this section, we provided our view on the refinement on the latency evaluation for the typical deployment, including refined procedures and refine values for the latency components.

Refined procedures

We think the following steps related to each positioning methods should be considered as “non-typical” and should be omitted. In addition, additional omission of steps related to capabilities is possible considering LMF storing UE positioning capability and/or deferred MT-LR.

| Positioning methods | Steps that can be omitted for typical positioning operation | Reasons |
| --- | --- | --- |
| DL-TDOA/DL-AoD | Step 1. LPP request capabilities Step 2. LPP provide capabilities | Deferred MT-LR and/or LMF storing the capability. |
| UL-TDOA/UL-AoA | Step 1. LPP request capabilities Step 2. LPP provide capabilities | Deferred MT-LR and/or LMF storing the capability and/or reusing MIMO SRS for positioning |
|  | Step 4. RRC SRS reconfiguration | Reusing MIMO SRS for typical indoor scenarios |
|  | Step 6. NRPPa request UE SRS activation Step 7. MAC activate SRS transmission Step 8. NRPPa request UE SRS activation response | Using periodic SRS configuration |
| Multi-RTT | Step 1. LPP request capabilities Step 2. LPP provide capabilities | Deferred MT-LR and/or LMF storing the capability. |
| DL E-CID | Step 1. LPP request capabilities Step 2. LPP provide capabilities | The capability exchange is not necessary, as UE would anyway provide anything that it supports and has available. |
| UL E-CID | Step 2. RRC measurement configuration/SRS configuration Step 3. MAC activate SRS transmission | Reusing MIMO SRS, which is quite typical for E-CID and UL measurement is only by the serving TRP. |
|  | Step 4. gNB measurement | Parallel processing with UE measurement, similar to multi-RTT. |

With those omitted steps, the evaluation results can also provide a lower bound on the positioning latency, which to our understanding can be easily achieved in commercial network.

Proposal 1: If the latency evaluation is to be captured in the TR, also include the evaluation of latency for the refined procedures by omitting the procedures listed below.

| Positioning methods | Omitted steps for typical positioning operation |
| --- | --- |
| DL-TDOA/DL-AoD | Step 1. LPP request capabilities Step 2. LPP provide capabilities |
| UL-TDOA/UL-AoA | Step 1. LPP request capabilities Step 2. LPP provide capabilities |
|  | Step 4. RRC SRS reconfiguration |
|  | Step 6. NRPPa request UE SRS activation Step 7. MAC activate SRS transmission Step 8. NRPPa request UE SRS activation response |
| Multi-RTT | Step 1. LPP request capabilities Step 2. LPP provide capabilities |
| DL E-CID | Step 1. LPP request capabilities Step 2. LPP provide capabilities |
| UL E-CID | Step 2. RRC measurement configuration/SRS configuration Step 3. MAC activate SRS transmission Step 4. gNB measurement |

Refined values

Even for the worst case scenario, some values are too optimistic, e.g. UE-gNB interface latency [0-0.5ms] that is contradicting with RAN1 analysis provided by most sources [2].

TUE-gNB

[0-0.5ms]

This value is problematic as DL and UL may be asymmetric considering traditional TDD configuration assuming a larger portion of DL slots than UL slots. The value also does not take into account the physical layer processing at UE/gNB on PDSCH/PUSCH, respectively. For UL, gNB may not be aware of data transmission from UE and the data size and thus normally is triggered by SR/BSR/UL grant. For UL configured grant, the alignment delay for UL data transmission is also subject to TDD configuration.

So from our side, at least we suggest the following alternative set of values replacing TUE-gNB.

| TUE-gNB-DL | TUE-gNB-UL |
| --- | --- |
| 0.5-1 | 0.5-8 |

In addition, the interface delay between NG-RAN and 5GC (AMF) and between 5GC functions (LMF, AMF) is subject to deployment. For example, for the case when the positioning service targeting the indoor factory, which demands low latency requirements, it is possible that both AMF and LMF can be deployed locally and could even be in the single cabinet inside the factory, due to e.g. security concerns. The existing values already raised concerns from RAN3. RAN3 already had discussion in the last meeting, and the concern has been reported to RAN#90. RAN3 is expected to continue discussion on the values in RAN3#111.

Proposal 2: If the latency evaluation is to be captured in the TR, modify TUE-gNB to the value in the following table highlighted in red.

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TUEProc-RRCReconf | 10 | RRC Reconfiguration processing |
| TUEProc-RRCDLInfo | 5 | RRC DL information transfer |
| TUEProc-RRCULInfo | 2-5 | RRC UL information transfer |
| TUEProc-RRCLocationMeas | 2-5 | RRC Location Measurement Indication |
| TUEProc-LPPCapab | 10-20 | LPP Provide Capabilities |
| TUEProc-LPPAssi | 10 | LPP Provide Assistance Data |
| TUEProc-LPPLocationRe | 5 | LPP Request/Provide Location Information |
| TUEProc-MAC-SRSAct | 1-3 | MAC-CE SRS Activation/Deactivation |
| TgNBProc-RRC | 3 | RRC Processing |
| TgNBProc-NRPPa | 3 | NRPPa Processing |
| TgNBProc-NAS/LPP | 3 | NAS/LPP Processing |
| TAMFProc | 3 | AMF Processing |
| TLMFProc | 3 | LMF Processing |
| Signalling Propagation Delays between Nodes |  |  |
| TUE-gNB-DL | 0.5-1 |  |
| TUE-gNB-UL | 0.5-8 |  |
| TgNB-AMF | 3-10 |  |
| TAMF-LMF | 1-10 |  |
| TAMF-GMLC | 3-10 |  |
| Positioning Measurement Latencies |  |  |
| TLMF-Calc | 2-30 | Position Calculation latency |
| TDL-Meas | 88.5 | Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings. |
| TUL-Meas | 12 | SRS for positioning measurement time of 12 ms can be achieved under certain SRS for positioning configuration settings depending on the frame configuration. |

Latency enhancements

Collocated LMF/AMF

Based on the existing evaluation, there are two hops between LMF with gNB/TRP, which could be 3 for disaggregated gNB architecture, and as a result it introduces transmission delay and processing delay with respect to the relay node(s) (AMF/gNB-CU). One potential solution to reduce the delay is to use collocated LMF/AMF via e.g. NPN architecture, where the LMF/AMF and the NG-RAN could be in a single-site.

For the latency-sensitive location service, the deployment optimization can offer approximately 0ms latency on the NG interface. If LMF/AMF functionality can be aggregated into a single network entity, delay between AMF and LMF can be 0, and the AMF processing time of the data exchanged between gNB and LMF can also be reduced to 0.

In summary, for the deployment optimization, we have the following proposal.

Proposal 3: For the purpose of evaluation collocated LMF/AMF, the following values highlighted in red should be assumed.

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TUEProc-RRCReconf | 10 | RRC Reconfiguration processing |
| TUEProc-RRCDLInfo | 5 | RRC DL information transfer |
| TUEProc-RRCULInfo | 2-5 | RRC UL information transfer |
| TUEProc-RRCLocationMeas | 2-5 | RRC Location Measurement Indication |
| TUEProc-LPPCapab | 10-20 | LPP Provide Capabilities |
| TUEProc-LPPAssi | 10 | LPP Provide Assistance Data |
| TUEProc-LPPLocationRe | 5 | LPP Request/Provide Location Information |
| TUEProc-MAC-SRSAct | 1-3 | MAC-CE SRS Activation/Deactivation |
| TgNBProc-RRC | 3 | RRC Processing |
| TgNBProc-NRPPa | 3 | NRPPa Processing |
| TgNBProc-NAS/LPP | 3 | NAS/LPP Processing |
| TAMFProc | 3 | AMF Processing |
| TAMFProc-LPP/NRPPa | 0 | AMF Processing Routing messages |
| TLMFProc | 3 | LMF Processing |
| Signalling Propagation Delays between Nodes |  |  |
| TUE-gNB-DL | 0.5-1 |  |
| TUE-gNB-UL | 0.5-8 |  |
| TgNB-AMF | 0-3 |  |
| TAMF-LMF | 0 |  |
| TAMF-GMLC | 0 |  |
| Positioning Measurement Latencies |  |  |
| TLMF-Calc | 2-30 | Position Calculation latency |
| TDL-Meas | 88.5 | Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings. |
| TUL-Meas | 12 | SRS for positioning measurement time of 12 ms can be achieved under certain SRS for positioning configuration settings depending on the frame configuration. |

LMF functionality in RAN

In the TR 38.856 [3], three architectures were analysed. As the conclusion, Architecture 3 is considered as the most promising one.

Architecture 3 seems like the most promising option among the ones studied. RAN3 did not evaluate the benefits of any of the architecture options in terms of latency towards the core network, RAN3 also did not fully evaluate, e.g., mobility issues associated with the introduction of the LMC.

Therefore, for the purpose of evaluating latency of LMF functionality, we suggest to only consider Architecture 3.

Figure 1 Architecture 3 from TR 38.856[3]

    - Signaling transport model

The signalling transport model involving LMC with AMF and gNB is shown in Figure 2 and Figure 3, respectively.

gNB

AMF

NG-AP

LMC

Itf-AP

Figure 2 Application protocol between AMF and LMC

LMC

gNB

Itf-AP

Figure 3 Application protocol between LMC and gNB

    - Latency model

To establish the latency analysis with LMC based on the supported application protocol with AMF and gNB, we suggest to add additional assumption on the latency assumptions with respect to LMC.

For LMC processing TLMCProc and gNB processing of Itf-AP signalling TgNBProc-ltf, 3ms processing delay can be assumed similar to gNB processing NRPPa.

For transport delay TgNB-LMC between gNB and LMC, a similar assumption to transport between gNB and AMF can be made.

For transport delay TAMF-LMC between AMF and LMF, since a NG-RAN node is needed for the relay, under ideal processing delay on the NG-RAN node, it can be the sum of TgNB-LMC and TgNB-AMF, which takes the range of 1-6ms.

In summary, we have the following proposal.

Proposal 4: For the purpose of evaluating local LMF, the following values should be assumed.

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TLMCProc | 3 |  |
| TgNBProc-ltf | 3 |  |
| Signalling Propagation Delays between Nodes |  |  |
| TgNB-LMC | 0.5-3 |  |
| TAMF-LMC | 1-6 |  |

Latency evaluation

The latency evaluation results are summarized in Table 1. The detailed evaluation for baseline/collocated LMF/AMF and LMF functionality in RAN can be found in Appendix A and Appendix B, respectively.

Table 1 Summary of latency evaluation results

|  | Latency for baseline | Latency for collocated LMF/AMF |  | Latency for LMF functionality in RAN |  |
| --- | --- | --- | --- | --- | --- |
|  | Optimized procedures | Optimized procedures | Latency reduction | Optimized procedures | Latency reduction |
| DL-TDOA/DL-AoD/Multi-RTT | 179.5-264 | 161-227.5 | 18.5ms | 162.5-227.5 | 17ms |
| UL-TDOA/UL-AoA | 66-158 | 38-78 | 28ms | 40-78 | 26ms |
| DL E-CID | 45-109 | 32-84 | 13ms | 33-84 | 12ms |
| UL E-CID | 33-96.5 | 19.5-64 | 13.5ms | 20.5-64 | 12.5ms |

Proposal 5: The latency for collocated LMF/AMF is quite close to LMF functionality in RAN with optimized procedure.

Conclusion

Base on the analysis above, we propose the following:

Proposal 1: If the latency evaluation is to be captured in the TR, also include the evaluation of latency for the refined procedures by omitting the procedures listed below.

| Positioning methods | Omitted steps for typical positioning operation |
| --- | --- |
| DL-TDOA/DL-AoD | Step 1. LPP request capabilities Step 2. LPP provide capabilities |
| UL-TDOA/UL-AoA | Step 1. LPP request capabilities Step 2. LPP provide capabilities |
|  | Step 4. RRC SRS reconfiguration |
|  | Step 6. NRPPa request UE SRS activation Step 7. MAC activate SRS transmission Step 8. NRPPa request UE SRS activation response |
| Multi-RTT | Step 1. LPP request capabilities Step 2. LPP provide capabilities |
| DL E-CID | Step 1. LPP request capabilities Step 2. LPP provide capabilities |
| UL E-CID | Step 2. RRC measurement configuration/SRS configuration Step 3. MAC activate SRS transmission |
|  | Step 4. gNB measurement |

Proposal 2: If the latency evaluation is to be captured in the TR, modify TUE-gNB to value in the the following table highlighted in red.

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TUEProc-RRCReconf | 10 | RRC Reconfiguration processing |
| TUEProc-RRCDLInfo | 5 | RRC DL information transfer |
| TUEProc-RRCULInfo | 2-5 | RRC UL information transfer |
| TUEProc-RRCLocationMeas | 2-5 | RRC Location Measurement Indication |
| TUEProc-LPPCapab | 10-20 | LPP Provide Capabilities |
| TUEProc-LPPAssi | 10 | LPP Provide Assistance Data |
| TUEProc-LPPLocationRe | 5 | LPP Request/Provide Location Information |
| TUEProc-MAC-SRSAct | 1-3 | MAC-CE SRS Activation/Deactivation |
| TgNBProc-RRC | 3 | RRC Processing |
| TgNBProc-NRPPa | 3 | NRPPa Processing |
| TgNBProc-NAS/LPP | 3 | NAS/LPP Processing |
| TAMFProc | 3 | AMF Processing |
| TLMFProc | 3 | LMF Processing |
| Signalling Propagation Delays between Nodes |  |  |
| TUE-gNB-DL | 0.5-1 |  |
| TUE-gNB-UL | 0.5-8 |  |
| TgNB-AMF | 3-10 |  |
| TAMF-LMF | 1-10 |  |
| TAMF-GMLC | 3-10 |  |
| Positioning Measurement Latencies |  |  |
| TLMF-Calc | 2-30 | Position Calculation latency |
| TDL-Meas | 88.5 | Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings. |
| TUL-Meas | 12 | SRS for positioning measurement time of 12 ms can be achieved under certain SRS for positioning configuration settings depending on the frame configuration. |

Proposal 3: For the purpose of evaluation collocated LMF/AMF, the following values highlighted in red should be assumed.

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TUEProc-RRCReconf | 10 | RRC Reconfiguration processing |
| TUEProc-RRCDLInfo | 5 | RRC DL information transfer |
| TUEProc-RRCULInfo | 2-5 | RRC UL information transfer |
| TUEProc-RRCLocationMeas | 2-5 | RRC Location Measurement Indication |
| TUEProc-LPPCapab | 10-20 | LPP Provide Capabilities |
| TUEProc-LPPAssi | 10 | LPP Provide Assistance Data |
| TUEProc-LPPLocationRe | 5 | LPP Request/Provide Location Information |
| TUEProc-MAC-SRSAct | 1-3 | MAC-CE SRS Activation/Deactivation |
| TgNBProc-RRC | 3 | RRC Processing |
| TgNBProc-NRPPa | 3 | NRPPa Processing |
| TgNBProc-NAS/LPP | 3 | NAS/LPP Processing |
| TAMFProc | 3 | AMF Processing |
| TAMFProc-LPP/NRPPa | 0 | AMF Processing Routing messages |
| TLMFProc | 3 | LMF Processing |
| Signalling Propagation Delays between Nodes |  |  |
| TUE-gNB-DL | 0.5-1 |  |
| TUE-gNB-UL | 0.5-8 |  |
| TgNB-AMF | 0-3 |  |
| TAMF-LMF | 0 |  |
| TAMF-GMLC | 0 |  |
| Positioning Measurement Latencies |  |  |
| TLMF-Calc | 2-30 | Position Calculation latency |
| TDL-Meas | 88.5 | Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings. |
| TUL-Meas | 12 | SRS for positioning measurement time of 12 ms can be achieved under certain SRS for positioning configuration settings depending on the frame configuration. |

Proposal 4: For the purpose of evaluating local LMF, the following values should be assumed.

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TLMCProc | 3 |  |
| TgNBProc-ltf | 3 |  |
| Signalling Propagation Delays between Nodes |  |  |
| TgNB-LMC | 0.5-3 |  |
| TAMF-LMC | 1-6 |  |

Proposal 5: The latency for collocated LMF/AMF is quite close to LMF functionality in RAN with optimized procedure.

- References

- RAN2#112, Chairman notes.

- TR 38.857, Study on NR Positioning Enhancements.

- TR 38.856, Study on local NR positioning in NG-RAN

- Appendix A: Positioning latency analysis for 5GC-LMF for collocated LMF/AMF

The latency of elementary procedures is provided in Table 2, for the following elementary procedures

- DL LPP

- UL LPP

- DL RRC

- UL RRC

- NRPPa

- MAC CE

The “Baseline” column is based on the latency assumption made by the rapporteur, with modified Uu interface delay discussed in section 2.2.

The “Collocated LMF/AMF” columns is based on the latency assumption in section 3.1 for collocated LMF/AMF.

The two columns under “Baseline” and “Collocated LMF/AMF” correspond to the lower bound and upper bound.

Table 2 Latency of elementary procedures for 5GC-LMF

| DL LPP | Baseline |  | Collocated LMF/AMF |  |
| --- | --- | --- | --- | --- |
| TLMFProc | 3 | 3 | 3 | 3 |
| TAMF-LMF | 1 | 10 | 0 | 0 |
| TAMFProc/TAMFProc-LPP/NRPPa | 3 | 3 | 0 | 0 |
| TgNB-AMF | 3 | 10 | 0 | 3 |
| TgNBProc-NAS/LPP | 3 | 3 | 3 | 3 |
| TUE-gNB | 0 | 0.5 | 0.5 | 1 |
| TUEProc-RRCDLInfo | 5 | 5 | 5 | 5 |
| Total | 18 | 34.5 | 11.5 | 15 |
| UL LPP | Baseline |  | Collocated LMF/AMF |  |
| TLMFProc | 3 | 3 | 3 | 3 |
| TAMF-LMF | 1 | 10 | 0 | 0 |
| TAMFProc/TAMFProc-LPP/NRPPa | 3 | 3 | 0 | 0 |
| TgNB-AMF | 3 | 10 | 0 | 3 |
| TgNBProc-NAS/LPP | 3 | 3 | 3 | 3 |
| TUE-gNB | 0 | 0.5 | 0.5 | 8 |
| TUEProc-RRCULInfo | 2 | 5 | 2 | 5 |
| Total | 15 | 34.5 | 8.5 | 22 |
| DL RRC | Baseline |  | Collocated LMF/AMF |  |
| TgNBProc-RRC | 3 | 3 | 3 | 3 |
| TUE-gNB | 0 | 0.5 | 0.5 | 1 |
| TUEProc-RRCReconf | 10 | 10 | 10 | 10 |
| Total | 13 | 13.5 | 13.5 | 14 |
| UL RRC | Baseline |  | Collocated LMF/AMF |  |
| TgNBProc-RRC | 3 | 3 | 3 | 3 |
| TUE-gNB | 0 | 0.5 | 0.5 | 8 |
| TUEProc-RRC | 2 | 5 | 2 | 5 |
| Total | 5 | 8.5 | 5.5 | 16 |
| NRPPa | Baseline |  | Collocated LMF/AMF |  |
| TLMFProc | 3 | 3 | 3 | 3 |
| TAMF-LMF | 1 | 10 | 0 | 0 |
| TAMFProc/TAMFProc-LPP/NRPPa | 3 | 3 | 0 | 0 |
| TgNB-AMF | 3 | 10 | 0 | 3 |
| TgNBProc-NRPPa | 3 | 3 | 3 | 3 |
| Total | 13 | 29 | 6 | 9 |
| MAC CE | Baseline |  | Collocated LMF/AMF |  |
| TUE-gNB | 0 | 0.5 | 0.5 | 1 |
| TUEProc-MAC-SRSAct | 1 | 3 | 1 | 3 |
| Total | 1 | 3.5 | 1.5 | 4 |

The latency of positioning methods is provided in Table 3, where the row “Optimized” corresponds to the refined procedures in section 2.1.

Table 3 Latency of positioning methods for 5GC-LMF

|  | DL-TDOA/AoD/Multi-RTT | Baseline |  | Collocated LMF/AMF |  |
| --- | --- | --- | --- | --- | --- |
| 1 | RequestCapabilities | 18 | 34.5 | 11.5 | 15 |
| 2 | ProvideCapabilities | 25 | 54.5 | 18.5 | 42 |
| 3 | ProvideAssitanceData | 28 | 44.5 | 21.5 | 25 |
| 4 | RequestLocationInformation | 23 | 39.5 | 16.5 | 27 |
| 5 | Location measurement indication | 5 | 8.5 | 5.5 | 16 |
| 6 | Measurement gap configuration | 13 | 13.5 | 13.5 | 14 |
| 7 | DL PRS measurement | 88.5 | 88.5 | 88.5 | 88.5 |
| 8 | ProvideLocationInformation | 20 | 39.5 | 13.5 | 27 |
| 9 | LMF calculation | 2 | 30 | 2 | 30 |
|  | Total | 222.5 | 353 | 191 | 284.5 |
|  | Optimized | 179.5 | 264 | 161 | 227.5 |
|  | UL-TDoA/AoA | Baseline |  | Collocated LMF/AMF |  |
| 1 | RequestCapabilities | 18 | 34.5 | 11.5 | 15 |
| 2 | ProvideCapabilities | 25 | 54.5 | 18.5 | 42 |
| 3 | Positioning information request | 13 | 29 | 6 | 9 |
| 4 | RRC configuration | 13 | 13.5 | 13.5 | 14 |
| 5 | Positioning Information Response | 13 | 29 | 6 | 9 |
| 6 | SRS activation request | 13 | 29 | 6 | 9 |
| 7 | SRS activation MAC CE | 1 | 3.5 | 1.5 | 4 |
| 8 | SRS activation response | 13 | 29 | 6 | 9 |
| 9 | Measurement request | 13 | 29 | 6 | 9 |
| 10 | SRS measurement | 12 | 12 | 12 | 12 |
| 11 | Measurement reponse | 13 | 29 | 6 | 9 |
| 12 | LMF calculation | 2 | 30 | 2 | 30 |
|  | Total | 149 | 322 | 95 | 171 |
|  | Optimized | 66 | 158 | 38 | 78 |
|  | DL-ECID | Baseline |  | Collocated LMF/AMF |  |
| 1 | RequestCapabilities | 18 | 34.5 | 11.5 | 15 |
| 2 | ProvideCapabilities | 25 | 54.5 | 18.5 | 42 |
| 3 | RequestLocationInformation | 23 | 39.5 | 16.5 | 27 |
| 4 | UE measurement | 0 | 0 | 0 | 0 |
| 5 | ProvideLocationInformation | 20 | 39.5 | 13.5 | 27 |
| 6 | LMF calculation | 2 | 30 | 2 | 30 |
|  | Total | 88 | 198 | 62 | 141 |
|  | Optimized | 45 | 109 | 32 | 84 |
|  | UL-ECID | Baseline |  | Collocated LMF/AMF |  |
| 1 | E-CID measurement initiation request | 13 | 29 | 6 | 9 |
| 2 | RRC configuration | 13 | 13.5 | 13.5 | 14 |
| 3 | SRS activation MAC CE | 1 | 3.5 | 1.5 | 4 |
| 4 | gNB measurement | 12 | 12 | 12 | 12 |
| 5 | RRC measurement report | 5 | 8.5 | 5.5 | 16 |
| 6 | E-CID measurement initiation response | 13 | 29 | 6 | 9 |
| 7 | LMF calculation | 2 | 30 | 2 | 30 |
|  | Total | 59 | 125.5 | 46.5 | 94 |
|  | Optimized | 33 | 96.5 | 19.5 | 64 |

- Appendix B: Positioning latency analysis for local LMF

The latency of elementary procedures is provided in Table 2, for the following elementary procedures

- DL LPP

- UL LPP

- DL RRC

- UL RRC

- NRPPa

- MAC CE

The “LMF func. In RAN” columns is based on the latency assumption in section 3.2 for LMF functionality in RAN.

The two columns under “LMF func. In RAN” correspond to the lower bound and upper bound.

Table 4 Latency of elementary procedures for local LMF

| DL LPP | LMF func. In RAN |  |
| --- | --- | --- |
| TLMCProc | 3 | 3 |
| TgNB-LMC | 0.5 | 3 |
| TgNBProc-ltf | 3 | 3 |
| TUE-gNB | 0.5 | 1 |
| TUEProc-RRCDLInfo | 5 | 5 |
| Total | 12 | 15 |
| UL LPP | LMF func. In RAN |  |
| TLMCProc | 3 | 3 |
| TgNB-LMC | 0.5 | 3 |
| TgNBProc-ltf | 3 | 3 |
| TUE-gNB | 0.5 | 8 |
| TUEProc-RRCULInfo | 2 | 5 |
| Total | 9 | 22 |
| DL RRC | LMF func. In RAN |  |
| TgNBProc-RRC | 3 | 3 |
| TUE-gNB | 0.5 | 1 |
| TUEProc-RRCReconf | 10 | 10 |
| Total | 13.5 | 14 |
| UL RRC | LMF func. In RAN |  |
| TgNBProc-RRC | 3 | 3 |
| TUE-gNB | 0.5 | 8 |
| TUEProc-RRC | 2 | 5 |
| Total | 5.5 | 16 |
| NRPPa | LMF func. In RAN |  |
| TLMCProc | 3 | 3 |
| TgNB-LMC | 0.5 | 3 |
| TgNBProc-NRPPa | 3 | 3 |
| Total | 6.5 | 9 |
| MAC CE | LMF func. In RAN |  |
| TUE-gNB | 0.5 | 1 |
| TUEProc-MAC-SRSAct | 1 | 3 |
| Total | 1.5 | 4 |

The latency of positioning methods is provided in Table 5, where the row “Optimized” corresponds to the refined procedures in section 2.1.

Table 5 Latency of positioning methods for 5GC-LMF

|  | DL-TDOA/AoD/Multi-RTT | LMF func. In RAN |  |
| --- | --- | --- | --- |
| 1 | RequestCapabilities | 12 | 15 |
| 2 | ProvideCapabilities | 19 | 42 |
| 3 | ProvideAssitanceData | 22 | 25 |
| 4 | RequestLocationInformation | 17 | 27 |
| 5 | Location measurement indication | 5.5 | 16 |
| 6 | Measurement gap configuration | 13.5 | 14 |
| 7 | DL PRS measurement | 88.5 | 88.5 |
| 8 | ProvideLocationInformation | 14 | 27 |
| 9 | LMF calculation | 2 | 30 |
|  | Total | 193.5 | 284.5 |
|  | Optimized | 162.5 | 227.5 |
|  | UL-TDoA/AoA | LMF func. In RAN |  |
| 1 | RequestCapabilities | 12 | 15 |
| 2 | ProvideCapabilities | 19 | 42 |
| 3 | Positioning information request | 6.5 | 9 |
| 4 | RRC configuration | 13.5 | 14 |
| 5 | Positioning Information Response | 6.5 | 9 |
| 6 | SRS activation request | 6.5 | 9 |
| 7 | SRS activation MAC CE | 1.5 | 4 |
| 8 | SRS activation response | 6.5 | 9 |
| 9 | Measurement request | 6.5 | 9 |
| 10 | SRS measurement | 12 | 12 |
| 11 | Measurement reponse | 6.5 | 9 |
| 12 | LMF calculation | 2 | 30 |
|  | Total | 99 | 171 |
|  | Optimized | 40 | 78 |
|  | DL-ECID | LMF func. In RAN |  |
| 1 | RequestCapabilities | 12 | 15 |
| 2 | ProvideCapabilities | 19 | 42 |
| 3 | RequestLocationInformation | 17 | 27 |
| 4 | UE measurement | 0 | 0 |
| 5 | ProvideLocationInformation | 14 | 27 |
| 6 | LMF calculation | 2 | 30 |
|  | Total | 64 | 141 |
|  | Optimized | 33 | 84 |
|  | UL-ECID | LMF func. In RAN |  |
| 1 | E-CID measurement initiation request | 6.5 | 9 |
| 2 | RRC configuration | 13.5 | 14 |
| 3 | SRS activation MAC CE | 1.5 | 4 |
| 4 | gNB measurement | 12 | 12 |
| 5 | RRC measurement report | 5.5 | 16 |
| 6 | E-CID measurement initiation response | 6.5 | 9 |
| 7 | LMF calculation | 2 | 30 |
|  | Total | 47.5 | 94 |
|  | Optimized | 20.5 | 64 |