3GPP TSG-RAN WG2 #113e	R2-2101379

Electronic meeting, Jan 25th – 5th Feb 2021

Agenda Item:	5.5

Source:	Ericsson

Title:	GNSS RTK observations resolution indication

Document for:	Discussion, Decision

# 1	Introduction

GNSS RTK support was introduced in 3GPP LPP [1] in Rel 15 and 16. Part of the GNSS RTK concerns GNSS RTK Observation Space Representation (OSR), leveraged by the generic RTCM Multiple Signal Messages (MSM) [2] and introduced in Rel 15..

This paper concerns how the device can determine whether the 3GPP LPP information originates from standard or high resolution MSM, and the need for an optional indicator of high resolution.

# 2	Discussion

The RTCM MSM messages are generically represented by the IE GNSS-RTK-Observations-r15 with optional fields to enable representation of all the MSM types. The RTCM MSM types are listed in Table 1 below.

Table 1. MSM types, satellite signal attributes, from table 3.5-71 [2]

| MSM Type | Message Name |
| --- | --- |
| MSM1 | Compact GNSS Pseudoranges |
| MSM2 | Compact GNSS PhaseRanges |
| MSM3 | Compact GNSS Pseudoranges and PhaseRanges |
| MSM4 | Full GNSS Pseudoranges and PhaseRanges plus Carrier to Noise Ratio |
| MSM5 | Full GNSS Pseudoranges, PhaseRanges, PhaseRangeRate and Carrier to Noise Ratio |
| MSM6 | Full GNSS Pseudoranges and PhaseRanges plus Carrier to Noise Ratio (high resolution) |
| MSM7 | Full GNSS Pseudoranges, PhaseRanges, PhaseRangeRate and Carrier to Noise Ratio (high resolution) |

Table 2 further lists the attribute resolution per MSM types in terms of standard or high resolution.

Table 2. MSM satellite signal attribute resolutions, standard or high.

| Attribute | MSM1 | MSM2 | MSM3 | MSM4 | MSM5 | MSM6 | MSM7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GNSS signal fine Pseudoranges | Std |  | Std | Std | Std | High | High |
| GNSS signal fine Phaserange data |  | Std | Std | Std | Std | High | High |
| GNSS Phaserange Lock Time Indicator |  | Std | Std | Std | Std | High | High |
| Half-cycle ambiguity indicator |  | - | - | - | - | - | - |
| GNSS signal Carrier-to-Noise Ratio |  |  |  | Std | Std | High | High |
| GNSS signal fine Phaserange Rates |  |  |  |  | Std |  | High |

The MSM6 and MSM7 provides observations in high resolution, while the other MSM types provide observations in standard resolution. For the IE GNSS-RTK-Observations-r15, all the attributes are represented according to the high resolution.

-- ASN1START

GNSS-RTK-Observations-r15 ::= SEQUENCE {

epochTime-r15							GNSS-SystemTime,

gnss-ObservationList-r15				GNSS-ObservationList-r15,

...

}

GNSS-ObservationList-r15 ::= SEQUENCE (SIZE(1..64)) OF GNSS-RTK-SatelliteDataElement-r15

GNSS-RTK-SatelliteDataElement-r15 ::= SEQUENCE{

svID-r15								SV-ID,

integer-ms-r15							INTEGER (0..254)				OPTIONAL,	-- Need ON

rough-range-r15							INTEGER (0..1023),

rough-phase-range-rate-r15				INTEGER (-8192..8191)			OPTIONAL,	-- Need ON

gnss-rtk-SatelliteSignalDataList-r15	GNSS-RTK-SatelliteSignalDataList-r15,

...

}

GNSS-RTK-SatelliteSignalDataList-r15 ::= SEQUENCE (SIZE(1..24)) OF

GNSS-RTK-SatelliteSignalDataElement-r15

GNSS-RTK-SatelliteSignalDataElement-r15 ::= SEQUENCE {

gnss-SignalID-r15					GNSS-SignalID,

fine-PseudoRange-r15				INTEGER (-524288..524287),

fine-PhaseRange-r15					INTEGER (-8388608..8388607),

lockTimeIndicator-r15				INTEGER (0..1023),

halfCycleAmbiguityIndicator-r15		BIT STRING (SIZE (1)),

carrier-to-noise-ratio-r15			INTEGER (0..1023)					OPTIONAL,	-- Need ON

fine-PhaseRangeRate-r15				INTEGER (-16384..16383)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

For a UE, especially based on a legacy positioning engine defined for RTCM MSM, one possible handling of the 3GPP LPP IE is to translate that into MSM and forward to the positioning engine. In this case, it is not possible for the UE to determine if the IE originates from MSM4 or MSM6, or from MSM5 or MSM7. Furthermore, the device may use the standard/high resolution information to assess the observation uncertainty in addition to the RTK residuals information.

It is not possible to determine if an IE GNSS-RTK-Observations-r15 originates from MSM4 or MSM6, of from MSM5 or MSM7.

Devices that internally translates the obtained GNSS-RTK-Observations-r15  into corresponding MSM cannot correctly do that.

A simple indication can solve this issue – by indicating an optional indication about what resolution that the information originates from.

Option 1, Indicate optionally that a standard resolution has been used. If omitted, the assumption is that a high resolution has been used.

-- ASN1START

GNSS-RTK-Observations-r15 ::= SEQUENCE {

epochTime-r15							GNSS-SystemTime,

gnss-ObservationList-r15				GNSS-ObservationList-r15,

...,

[[

gnss-ObservationStdRes-r15			NULL							OPTIONAL

]]

}

GNSS-ObservationList-r15 ::= SEQUENCE (SIZE(1..64)) OF GNSS-RTK-SatelliteDataElement-r15

GNSS-RTK-SatelliteDataElement-r15 ::= SEQUENCE{

svID-r15								SV-ID,

integer-ms-r15							INTEGER (0..254)				OPTIONAL,	-- Need ON

rough-range-r15							INTEGER (0..1023),

rough-phase-range-rate-r15				INTEGER (-8192..8191)			OPTIONAL,	-- Need ON

gnss-rtk-SatelliteSignalDataList-r15	GNSS-RTK-SatelliteSignalDataList-r15,

...

}

GNSS-RTK-SatelliteSignalDataList-r15 ::= SEQUENCE (SIZE(1..24)) OF

GNSS-RTK-SatelliteSignalDataElement-r15

GNSS-RTK-SatelliteSignalDataElement-r15 ::= SEQUENCE {

gnss-SignalID-r15					GNSS-SignalID,

fine-PseudoRange-r15				INTEGER (-524288..524287),

fine-PhaseRange-r15					INTEGER (-8388608..8388607),

lockTimeIndicator-r15				INTEGER (0..1023),

halfCycleAmbiguityIndicator-r15		BIT STRING (SIZE (1)),

carrier-to-noise-ratio-r15			INTEGER (0..1023)					OPTIONAL,	-- Need ON

fine-PhaseRangeRate-r15				INTEGER (-16384..16383)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

Option 2, Indicate optionally that a high resolution has been used. If omitted, the assumption is that a standard resolution has been used.

-- ASN1START

GNSS-RTK-Observations-r15 ::= SEQUENCE {

epochTime-r15							GNSS-SystemTime,

gnss-ObservationList-r15				GNSS-ObservationList-r15,

...,

[[

gnss-ObservationHighRes-r15			NULL							OPTIONAL

]]

}

GNSS-ObservationList-r15 ::= SEQUENCE (SIZE(1..64)) OF GNSS-RTK-SatelliteDataElement-r15

GNSS-RTK-SatelliteDataElement-r15 ::= SEQUENCE{

svID-r15								SV-ID,

integer-ms-r15							INTEGER (0..254)				OPTIONAL,	-- Need ON

rough-range-r15							INTEGER (0..1023),

rough-phase-range-rate-r15				INTEGER (-8192..8191)			OPTIONAL,	-- Need ON

gnss-rtk-SatelliteSignalDataList-r15	GNSS-RTK-SatelliteSignalDataList-r15,

...

}

GNSS-RTK-SatelliteSignalDataList-r15 ::= SEQUENCE (SIZE(1..24)) OF

GNSS-RTK-SatelliteSignalDataElement-r15

GNSS-RTK-SatelliteSignalDataElement-r15 ::= SEQUENCE {

gnss-SignalID-r15					GNSS-SignalID,

fine-PseudoRange-r15				INTEGER (-524288..524287),

fine-PhaseRange-r15					INTEGER (-8388608..8388607),

lockTimeIndicator-r15				INTEGER (0..1023),

halfCycleAmbiguityIndicator-r15		BIT STRING (SIZE (1)),

carrier-to-noise-ratio-r15			INTEGER (0..1023)					OPTIONAL,	-- Need ON

fine-PhaseRangeRate-r15				INTEGER (-16384..16383)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

Add an optional indicator in the IE GNSS-RTK-Observations-r15 for the attribute resolution of the origin MSM message.

RAN2 agree to the TP in Appendix

# Conclusion

In the previous sections we made the following observations:

Observation 1	It is not possible to determine if an IE GNSS-RTK-Observations-r15 originates from MSM4 or MSM6, of from MSM5 or MSM7.

Observation 2	Devices that internally translates the obtained GNSS-RTK-Observations-r15  into corresponding MSM cannot correctly do that.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add an optional indicator in the IE GNSS-RTK-Observations-r15 for the attribute resolution of the origin MSM message.

Proposal 2	RAN2 agree to the CR [3]

# References

3GPP TS 37.355 V 16.3.0

RTCM SC 104 v3.x

R2-21xxxxx Correction of A-GNSS Assistance Data RTK Observation, Ericsson