3GPP TSG-RAN WG2 Meeting #113-e	R2-2101400

# E-meeting, 25th Jan – 5th Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0246 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on support of NR-DC within the same gNB-DU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh_Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on current TS 37.340, NR-DC can be supported in two scenarios: - UE connected to two gNBs, one gNB acts as MN, the other gNB acts as SN; - UE connected to two gNB-DUs, one gNB-DU acts as MN, the other gNB-DU acts as SN, both are connected to the same gNB-CU. However, in real deployment, the one gNB-DU may have multiple cells operating on different frequencies, and it is observed for a given band combination, some UEs may only support NR-DC operation, but not support NR CA for the same BC (e.g. FR1+FR2). So, network is unable to configure multiple carriers to the UE for higher throughput. From UE perspective, it is transparent to UE whether the cells configured for NR-DC are deployed in the same DU or not. So we propose to clarify in stage 2 spec, that NR-DC can also be supported between different cells within the same DU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add note to section 4.1.1: “NOTE 3: NR-DC is designed between different nodes, but can also be used between different cells within the same node” - Clarify NR-DC can be used in when a UE is connected to two cells within the same gNB-DU in section 4.1.3.3. mpact analysis Impacted 5G architecture options: NR-DC Impacted functionality: NR-DC configuration Inter-operability: - If the network is implemented according to the CR and the UE is not, the configuration (e.g. gNB’s deployment) is transparent to UE, so no inter-operability issue is foreseen. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | According to Stage2, the network cannot configure NR-DC in case the cells belong to the same gNB-DU. Thus when network deploys both FR1 and FR2 cells within a DU, and a UE supports FR1+FR2 NR-DC but does not support FR1+FR2 CA, it may not be configured with multiple carriers, so it cannot be benefit from higher data throughput. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1, 4.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

4.1	General

4.1.1	Common MR-DC principles

Multi-Radio Dual Connectivity (MR-DC) is a generalization of the Intra-E-UTRA Dual Connectivity (DC) described in TS 36.300 [2], where a multiple Rx/Tx capable UE may be configured to utilise resources provided by two different nodes connected via non-ideal backhaul, one providing NR access and the other one providing either E-UTRA or NR access. One node acts as the MN and the other as the SN. The MN and SN are connected via a network interface and at least the MN is connected to the core network.

The MN and/or the SN can be operated with shared spectrum channel access.

All functions specified for a UE may be used for an IAB-MT unless otherwise stated. Similar as specified for UE, the IAB-MT can access the network using either one network node or using two different nodes with EN-DC and NR-DC architectures. In EN-DC, the backhauling traffic over the E-UTRA radio interface is not supported.

NOTE 1:	MR-DC is designed based on the assumption of non-ideal backhaul between the different nodes but can also be used in case of ideal backhaul.

NOTE 2:	All MR-DC normative text and procedures in this version of the specification show the aggregated node case. The details about non-aggregated node for MR-DC operation are described in TS 38.401 [7].

NOTE 3:	NR-DC is designed between different nodes, but can also be used between different cells within the same node.

4.1.2	MR-DC with the EPC

E-UTRAN supports MR-DC via E-UTRA-NR Dual Connectivity (EN-DC), in which a UE is connected to one eNB that acts as a MN and one en-gNB that acts as a SN. The eNB is connected to the EPC via the S1 interface and to the en-gNB via the X2 interface. The en-gNB might also be connected to the EPC via the S1-U interface and other en-gNBs via the X2-U interface.

The EN-DC architecture is illustrated in Figure 4.1.2-1 below.

# Figure 4.1.2-1:	EN-DC Overall Architecture

### 4.1.3	MR-DC with the 5GC

#### 4.1.3.1	E-UTRA-NR Dual Connectivity

NG-RAN supports NG-RAN E-UTRA-NR Dual Connectivity (NGEN-DC), in which a UE is connected to one ng-eNB that acts as a MN and one gNB that acts as a SN.

#### 4.1.3.2	NR-E-UTRA Dual Connectivity

NG-RAN supports NR-E-UTRA Dual Connectivity (NE-DC), in which a UE is connected to one gNB that acts as a MN and one ng-eNB that acts as a SN.

#### 4.1.3.3	NR-NR Dual Connectivity

NG-RAN supports NR-NR Dual Connectivity (NR-DC), in which a UE is connected to one gNB that acts as a MN and another gNB that acts as a SN. In addition, NR-DC can also be used when a UE is connected to two gNB-DUs (or two cells within the same gNB-DU), one serving the MCG and the other serving the SCG, connected to the same gNB-CU, acting both as a MN and as a SN.

End of change