3GPP TSG-RAN WG2 #113-e	Tdoc R2-2101428

Electronic meeting, Jan 25th – Feb 5th, 2021	Revision of R2-2010534

Agenda Item:	8.3.2

Source:	Ericsson

Title:	Paging collision avoidance

Document for:	Discussion, Decision

# 1	Introduction

MultiSIM devices enable use of multiple subscriptions in one device, e.g. more than one USIM and concurrent registration in more than one network. The USIMs are from same or different operator (PLMN).

A MultiSIM device needs to handle “simultaneous state” situations like:

- RRC_IDLE/INACTIVE + RRC_IDLE/INACTIVE, e.g.

  - check paging in multiple systems

  - receive system information for multiple systems

  - perform measurements on multiple systems

- RRC_CONNECTED+ RRC_INACTIVE/IDLE, e.g.

  - voice + paging/SI/measurements

RAN2 WID RP-201309 [1] defines three objectives for the support for MultiSIM devices in Rel-17, In this contribution the focus is on the objective 1, reported here:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs:

  - RAT Concurrency: Network A can be NR. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx.

This contribution was revised to account the agreement taken in SA2 group. In this respect, the proposal to use a solution based on UE implementation has been removed.

# 2	Discussion

​A MultiSIM UE with a single Rx processing unit/chain can receive transmissions from one PLMN at a time. One major issue is the possibility to miss the Paging message sent by a PLMN, if the UE listening to the Paging in the other PLMN, causing performance degradation in terms of paging detection.

This may happen if the Paging Occasions (PO) / Paging Frame (PF) for the different USIMs are overlapping in time domain.

The figure shows an example of Paging collision: the slots in bold indicates where the Multi-SIM UE is listening to Paging (PLMN1 or PLMN2). If the UE is listening to PLMN1 while the Paging message is sent by PLMN2, the message is missed (red slot). Even though the paging is repeated (after CN timer T3513 expires) the message may be missed again.

According to TS 36.304 [2] and TS 38.304 [3] the formulas for calculating Paging Frame and Paging Occasions are the following:

(System Frame Number) SFN for the PF is determined by:

5G:	(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

LTE:	SFN mod T= (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

Where:

UE_ID: 5G-S-TMSI mod 1024		(in the 5GS)

UE_ID: IMSI mod 1024			(in the EPS)

The SFN and PO depends on the UE_ID, which can be reassigned (in case of 5GS) over time at the Mobility Registration procedure, and on other parameters configurable on cell level (e.g. T, N, Ns).

In TR 23.761 [4], SA2 group agreed to the following solution for EPS:

-	For paging reception in EPS when the paging collision is detected, the following principles are agreed:

-	Upon the UE detecting paging collisions between two networks, the UE initiates a TAU procedure to the MME of one network, to request an IMSI offset.

-	UE may provide an IMSI offset to MME during TAU procedure.

NOTE: Details on the request e.g. offset range will be defined during the normative phase.

-	The MME returns an IMSI offset to the UE in the TAU Accept.

-	During CN paging delivery, the MME provides to the RAN the UE_ID which is derived based on the IMSI and the IMSI offset. RAN and UE use the UE ID as the IMSI to calculate the PF/PO.

Based on the SA2 agreement, an additional offset is included in the SFN and PO calculation for LTE in TS 36.304.

This corresponds to use an additional offset (here called “UE_offset”) in the SFN and PO calculation formulas in [2]:

LTE:	SFN mod T= (T div N)*((UE_ID + UE_offset) mod N)

i_s = floor ((UE_ID + UE_offset) / N) mod Ns

Alternatively, the offset can be included directly in the UE_ID calculation formula, which leads to a similar outcome after the modulo N operation:

UE_ID: (IMSI + UE_offset) mod 1024			(in the EPS)

This requires no change to the SFN and PO calculation formulas.

RAN2 to decide if to include the offset in the SFN and PO formulas or alternatively in the UE_ID formula.

Text proposal for TS.36.304 update is shown in Appendix A1 (UE_offset in the SFN and PO formulas) and Appendix A2 (UE_offset in the UE_ID formula). SA2 has taken no decision for the NR/5GS case.

SA2 group agreed to use an offset for EPS for paging collision avoidance, but no decision has been taken for the 5GS scenario.

For the sake of keeping the specification complexity on reasonable level, we should avoid introducing multiple solutions to the same problem. Since SA2 already agreed on usage of the offset for E-UTRA/EPS scenario, the same solution should be used for NR/5GS, as well. This allows to have a common procedure to cover all the possible scenarios.

For the sake of keeping the specification complexity on reasonable level, we should avoid introducing multiple solutions to the same problem.

So, here is it proposed to use the same approach and include an additional offset in the SFN and PO calculation formulas for the 5G case in [3]:

5G:	(SFN + PF_offset) mod T = (T div N)*((UE_ID + UE_offset) mod N)

i_s = floor ((UE_ID + UE_offset) / N) mod Ns

Or, alternatively, the offset can be included directly in the UE_ID calculation:

UE_ID: (5G-S-TMSI + UE_offset) mod 1024		(in the 5GS)

In order to have a common solution to the paging collision problem, the same approach as decided for EPS should be used to 5GS case, as well. That is, an offset is included to the SFN and PO or UE_ID calculation in TS 38.304.

The procedures to notify the PO collision and assign the UE offset are described in the Appendix B.

# Conclusion

In the previous sections we made the following observations:

- SA2 group agreed to use an offset for EPS for paging collision avoidance, but no decision has been taken for the 5GS scenario .

- For the sake of keeping the specification complexity on reasonable level, we should avoid introducing multiple solution to the same problem.

- Based on the discussion in the previous sections we propose the following:

- Based on the SA2 agreement, an additional offset is included in the SFN and PO calculation for LTE in TS 36.304.

RAN2 to decide if to include the offset in the SFN and PO formulas or alternatively in the UE_ID formula.

In order to have a common solution to the paging collision problem, the same approach as decided for EPS should be used to 5GS case, as well. That is, an offset is included to the SFN and PO or UE_ID calculation in TS 38.304.

# References

RP-201309, WID: Support for Multi-SIM devices for LTE/NR.

TS 36.304 v.16.0.0, User Equipment (UE) procedures in idle mode

TS 38.304 v.16.0.0, User Equipment (UE) procedures in idle mode

TR 23.761 v.1.2.0, Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM)

# Appendix A1: Text proposal to add offset in SFN and PO formulas in TS 36.304

START OF THE CHANGE

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in idle mode in order to reduce power consumption. One Paging Occasion (PO) is a subframe where there may be P-RNTI transmitted on PDCCH or MPDCCH or, for NB-IoT on NPDCCH addressing the paging message. In P-RNTI transmitted on MPDCCH case, PO refers to the starting subframe of MPDCCH repetitions. In case of P-RNTI transmitted on NPDCCH, PO refers to the starting subframe of NPDCCH repetitions unless subframe determined by PO is not a valid NB-IoT downlink subframe then the first valid NB-IoT downlink subframe after PO is the starting subframe of the NPDCCH repetitions. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS.

One Paging Frame (PF) is one Radio Frame, which may contain one or multiple Paging Occasion(s). When DRX is used the UE needs only to monitor one PO per DRX cycle.

One Paging Narrowband (PNB) is one narrowband, on which the UE performs the paging message reception.

PF, PO, and PNB are determined by following formulae using the DRX parameters provided in System Information:

PF is given by following equation:

SFN mod T= (T div N)*(UE_ID mod N)

Index i_s pointing to PO from subframe pattern defined in 7.2 will be derived from following calculation:

i_s = floor(UE_ID/N) mod Ns

For the UE supporting MultiSIM, the PF and Index i_s pointing to PO from subframe pattern defined in 7.2 are determined by the following equations:

SFN mod T= (T div N)*((UE_ID + UE_offset) mod N)

i_s = floor((UE_ID + UE_offset)/N) mod Ns

If P-RNTI is monitored on MPDCCH, the PNB is determined by the following equation:

PNB = floor(UE_ID/(N*Ns)) mod Nn

If P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information, then the paging carrier is determined by the paging carrier with smallest index n (0 ≤ n ≤ Nn-1) fulfilling the following equation:

floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n)

System Information DRX parameters stored in the UE shall be updated locally in the UE whenever the DRX parameter values are changed in SI. If the UE has no IMSI, for instance when making an emergency call without USIM, the UE shall use as default identity UE_ID = 0 in the PF, i_s, and PNB formulas above. If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

The following Parameters are used for the calculation of the PF, i_s, PNB, and the NB-IoT paging carrier:

-	T: DRX cycle of the UE.

Except for NB-IoT: If a UE specific extended DRX value of 512 radio frames is configured by upper layers according to 7.3, T =512. Otherwise, T is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information. If UE specific DRX is not configured by upper layers, the default value is applied. In RRC_INACTIVE state, if extended DRX is not configured by upper layers as defined in 7.3, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, and the default paging cycle, if allocated by upper layers. Otherwise, in RRC_INACTIVE state when extended DRX is configured by upper layers, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, if allocated by upper layers and the default paging cycle during the PTW as defined in 7.3, and by the RAN paging cycle outside the PTW.

For NB-IoT: If UE specific DRX value is allocated by upper layers and minimum UE specific DRX value is broadcast in system information, T = min (default DRX value, max (UE specific DRX value, minimum UE specific DRX value broadcast in system information)). If UE specific DRX is not configured by upper layers or if the minimum UE specific DRX value is not broadcast in system information, the default DRX value is applied.

-	nB: 4T, 2T, T, T/2, T/4, T/8, T/16, T/32, T/64, T/128, and T/256, and for NB-IoT also T/512, and T/1024.

-	N: min(T,nB)

-	Ns: max(1,nB/T)

-	Nn: number of paging narrowbands (for P-RNTI monitored on MPDCCH) or paging carriers (for P-RNTI monitored on NPDCCH) determined as follows:

If UE monitors GWUS according to clause 7.5.1:

this is the number of paging narrowbands (paging carriers) that are configured with GWUS.

else:

this is the number of paging narrowbands (paging carriers) provided in system information.

-	UE_ID:

If the UE supports E-UTRA connected to 5GC and NAS indicated to use 5GC for the selected cell:

5G-S-TMSI mod 1024, if P-RNTI is monitored on PDCCH.

5G-S-TMSI mod 16384, if P-RNTI is monitored on NPDCCH or MPDCCH.

else

IMSI mod 1024, if P-RNTI is monitored on PDCCH.

IMSI mod 4096, if P-RNTI is monitored on NPDCCH.

IMSI mod 16384, if P-RNTI is monitored on MPDCCH or if P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information.

-	W(i): Weight for NB-IoT paging carrier i.

-	W: Total weight of all NB-IoT paging carriers, i.e. W = W(0) + W(1) + … + W(Nn-1). If UE monitors GWUS according to clause 7.5.1, Total weight of all NB-IoT paging carriers configured with GWUS.

-	UE_offset: Offset used to avoid possible SFN and PO overlapping for a UE supporting MultiSIM.

IMSI is given as sequence of digits of type Integer (0..9), IMSI shall in the formulae above be interpreted as a decimal integer number, where the first digit given in the sequence represents the highest order digit.

For example:

IMSI = 12 (digit1=1, digit2=2)

In the calculations, this shall be interpreted as the decimal integer "12", not "1x16+2 = 18".

5G-S-TMSI is a 48 bit long bit string as defined in TS 23.501 [39]. 5G-S-TMSI shall in the PF and i_s formulae above be interpreted as a binary number where the left most bit represents the most significant bit.

END OF THE CHANGE

# Appendix A2: Text proposal to add offset in UE_ID formula in TS 36.304

START OF THE CHANGE

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in idle mode in order to reduce power consumption. One Paging Occasion (PO) is a subframe where there may be P-RNTI transmitted on PDCCH or MPDCCH or, for NB-IoT on NPDCCH addressing the paging message. In P-RNTI transmitted on MPDCCH case, PO refers to the starting subframe of MPDCCH repetitions. In case of P-RNTI transmitted on NPDCCH, PO refers to the starting subframe of NPDCCH repetitions unless subframe determined by PO is not a valid NB-IoT downlink subframe then the first valid NB-IoT downlink subframe after PO is the starting subframe of the NPDCCH repetitions. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS.

One Paging Frame (PF) is one Radio Frame, which may contain one or multiple Paging Occasion(s). When DRX is used the UE needs only to monitor one PO per DRX cycle.

One Paging Narrowband (PNB) is one narrowband, on which the UE performs the paging message reception.

PF, PO, and PNB are determined by following formulae using the DRX parameters provided in System Information:

PF is given by following equation:

SFN mod T= (T div N)*(UE_ID mod N)

Index i_s pointing to PO from subframe pattern defined in 7.2 will be derived from following calculation:

i_s = floor(UE_ID/N) mod NsIf P-RNTI is monitored on MPDCCH, the PNB is determined by the following equation:

PNB = floor(UE_ID/(N*Ns)) mod Nn

If P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information, then the paging carrier is determined by the paging carrier with smallest index n (0 ≤ n ≤ Nn-1) fulfilling the following equation:

floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n)

System Information DRX parameters stored in the UE shall be updated locally in the UE whenever the DRX parameter values are changed in SI. If the UE has no IMSI, for instance when making an emergency call without USIM, the UE shall use as default identity UE_ID = 0 in the PF, i_s, and PNB formulas above. If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

The following Parameters are used for the calculation of the PF, i_s, PNB, and the NB-IoT paging carrier:

-	T: DRX cycle of the UE.

Except for NB-IoT: If a UE specific extended DRX value of 512 radio frames is configured by upper layers according to 7.3, T =512. Otherwise, T is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information. If UE specific DRX is not configured by upper layers, the default value is applied. In RRC_INACTIVE state, if extended DRX is not configured by upper layers as defined in 7.3, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, and the default paging cycle, if allocated by upper layers. Otherwise, in RRC_INACTIVE state when extended DRX is configured by upper layers, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, if allocated by upper layers and the default paging cycle during the PTW as defined in 7.3, and by the RAN paging cycle outside the PTW.

For NB-IoT: If UE specific DRX value is allocated by upper layers and minimum UE specific DRX value is broadcast in system information, T = min (default DRX value, max (UE specific DRX value, minimum UE specific DRX value broadcast in system information)). If UE specific DRX is not configured by upper layers or if the minimum UE specific DRX value is not broadcast in system information, the default DRX value is applied.

-	nB: 4T, 2T, T, T/2, T/4, T/8, T/16, T/32, T/64, T/128, and T/256, and for NB-IoT also T/512, and T/1024.

-	N: min(T,nB)

-	Ns: max(1,nB/T)

-	Nn: number of paging narrowbands (for P-RNTI monitored on MPDCCH) or paging carriers (for P-RNTI monitored on NPDCCH) determined as follows:

If UE monitors GWUS according to clause 7.5.1:

this is the number of paging narrowbands (paging carriers) that are configured with GWUS.

else:

this is the number of paging narrowbands (paging carriers) provided in system information.

-	UE_ID:

If the UE supports E-UTRA connected to 5GC and NAS indicated to use 5GC for the selected cell:

5G-S-TMSI mod 1024, if P-RNTI is monitored on PDCCH.

5G-S-TMSI mod 16384, if P-RNTI is monitored on NPDCCH or MPDCCH.

else

IMSI mod 1024, if P-RNTI is monitored on PDCCH.

(IMSI + UE_offset) mod 1024, if P-RNTI is monitored on PDCCH and the UE supports MultiSIM.

IMSI mod 4096, if P-RNTI is monitored on NPDCCH.

IMSI mod 16384, if P-RNTI is monitored on MPDCCH or if P-RNTI is monitored on NPDCCH and the UE supports paging on a non-anchor carrier, and if paging configuration for non-anchor carrier is provided in system information.

-	W(i): Weight for NB-IoT paging carrier i.

-	W: Total weight of all NB-IoT paging carriers, i.e. W = W(0) + W(1) + … + W(Nn-1). If UE monitors GWUS according to clause 7.5.1, Total weight of all NB-IoT paging carriers configured with GWUS.

-	UE_offset: Offset used to avoid possible SFN and PO overlapping for a UE supporting MultiSIM.

IMSI is given as sequence of digits of type Integer (0..9), IMSI shall in the formulae above be interpreted as a decimal integer number, where the first digit given in the sequence represents the highest order digit.

For example:

IMSI = 12 (digit1=1, digit2=2)

In the calculations, this shall be interpreted as the decimal integer "12", not "1x16+2 = 18".

5G-S-TMSI is a 48 bit long bit string as defined in TS 23.501 [39]. 5G-S-TMSI shall in the PF and i_s formulae above be interpreted as a binary number where the left most bit represents the most significant bit.

END OF THE CHANGE

# Appendix B: Procedures to notify the PO collision and assign the UE offset

The procedure to notify the PO collision and assign the new offset is shown in the figure below:

The solution requires the UE to check if there is a collision for the PO between the two PLMNs.

The check is done at cell change and whenever a new 5G-GUTI is assigned.

If the collision is detected, the UE (in RRC_IDLE or RRC_INACTIVE) sets up the connection to one of the PLMNs and notifies the collision to the CN via NAS signaling (e.g. NAS Mobility Registration/TAU Request). The UE may propose an offset value together with the collision notification.

The CN selects the offset value (based on the proposed value from UE, or it can be a random value or other means can be used) and provides it to the UE via the NAS Mobility Registration/TAU Accept message.

In case of 5GS and NR, the CN signals the UE_offset to gNB by performing the UE Context Modification procedure. The gNB uses the value for the SFN/PO calculation for RAN Paging when the UE is in RRC_INACTIVE state.

The following figure shows when the UE is paged:

If the UE is in RRC_IDLE, the CN includes the UE_offset in the NGAP/S1AP Paging, so the gNB/eNB can use it for the SFN and PO calculation.

Note: 	The NGAP Paging only carries the 5G-S-TMSI, then the UE_offset has to be provided in a new Information Element to allow the gNB to generate the new UE_ID. On the other hand, the S1AP Paging carries both the IMSI and the UE_ID, so the CN can directly provide the new UE_ID (e.g. (IMSI + UE_offset) mod 1024), without sending the UE_offset. This implies that no change is required to the S1AP Paging.

If the UE is in RRC_INACTIVE, the gNB uses the UE_offset previously received from the CN in the UE Context Modification procedure.

The RRC Paging is sent to the UE according to the updated SFN/PO formulas and received by the UE.