3GPP TSG-RAN WG2 Meeting #113-e	R2-2101431

# Electronic meeting, Jan 25 – Feb 05, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0507 | rev | - | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of fallback per CC feature set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The definition below (38.306) was made explicitly for MIMO and BW, the subcarrier spacing was intentionally excluded, since SubcarrierSpacing cannot fallback. Fallback per CC feature set: A feature set per CC that has lower value of UE supported MIMO layers and BW while keeping the numerology and other parameters the same from the reported feature set per CC for a given carrier per band. But the sentence above about subcarrier spacing also states that other parameters shall remain the same in case of a fallback per CC feature set. This is not correct since such fallback should also be applicable to ModulationOrder, e.g. if the UE indicated 64-QAM it could as well do 16-QAM, which contradicts the fallback definition that states “while keeping the numerology and other parameters the same” and makes an exception only for MIMO and BW. To avoid further confusion, a more general wording could be used, in order to exclude solely SubcarrierSpacing from the definition of fallback per CC feature set. Impact analysis Impacted 5G architecture options: Standalone, EN-DC, NGEN-DC, NE-DC, NR-DC Impacted functionality: Feature set per CC Inter-operability: If the network implements the CR and the UE does not, the network may assume the UE supports lower values, which the UE may no support, for parameters signaled within a feature set per CC. If the UE implements the CR and the network does not, there is no inter-operability issue, since the UE would support the lower values for parameters signaled within a feature set per CC. Hence, whether the network assumes or does not assume that the UE supports such lower values would not impact the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 3.1 Definitions - Update definition of “Fallback per CC feature set” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Parameters not explicitly mentioned in the definiton of “Fallback per CC feature set” may be considered as not supporting lower values, which would degrade what the UE can actually support. There could also be a case where the network may assume the UE supports lower values, which the UE may no support, for parameters signaled within a feature set per CC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Fallback band combination: A band combination that would result from another band combination by releasing at least one SCell or uplink configuration of SCell, or SCG. An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination.

Fallback per band feature set: A feature set per band that has same or lower values than the reported values from the reported feature set per band for a given band.

Fallback per CC feature set: A feature set per CC that has same or lower values than the values for the reported feature set per CC for a given carrier per band, while keeping the numerology.

END OF CHANGE