3GPP TSG-RAN WG2 Meeting #113-e	R2-2101465

# Electronic, 25 January - 05 February 2021

| ElectronicCR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | 0061 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support OTDOA assistance data for case of NR serving cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - When the UE is served by a NR cell, and the LMF requests the UE to perform measurements for OTDOA positioning, the LMF may provide assistance data including optionally timing information to enable the UE to quickly acquire the LTE reference cell. However the LMF is not aware of timing relationships between the LTE and NR cells because it does not receive the SFN Initialization Time for NR cells, although it is of course able to do so for LTE cells. RAN3 has agreed a Rel-15 NRPPa CR (R3-207173, with Rel-16 mirror in R3-207174) whereby the OTDOA Information Exchange procedure is used towards gNBs in order to enable the LMF to retrieve timing and access point location information for cells in a gNB. With this, it becomes possible for the LMF to construct the assistance data in LPP related to LTE-NR timing offset. However stage 2 is not aligned to this. - There is a remaining Editor's Note in clause 8.2.3.2.2 which has been removed in the Rel-16 version (CR#0053) but not in the Rel-15 version of the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - OTDOA Positioning support includes potential interaction with gNBs, and this is captured in relevant sections. A new section is introduced under clause 8.2.2 to list the information that may be transferred from the gNB to LMF. The detail on Assistance Data Delivery between LMF and ng-eNB for OTDOA is generalized (i.e. ng-eNB is replaced by NG-RAN node wherever applicable). - Obsolete Editor's Note is deleted. Impact analysis Impacted functionality: - OTDOA Positioning Inter-operability: No Inter-operability issue foreseen (alignment of Stage 2 with Stage 3 specification). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misalignment with Stage 3 specification. - Obsolete Editor's Note remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2, 6.3.1, 8.2.2.2, 8.2.2.X (new), 8.2.3.2.2, 8.2.3.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.3	NG-RAN Positioning Operations

[…]

### 5.3.2	OTDOA Positioning Support

An LMF can interact with any ng-eNB reachable from any of the AMFs with signalling access to the LMF in order to obtain location related information to support the OTDOA for E-UTRA positioning method, including PRS-based TBS for E-UTRA. The information can include timing information for the TP in relation to either absolute GNSS time or timing of other TPs and information about the supported cells and TPs including PRS schedule.

Signalling access between the LMF and ng-eNB may be via any AMF with signalling access to both the LMF and ng-eNB.

An LMF can also interact with any gNB reachable from any of the AMFs with signalling access to the LMF in order to obtain NR cell timing information to support the OTDOA for E-UTRA positioning method, in case the UE is served by a NR cell.

>>>> NEXT CHANGE <<<<

## 6.3	NG-RAN Node terminated protocols

### 6.3.1	NR Positioning Protocol A (NRPPa)

The NR Positioning Protocol A (NRPPa) carries information between the NG-RAN Node and the LMF. It is used to support the following positioning functions:

-	E-CID for E-UTRA where measurements are transferred from the ng-eNB to the LMF.

-	Data collection from ng-eNB's and gNB’s for support of OTDOA positioning for E-UTRA.

-	Cell-ID and Cell Portion ID retrieval from gNB's for support of NR Cell ID positioning method.

The NRPPa protocol is transparent to the AMF. The AMF routes the NRPPa PDUs transparently based on a Routing ID corresponding to the involved LMF over NG-C interface without knowledge of the involved NRPPa transaction. It carries the NRPPa PDUs over NG-C interface either in UE associated mode or non-UE associated mode.

>>>> NEXT CHANGE <<<<

## 8.2	OTDOA positioning

#### […]

#### 8.2.2.2	Information that may be transferred from the ng-eNB to LMF

The following assistance data may be transferred from the ng-eNB to the LMF:

-	PCI, GCI, and TP IDs of the TPs served by the ng-eNB;

-	Timing information of TPs served by the ng-eNB;

-	PRS configuration of the TPs served by the ng-eNB;

-	Geographical coordinates of the TPs served by the ng-eNB.

An ng-eNB may provide assistance data relating only to itself and served TPs via NRPPa signalling, although assistance data from several ng-eNBs and served TPs may be acquired through other mechanisms, see NOTE below.

NOTE:	The assistance data described in this clause are not necessarily transferred only from the ng-eNB, and in some deployment options may not be delivered from the ng-eNB at all; they may also be delivered to the LMF through OAM or other mechanisms external to the NG-RAN. In addition, in cases where assistance data are delivered from the ng-eNB, how the ng-eNB acquires the data is outside the scope of this specification.

#### 8.2.2.3	Information that may be transferred from the UE to LMF

The information that may be signalled from UE to the LMF is listed in Table 8.2.2.3-1. The individual UE measurements are defined in TS 36.214 [17].

Table 8.2.2.3-1: Information that may be transferred from UE to the LMF

| Information | Measurements |
| --- | --- |
| Downlink Measurement Results List for EUTRA TPs | Physical cell IDs |
|  | Global cell IDs |
|  | TP IDs |
|  | Downlink timing measurements |
| Information used to compute a reference time corresponding to the downlink timing measurements to allow correlation of downlink timing measurements with motion information independently obtained from motion sensors | Delta SFN for each E-UTRA neighbour cell |

#### 8.2.2.X	Information that may be transferred from the gNB to LMF

The following assistance data may be transferred from the gNB to the LMF:

-	NR CGI and timing information of the cells served by the gNB;

-	Geographical coordinates of the access points associated with each cell served by the gNB.

>>>> NEXT CHANGE <<<<

### 8.2.3	OTDOA Positioning Procedures

[…]

#### 8.2.3.2	Assistance Data Transfer Procedure

[…]

##### 8.2.3.2.2	Assistance Data Delivery between LMF and NG-RAN node

The purpose of this procedure is to enable the NG-RAN node to provide assistance data to the LMF, for subsequent delivery to the UE using the procedures of clause 8.2.3.2.1 or for use in the calculation of positioning estimates at the LMF.

###### 8.2.3.2.2.1	LMF-initiated assistance data delivery to the LMF

Figure 8.2.3.2.2.1-1 shows the Assistance Data Delivery operation from the -NG-RAN node to the LMF for the OTDOA positioning method, in the case that the procedure is initiated by the LMF.

Figure 8.2.3.2.2.1-1: LMF-initiated Assistance Data Delivery Procedure

(1)	The LMF determines that certain OTDOA positioning assistance data are desired (e.g., as part of a periodic update or as triggered by OAM) and sends an NRPPa OTDOA INFORMATION REQUEST message to the NG-RAN node-. This request includes an indication of which specific OTDOA assistance data are requested.

(2)	The NG-RAN node provides the requested assistance in an NRPPa OTDOA INFORMATION RESPONSE message, if available at the NG-RAN node-. If the NG-RAN node is not able to provide any information, it returns an OTDOA INFORMATION FAILURE message indicating the cause of the failure.