# 3GPP TSG RAN WG2 Meeting #113e						    R2-2101488

Online, January 25 – February 5, 2021

# Title:	DRAFT Reply LS on Cell Configuration within TA/RA to Support Allowed NSSAI

# Response to:	R2-2008759/S2-2006526

# Release:	Release 17

# Work Item:	FS_NR_Slice, FS_eNS_Ph2

Source:                 Futurewei [to be RAN2]

To:                         SA2, RAN3, CT1

Cc:

Contact Person:

Name:	Hao Bi

Tel. Number:

E-mail Address:	hao.bi@futurewei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for the LS on Cell Configuration within TA/RA to Support Allowed NSSAI. RAN2 would like to provide the following feedback:

Q1: In Rel-15 and 16, is it expected that each cell in the tracking area supports the same S-NSSAI(s)? (or, said otherwise, do all cells advertising the same TAC support the same set of S-NSSAIs?).

Answer: In Rel-15 and 16, is it not expected that each cell in the tracking area supports the same S-NSSAI(s) (or said otherwise, not all cells advertising the same TAC support the same set of S-NSSAIs).

Q2a: Can RAN WGs and CT1 explain if it can happen that a UE, e.g. due to local radio conditions, can only use a cell in the TA where not all S-NSSAIs are present in the Allowed NSSAI it received (and that the TA supports), and can RAN WGs and CT1 explain how it is handled today in rel-15/16?

Answer: It is possible to fully dedicate NG-RAN resources to a certain slice, and RAN may admit or reject access to a slice based on its support for the slice, availability of resources, support of the requested service, etc.

Q2b: If an S-NSSAI can be rejected depending on which cell the UE camps on even though it is supported in the TA, for the reason that it is not supported in the cell, is there in rel-15/16 a CT1 error code to handle this case (i.e. can a S-NSSAI be rejected, with a suitable cause code, depending on which cell of the TA the UE camps on, even though this S-NSSAI is known to be supported in the TA, for the reason that this S-NSSAI is actually not supported in the cell of the TA)? Is there any provisions in the RAN or CT1 specifications to handle this case?

Answer: RAN support of network slicing relies on the principle that traffic for different slices is handled by different PDU sessions. RAN performs the admission/rejection of a network slice at PDU session level, when the S-NSSAI corresponding to the PDU Session is indicated in signalling containing PDU session resource information, e.g., during initial context setup or PDU session setup.

2. Actions:

To SA WG2, RAN WG3 and CT WG1.

ACTION: 	RAN2 kindly asks SA2, RAN3 and CT1 to take the above information into account.

3. Date of Next RAN2 Meetings:

RAN2#113-bis-e		April 12 – 20, 2021			Online

RAN2#114-e		May 19 – 27, 2021			Online