3GPP TSG-RAN WG2 #113e	R2-2101494

E-Meeting, 2021-01-25 - 2021-02-05	Revision of R2-2010980

Agenda Item:	8.10.2.1

Source:	Ericsson

Title:	On Random Access in NTNs

Document for:	Discussion, Decision

# Introduction

At the RAN#86 meeting, the Study Item on “Study on solutions for NR to support non-terrestrial network” was completed [1] and the corresponding work item was then approved and later updated at RAN#88 meeting [2]. The new work item aims to specify the enhancements identified for NR NTN and has the following RAN2 objectives on user plane aspects:

- MAC

  - Random access:

    - Definition of an offset for the start of the ra-ResponseWindow for NTN.

    - Introduction of an offset for the start of the ra-ContentionResolutionTimer to resolve Random access contention

    - Solutions for resolving preamble ambiguity and extension of RAR window.

    - Adaptation for Msg-3 scheduling

      - Only for the case with pre-compensation of timing and frequency offset at UE side)

  - Enhancement on UL scheduling to reduce scheduling latency.

  - DRX:

    - If HARQ feedback is enabled, introduction of offset for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL.

    - If HARQ is turned off per HARQ process, adaptions in HARQ procedure

  - Scheduling Request: Extension of the value range of sr-ProhibitTimer

- RLC

  - Status reporting: Extension of the value range of t-Reassembly

  - Sequence Numbers: extension of the SN space only for GEO scenarios

- PDCP

  - SDU discard: Extension of the value range of discardTimer.

  - Sequence Numbers: extension of the SN space for GEO scenarios.

At RAN2#112-e the following agreements were made

Agreements:

- RAN2 working assumption (for RRC idle. FFS for Inactive/Connected): Rel-17 UE with pre-compensation capability obtains UE specific UE-gNB RTT based on its GNSS in LEO/GEO. FFS how this is calculated and what/if anything needs to be broadcasted for the different pre-compensation methods (e.g. common TA) to help the UE to obtain the full UE-gNB RTT.

- If the UE-gNB RTT is pre-compensated, preamble ambiguity is not an issue in Rel-17 NTN (i.e. no enhancements are necessary). FFS how and by whom the possibly multiple components of UE-gNB RTT are pre-compensated

- From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to gNB implementation to ensure sufficient time on UE side for the Msg3 transmission.

Agreements via email - offline 103:

- If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO.

- At least the following are FFS in Rel-17 NTN:

- Report UE-calculated TA in e.g. msg3/msg5/msgA

- Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH

- LCP impact caused by disabling HARQ UL retransmission

Agreements online:

- RAN2 decision on starting ra-ContentionResolutionTimer, ra-ResponseWindow and msgB-ResponseWindow is postponed until further progress in RAN1 regarding UE pre-compensation method and TA estimation accuracy.

In this paper, we discuss the highlighted agreements, random access enhancements, and issues of adapting NR to NTN.

# Discussion on 4-step and 2-step Random Access

## 4-step and 2-step RA basics

The 4-step RA is the baseline in NR. In 4-step RA the Msg1 (preamble) is sent to the gNB and gNB replies with Msg2 (random access response, RAR, addressed to the preamble ID used to send Msg1) which contains a grant for the UE to send Msg3 (for example RRC setup request) which gNB replies to with Msg4 (RRC setup, and contention resolution in case multiple UEs used the same preamble ID), as illustrated in Figure 1.

The design of 2-step RA was completed in Rel-16. 2-step random access transforms the 4-step random access to a 2-step procedure by combining the two uplink-messages Msg1 (preamble) and Msg3 to a MsgA, where the preamble is transmitted in a PRACH occasion as usual, followed by a PUSCH transmission (MsgA PUSCH) with the same content as Msg3 would have had, and combining the two downlink messages Msg2 (RAR) and Msg4 into MsgB, as illustrated in Figure 1.

The obvious benefit of 2-step is completing the RA in only one RTT.

Figure 1. 4-step vs 2-step Random access

In case the UE does not provide its C-RNTI in MsgA PUSCH (i.e. the UE is not in RRC_CONNECTED state), the network responds with a MsgB with a MAC sub-PDU addressed to the preamble index (RAPID) of the preamble the UE transmitted in the PRACH occasion. Similar to a RAR, the MsgB is addressed to a MSGB-RNTI on the PDCCH, where the MSGB-RNTI is derived from the PRACH occasion. The MsgB may contain one or more successRAR MAC sub-PDU(s), each addressed to a UE’s preamble index. The successRAR is sent in response to a UE when the network successfully received the entire MsgA (i.e. both preamble and MsgA PUSCH). The successRAR contains a UE Contention Resolution ID, TPC, HARQ and PUCCH information, Timing Advance Command and C-RNTI and may also include backoff indicator to be used by UEs who don’t receive any response within the MsgB window. The MsgB may also contain one or more fallbackRAR MAC sub-PDU(s) addressed to a UE’s preamble index where the content in the fallbackRAR is the same as in a MAC RAR. The fallbackRAR is sent in response to a UE when the network successfully received the preamble but failed to receive MsgA PUSCH. The fallbackRAR effectively turns the 2-step RA into a 4-step RA and the UE retransmits the MsgA PUSCH content in the form of Msg3.

The UE monitors the PDCCH for a response within a configured MsgB response window with a max length of 40 ms where two SFN LSBs can be included in the DCI scheduling MsgB to make the corresponding PRACH occasion uniquely identified.

## Discussion on 2-step RA for NTN

In the WI description it is assumed that NTN UEs also have GNSS capabilities and the UE may then use its GNSS capability to pre-compensate for the uplink Timing Advance (TA) before sending the random access preamble and the MsgA PUSCH, to minimize the guard times needed. The considerations will then be very similar to 4-step RA and the solution for ra-ResponseWindow and ra-ContentionResolutionTimer applies equally to msgB-ResponseWindow.

It is noted that the start of the msgB-ResponseWindow is defined in RAN1 specification TS 38.213, So, it may be more suitable for RAN1 to decide how to adapt the start and the value range of the msgB-ResponseWindow for NTN.

Starting of msgB-ResponseWindow is specified in the RAN1 spec.

Due to larger TA and Doppler shift variations among UEs in NTN cells, greater guard gaps in time and frequency between MsgA PUSCH resources (PUSCH occasions) may be needed for NTNs.

A UE with GNSS capabilities may estimate its differential delay in the cell and apply the correct TA value, i.e. pre-compensate, before the preamble is sent. This will however introduce an ambiguity in the NW on what TA value the UE used for the pre-compensation. If the UE can include the TA compensation it used for the preamble transmission in MsgA PUSCH, it would allow the gNB to adapt its UL grants scheduling accordingly.

By including the TA value used for pre-compensation in MsgA, the ambiguity of scheduling Msg3 in a fallbackRAR of MsgB can be avoided.

The use of 2-step RA in NTN could reduce the latency for the attach sequence compared to 4-step RA and should be further studied by RAN2 to ensure support for NTN in Rel-17.

In Rel-16 the selection mechanism between 2-step RA or 4-step RA is based on the RSRP. There have been proposals to further optimize this selection in NTNs that may be based on new measures for example the Timing Advance or distance of UE from cell center or even some load measure (requiring extra SIB signaling?). These methods may be feasible but the possible gain from them will not be worth the extra specification effort. We will always need the RSRP conditions to handle situations when the path loss is high, say if a users is indoors or in non-LoS state, as 4-step always have better coverage.

The performance of 2-step RA will start to deteriorate when many users attempt to access or path loss is too high, and the UEs will then fallback to 4-step RA and thus experience only one extra RTT – this will be noticed by the gNB and gNB can manage the resources reserved for 2-step RA to handle the situation. Therefore we do not need to enhance the selection mechanism for now, it may be done in later releases if problems are experienced in the field.

The selection mechanism for 2-step vs. 4-step RA is not updated.

## Pre-compensation estimation

It is noted that RAN1 is discussing different options for estimating what the TA pre-compensation shall be as well as if all UEs can do pre-compensation. Therefore, we propose to postpone all such discussions until RAN1 have conclude.

RAN2 to postpone all discussions on TA estimation and offset estimation until RAN1 has concluded the TA estimation and UE pre-compensation capabilities.

## Discussion on uplink and downlink relative timing.

4-step random access is the random access procedure in NR and should be the baseline for NTN as well. In Rel-16 SI, the needed enhancements for adapting 4-step random access have been heavily discussed. Several of the enhancements are also explicitly described in the Rel-17 NTN WID. RAN2 made the following decision at RAN2#113

Agreements online:

- RAN2 decision on starting ra-ContentionResolutionTimer, ra-ResponseWindow and msgB-ResponseWindow is postponed until further progress in RAN1 regarding UE pre-compensation method and TA estimation accuracy.

In 38.321 ra-ContentionResolutionTimer is started:

Once Msg3 is transmitted the MAC entity shall:

1>	start the ra-ContentionResolutionTimer and restart the ra-ContentionResolutionTimer at each HARQ retransmission in the first symbol after the end of the Msg3 transmission;

1>	monitor the PDCCH while the ra-ContentionResolutionTimer is running regardless of the possible occurrence of a measurement gap;

The purpose of the ra- ContentionResolutionTimer is that the UE shall monitor PDCCH while it is running, and if it expires the UE can take actions like sending another preamble.

If the UE do not correctly estimate the TA (or do not have GNSS capabilities) and the UE uses this TA as an offset from the Msg3 transmission to start the ra-ContentionResolutionTimer and the gNB manage to decode the Msg3 (even though it is not received with correct timing), the UE may start monitoring PDCCH  too early or too late. Such a misalignment may also occur if the TA changes between when UE send the preamble, gNB estimate how TA shall be updated and provide in the Timing Advance Command of the RAR until the UE use it to send Msg3.

If UE estimation of TA is not accurate, or if RTT changes before the TA is used, starting ra-ContentionResolutionTimer after an offset of TA after msg3 transmission, the UE may start PDCCH monitoring too early or too late.

In 38.211 we have the uplink-downlink timing relations at the UE:

There is one set of frames in the uplink and one set of frames in the downlink on a carrier.

Uplink frame number  for transmission from the UE shall start  before the start of the corresponding downlink frame at the UE where  is given by [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used.

Figure 4.3.1-1: Uplink-downlink timing relation.

Each frame is 10 milliseconds. In TNs the UL and DL are reasonable aligned, for example for cell sizes of up to 150 km, the TA is less than a millisecond.

In NTNs the RTT and the timing advance is much longer than one frame. Thus, at the UE, the uplink frame i do not overlap in time with the downlink frame i. So clearly there is a need for an offset, or is it?

In NTNs, the uplink system frame i will not overlap in time with downlink system frame i.

Thus, basing the start of the ra-ContentionResolutionTimer on the uplink is not completely logical as it is for deciding what the UE shall do on the downlink. Therefore, we suggest implementing the offset as a change of the base for starting the ra-ContentionResolutionTimer from the corresponding symbol of the downlink frame instead of an uplink frame.

Basing the start of UE timers, for monitoring of PDCCH, on the UL timing will always risk starting the timer late or early.

When UL and DL timing is aligned in the gNB, basing the start of UE timers, for monitoring of PDCCH, on the DL timing will always accurate no matter what the timing difference between DL and UL is at the UE.

Basing the start of ra-ContentionResolutionTimer on the downlink timing does not rely on the UE correctly estimating the TA and keeping it updated if the RTT drift away.

Thus, it is future proof to base the start of ra-ContentionResolutionTimer on the downlink timing.

Basing the start of ra-ContentionResolutionTimer on the downlink timing is future proof.

A similar but stronger argument can be made for the starting of ra-ResponseWindow as well as the msgB-ResponseWindow as the TA used for preamble respectively MsgA are estimated by the UE itself and not adjusted by the TAC of the RAR.

For UEs that do not have GNSS capabilities, the starting of ra-ResponseWindow based on the used TA as an offset after preamble transmission can result in the UE starting to monitor PDCCH during the wrong SFN, that is, the UE may accept an RAR intended for a different UE as valid and we have created a msg3 collision between different users (but as the TA used by the UE without GNSS capabilities is incorrect that UE will probably transmit outside of the intended msg3 transmission occasion). The situation for msgB-ResponseWindow is similar.

For UEs without GNSS capabilities, using an offset of TA after msg1 transmission to start ra-ResponseWindow, may create msg3 collisions.

From RAN2 perspective, the start of ra-ResponseWindow can be made in the first PDCCH occasion after the downlink symbol that has the same symbol number, slot number and system frame number as the last uplink symbol of the PRACH occasion where msg1 was transmitted.

From RAN2 perspective, the start of msgB-ResponseWindow can be made in the first PDCCH occasion after the downlink symbol that has the same symbol number, slot number and system frame number as the last uplink symbol of the PUSCH transmission of MsgA.

It is noted that the Rel-17 NTN WID assumes that NTN UE equipped with GNSS would have pre-compensation capability. RAN1 will further look into whether there are scenarios where UE has GNSS but does not have pre-compensation capability. Solutions for resolving preamble ambiguity and extension of RAR window are only required when UE cannot perform pre-compensation. Therefore, RAN2 could wait for RAN1 input before discussing solutions for resolving preamble ambiguity and extension of RAR window.

RAN2 to wait for RAN1 progress before discussing solutions for resolving preamble ambiguity.

The adaptation for Msg3 scheduling is listed as an objective for the case with pre-compensation of timing and frequency offset at UE side. This objective is a bit unclear. If it is about scheduling timing relationship for Msg3, it appears to be a topic within RAN1 expertise. If it is about UEs that cannot accurately pre-compensate the timing advance – then that is not an issue because in the WID it is assumed the UEs have GNSS capabilities. We think this objective is about the gNB receiving a preamble is not enough for the gNB to know the UE TA as the UE pre-compensates the RTT and thus the gNB do not know how much difference there is between the UL and the DL timing in the UE and consequently do not know how much time is needed from when the gNB sends msg2 until the UE has decoded it and prepared and is ready to send msg3. This issue is already handled in the RAN1 discussions on Koffset+k2, and RAN2 discussion can be postponed until RAN1 have concluded.

RAN2 to wait for RAN1 progress before discussing the scope of the objective “Adaptation for Msg-3 scheduling”.

What is pertinent to RAN2 could be the content of Msg-3. For example, Msg-3 may need to carry UE’s pre-compensated timing advance value so that network can utilize the knowledge for scheduling in later messages (that is the gNB can the provide the UE with a grant that is valid after a few slots to allow the UE to process the grant and prepare the transmission).

However, it is worth discussing which UL message (Msg-3/MsgA or a different one) should be chosen for reporting UE’s pre-compensated timing advance value. Increasing the size of Msg3/MsgA may decrease the coverage, thus it may be better to not mandate the sending of the used TA in Msg3/MsgA. As the gNB may need to control inclusion of TA based on the coverage, the gNB shall be able to request it by a flag in the RAR or by SI broadcasting or by the size of the grant for msg3/msgA.

The gNB can request the UE to report the used TA value in the msg3/msgA or msg5. FFS how to request the inclusion of TA.

# Discussion on RACH-less and 2-step CFRA

RACH-less handover was introduced as part of the mobility enhancements in LTE Rel-14. RACH-less HO was excluded during Rel-16 within the NR Mobility Enhancement WI when RAN4 questioned the benefit of NR RACH-less because in LTE it was only specified for TA=0 or TA equal between source and target cells. TA=0 is only for small cells and equal TA depends on propagation conditions in the deployment. The NW has thus limited ways to know that TA is equal and therfore if it can safely use RACH-less HO. Supporting RACH-less HO in NR Rel-16 was deemed to require a large amount of work effort from RAN1/4 to handle large TA differences between cells, which was the main reason the RACH-less HO was excluded.

CBRA and CFRA are supported in 2-step RA and the 2-step RA procedure is also considered to be a valid solution for NTN and the Rel-16 2-step RA design is considered as baseline for 2-step RA in NTN.

In the objectives in [2], it is assumed that an NTN UE will have GNSS capability. A GNSS capable UE with the ability to pre-compensate for the TA before the attach sequence is started will then not only be able to use 2-step RACH but it may also be able to use RACH less, at least for RA in connected mode where the TA is known. This could result in minimized interruption times, reduced signaling and decreased RACH load.

## RACH-less HO

The RACH-less feature basically means skipping Msg1 and Msg2 of the random access procedure when the UE accesses the target cell and that the first transmission is instead the message confirming the completion of the handover. In the absence of a random access procedure in the target cell, UL transmission resources for the UE’s first message in the target cell is allocated either through pre-allocated UL grants or UL grants dynamically allocated through PDCCH in the target cell without preceding scheduling requests. The pre-allocated UL grants potentially achieve shorter interruption, since the PDCCH allocation adds the delay waiting for a grant and when the grant is received a delay until it is valid, but PDCCH allocation is more flexible. We further discuss the need for RACH less HO in our connected mode paper [3].

## RACH-less HO for NTN

The mobility for GEO based scenarios will to a large extent be similar to the mobility of terrestrial system. For the non-GEO, i.e. both MEO and LEO, it is expected that frequent HO will be seen due to the rapid satellite movement. In TR 38.821, it has been captured how to estimate the HO rate for different scenarios (cell size, UE speed, satellite speed, etc.). For a stationary UE in a LEO cell of 50km in diameter, we get a Time to HO of 6.61s (see Table 7.3.2.1.3-1 in TR 38.821 pasted below). Even for low UE densities, the total amount of HO per second will be very high and cause a large signaling load in both UE and NW.

Table 7.3.2.1.3-1: Time to HO for min/max cell diameter and varying UE speed.

| Cell Diameter Size (km) | UE Speed (km/hr) | Satellite Speed (km/s) | Time to HO (s) |
| --- | --- | --- | --- |
| 50 (lower bound) | +500 | 7.56* | 6.49 |
|  | -500 |  | 6.74 |
|  | +1200 |  | 6.33 |
|  | - 1200 |  | 6.92 |
|  | Neglected |  | 6.61 |
| 1000 (upper bound) | +500 |  | 129.89 |
|  | -500 |  | 134.75 |
|  | +1200 |  | 126.69 |
|  | - 1200 |  | 138.38 |
|  | Neglected |  | 132.28 |

The HO rate in non-GEO NTN may require very high signalling load.

Even with a PRACH configuration that maximizes the available PRACH opportunities in a cell, the collision rate will be high. Adding attach attempts from other UEs than the ones that are doing HO, the collision rate will increase even further. This means that in order to support mobility, the signaling needs to be reduced.

To support mobility for NTN, signaling associated with handovers needs to be reduced.

The reason for excluding RACH-less from the mobility enhancement WI was the lack of accurate ways of estimating the TA between cells and thus, to be able to use RACH-less at HO, it requires the UE to have TA value equal to zero or a very small TA difference between the source and target cell. In an intra-satellite HO, the difference in propagation delay will be close to zero since the distance to the satellite is the same in the target and source cell (as illustrated in Figure 2) and therefore the TA can be reused at intra satellite HO in NTN.

Figure 2 TA value difference between cells at intra satellite HO is zero

For intra satellite HO, the difference in delay between cells is zero.

As the assumption is that the UEs will have GNSS capability, the UE may also be able to estimate the TA to a cell in a different satellite as the source cell or broadcasting may provide the information needed for an accurate estimate.

For inter satellite HO, the UE can accurately estimate the difference in delay.

It is believed that the RACH capacity will be limited even in sparsely populated areas due to the large supported cells in NTN. RACH-less will offload the PRACH resources and thus reduce the collision rate. It also has the potential to reduce the interruption time since it is possible to use dense pre-allocated grants in the HO command.

RACH-less would reduce PRACH load and decrease the RA collision rate. Thus, RACH-less is highly beneficial for NTN

## Comparison of RACH-less HO with 2-step RA

Since the main gains from RACH-less HO is the elimination of Msg1 and Msg2 in the 4-step RA procedure in the target cell, it is interesting to compare HO with 2-step RA with RACH-less HO. With 2-step RA in the target cell, the UE can transmit the RRCReconfigurationComplete message as a part of the first transmission in the target cell already. Hence, the delay caused by the Msg1-Msg2 message exchange is eliminated in the case of HO with 2-step RA too.

The delay caused by the Msg1-Msg2 message exchange is eliminated both with RACH-less HO and with HO with 2-step RA.

The interruption times are similar in RACH-less HO and HO with 2-step RA, depending on the relation between the (time domain) density of pre-allocated UL grants and that of PRACH occasions for 2-step RA.

However, there are other differences between the two methods. Since pre-allocated UL grants in the RACH-less HO case are allocated over a short temporary period, they can assumedly be dense than the PRACH occasions for 2-step RA. If dynamic grants are used for the RACH-less HO, there are some DL PDCCH resources used but the UL PUSCH resources needed will be kept to a minimum

RACH-less HO is a resource efficient alternative to 2-step RA.

Also, the pre-allocated or dynamic UL grants in RACH-less may be adapted for the UE, e.g. larger if desired and with tailored MCS, based on measurement reports from the UE. Furthermore, with RACH-less HO the UE can avoid the overhead of the RA preamble transmission and therefore the RA collision rate at HO is not increased.

RACH-less will not increase the preamble collision rate and will ensures the RA capacity of the cell can be used for other RA attempts.

The above analysis of RACH-less and to 2-step RA shows that both can be beneficial for NTN: 2-step RA has benefits over 4-step RA in in idle/inactive mode but in connected mode RACH-less HO shows clear benefits over HO with 2-step RA. We therefore propose RAN2 to further evaluate the support of RACH-less in the Rel-17 WI on “Solutions for NR to support non-terrestrial networks (NTN)”.

RAN2 to supports RACH-less HO for NTNs.

# Conclusion

We made the following observations:

Observation 1	Starting of msgB-ResponseWindow is specified in the RAN1 spec.

Observation 2	By including the TA value used for pre-compensation in MsgA, the ambiguity of scheduling Msg3 in a fallbackRAR of MsgB can be avoided.

Observation 3	The purpose of the ra- ContentionResolutionTimer is that the UE shall monitor PDCCH while it is running, and if it expires the UE can take actions like sending another preamble.

Observation 4	If UE estimation of TA is not accurate, or if RTT changes before the TA is used, starting ra-ContentionResolutionTimer after an offset of TA after msg3 transmission, the UE may start PDCCH monitoring too early or too late.

Observation 5	In NTNs, the uplink system frame i will not overlap in time with downlink system frame i.

Observation 6	Basing the start of UE timers, for monitoring of PDCCH, on the UL timing will always risk starting the timer late or early.

Observation 7	When UL and DL timing is aligned in the gNB, basing the start of UE timers, for monitoring of PDCCH, on the DL timing will always accurate no matter what the timing difference between DL and UL is at the UE.

Observation 8	Basing the start of ra-ContentionResolutionTimer on the downlink timing does not rely on the UE correctly estimating the TA and keeping it updated if the RTT drift away.

Observation 9	Basing the start of ra-ContentionResolutionTimer on the downlink timing is future proof.

Observation 10	For UEs without GNSS capabilities, using an offset of TA after msg1 transmission to start ra-ResponseWindow, may create msg3 collisions.

Observation 11	From RAN2 perspective, the start of ra-ResponseWindow can be made in the first PDCCH occasion after the downlink symbol that has the same symbol number, slot number and system frame number as the last uplink symbol of the PRACH occasion where msg1 was transmitted.

Observation 12	From RAN2 perspective, the start of msgB-ResponseWindow can be made in the first PDCCH occasion after the downlink symbol that has the same symbol number, slot number and system frame number as the last uplink symbol of the PUSCH transmission of MsgA.

Observation 13	The HO rate in non-GEO NTN may require very high signalling load.

Observation 14	To support mobility for NTN, signaling associated with handovers needs to be reduced.

Observation 15	For intra satellite HO, the difference in delay between cells is zero.

Observation 16	For inter satellite HO, the UE can accurately estimate the difference in delay.

Observation 17	RACH-less would reduce PRACH load and decrease the RA collision rate. Thus, RACH-less is highly beneficial for NTN

Observation 18	The delay caused by the Msg1-Msg2 message exchange is eliminated both with RACH-less HO and with HO with 2-step RA.

Observation 19	The interruption times are similar in RACH-less HO and HO with 2-step RA, depending on the relation between the (time domain) density of pre-allocated UL grants and that of PRACH occasions for 2-step RA.

Observation 20	RACH-less HO is a resource efficient alternative to 2-step RA.

Observation 21	RACH-less will not increase the preamble collision rate and will ensures the RA capacity of the cell can be used for other RA attempts.

We propose the following:

Proposal 1	The selection mechanism for 2-step vs. 4-step RA is not updated.

Proposal 2	RAN2 to postpone all discussions on TA estimation and offset estimation until RAN1 has concluded the TA estimation and UE pre-compensation capabilities.

Proposal 3	RAN2 to wait for RAN1 progress before discussing solutions for resolving preamble ambiguity.

Proposal 4	RAN2 to wait for RAN1 progress before discussing the scope of the objective “Adaptation for Msg-3 scheduling”.

Proposal 5	The gNB can request the UE to report the used TA value in the msg3/msgA or msg5. FFS how to request the inclusion of TA.

Proposal 6	RAN2 to supports RACH-less HO for NTNs.

# References

TR 38.821 “Study on solutions for NR to support non-terrestrial networks”

RP-201256, “Solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#88, July 2020

R2-2100346 “Connected mode aspects for NTN”, Ericsson, RAN2#113, Jan. 2021