3GPP TSG-RAN2 Meeting #113e	R2- 2101497

# e-Meeting, 25th January – 5th February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1035 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for handling of unforeseen protocol data during DAPS HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In case of DAPS handover, the UE continues the downlink user data reception from the source gNB until releasing the source cell. The source gNB can continue HARQ retransmission for MAC PDU, which was transmitted but not acknowledged before DAPS handover. The MAC PDU may include MAC subPDU corresponding to LCH for non-DAPS bearer during DAPS handover. If the source MAC entity receives the MAC PDU, then it can neither process the MAC subPDU for non-DAPS bearer due to the absence of the associated RLC entity nor discard it because the LCH for non-DAPS bearer is still configured. In this regard, the UE behavior is not clear to handle this unforeseen protocol data, which may cause an error in UE side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To specify handling of unforeseen protocol data for the source MAC entity during DAPS handover. Impact Analysis for archicture option The impacted 5G architecture options are SA, EN-DC, NE-DC, and NR-DC. Impacted functionality: Handling of unknown, unforeseen and erroneous protocol data Inter-operability: If the network is implemented according to the CR while the UE is not, it is not clear how UE is supposed to hanlde the unforeseen protocol data for configured LCH corresponding to non-DAPS bearer, which may cause an error. If the UE is implemented according to the CR while the network is not, no interoperability issue would be foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior is not clear to handle the unforeseen protocol data for configured LCH corresponding to non-DAPS bearer, which may cause an error in UE side. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

5.13	Handling of unknown, unforeseen and erroneous protocol data

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, the MAC entity shall at least:

1>	discard the received subPDU.

When the source MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment during DAPS handover, containing an LCID or eLCID value for non-DAPS bearer, the MAC entity shall at least:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for broadcast or groupcast, or an LCID value which is not configured, the MAC entity shall:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for unicast, the MAC entity shall:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.

<End of modification>