3GPP TSG-RAN WG2 Meeting #113-e	R2-2101518

Elbonia, Jan 25th – Feb 5th, 2021

Agenda item:	8.10.2.3

Source: 	Sequans Communications

Title: 	On RLC t-Reassembly for NTN

Document for:	Discussion and Decision

# Introduction

During last meeting, RAN2 agreed to extend the value range of t-Reassembly timer, to cope with longer RTD in NTN scenarios when HARQ feedback is enabled. In this contribution, we discuss possible side effects and enhancements. We also discuss the case when HARQ feedback is disabled.

# Discussion

The timer t-Reassembly in RLC is mainly needed to allow HARQ retransmissions from lower layers, before triggering ARQ (or moving windows in UM). Hence, its setting will depend on whether the NW intents to use HARQ feedback or not for DL transfer of a given RLC bearer.

It was agreed that HARQ feedback can be disabled at least on a per-process basis. In DL, the NW scheduler may dynamically decide to use “HARQ feedback enabled/disabled” processes, but we assume that:

- Large t-Reassembly would be configured for bearers mapped to “HARQ feedback enabled” processes. It is expected to be used for traffic requiring high reliability, e.g. SRBs. This would include application/services running over NAS. This may also be used for specific traffic such as control traffic (e.g. OAM etc).

- Short (or null) t-Reassembly would be configured for bearers mapped to “HARQ feedback disabled” processes, requiring high throughput

## Large t-Reassembly drawbacks

The following formula from [1] indicates the value required depending of RTD, maximum retransmissions and scheduling margin:

t-Reassembly = RTD * nrof_HARQ_retrans + scheduling_offset

Given RTD of up to 541ms in NTN, maximum value could be quite high, compared to the maximum of 200ms in NR.

In our view, a typical case of such traffic would be transmission of small data bursts (continuous transmission is not possible due to the shortage of HARQ processes relative to the large RTD). Note that typically, the NW would also poll on the last PDU of such data burst.

Configuring a high timer value comes with drawbacks (as explained in [3]). Indeed, there is only one such timer instance, to simplify specification and implementation. In case a gap Y (of SDU(s)/segments) occurs while the t-Reassembly timer is already running due to a previous gap X, the timer will be started again (to wait for recovery of Y) when previous timer instance is either:

- stopped (X is recovered, thanks to HARQ)

- or expired (X could not be fully recovered by HARQ, i.e. lost).

This has 2 undesirable consequences:

- An additional STATUS is triggered (to report missing Y) in case X was lost (which impacts UL scheduling)

- The STATUS reporting Y is unnecessarily delayed (up to t-Reassembly value, which might be further delayed by a StatusProhibit timer), whether X was recovered or lost.

This can be a common case, depending of HARQ operation point. E.g. with 10 % BLER, the probability to have 2 failed TBs in a burst of 20 slots is around 20%.

Observation 1: Additional and/or delayed STATUS transmission may happen in case of failed TBs in a burst

This is described in the following figure, in which X and Y were lost (if X was recovered while Y only is lost, the issue would be a delayed STATUS). The SR trigger from polling is delayed till the t-Reassembly expiry related to Y.

Figure 1 – Additional and delayed STATUS

## Enhancements when large t-Reassembly is used

In above scenario, ideally the t-Reassembly should be started at the end of the burst, and a single STATUS should be sent for the whole DL data burst at expiry of the timer.

One approach is to have t-Reassembly triggered from NW side (e.g. along with polling), instead or in addition to existing triggering conditions; or to consider a different timer triggered from NW side. This has however some complications.

In our view, a simple way to better handle the above scenario would be to introduce a t-Reassembly-delay timer, which would be used to delay the start of t-Reassembly upon missing PDU detection. The timer could also be stopped upon polling as this typically indicates the end of a data burst, reducing useless delay. An additional advantage is that the STATUS transmission could be then better predicted by the NW, which can preschedule the uplink resources.

The t-Reassembly-delay timer would be configured to the typical length of a data burst, which in NTN scenarios might be at least one order of magnitude shorter than the value of t-Reassembly timer.

This would result in the following optimal behavior:

Figure 2 – Fast unique STATUS with t-Reassembly-delay timer

A possible drawback of a t-Reassembly-delay timer could be that the STATUS reporting missing X would be delayed by up to t-Reassembly-delay value, on top of t-Reassembly. However, considering NTN scenarios where t-Reassembly-delay (~data burst size) << t-Reassembly (~several times RTD), this drawback is unsignificant.

Proposal 1: Consider introducing t-Reassembly-delay timer to avoid additional and/or delayed STATUS PDUs

Another approach is to introduce in NR the possibility to disable the triggering of Status Report upon missing PDU detection, as it was already introduced in LTE RLC for NB-IoT. However, this would only avoid the additional STATUS (as the only Status Report trigger would be polling from NW). The STATUS (reporting X and Y in that case) would remain delayed by up to 2*t-Reassembly.

## Short/Null t-Reassembly

For high throughput bearers, NW is expected to configure and use “HARQ feedback disabled” processes. It is expected that t-Reassembly is configured to a short (e.g. 10ms) value (for instance to allow blind HARQ retransmissions), or even to 0 if the NW does not plan any out-of-ordering scheduling.

Configuring a short t-Reassembly has in theory the benefit to detect/report quickly missing PDUs (assuming spare failures). However, following a missing PDU X, the RLC receiver will keep triggering SR while X is missing upon each new PDU arrival (if t-Reassembly is not running), as described in the following figure:

Figure 3 – “Continuous” SR Triggers upon PDU reception

This mandates a large SR prohibit timer (at least equal to RTD) and somehow defeats the benefit of the fast detection (except for the first one, for which SR prohibit is not yet running). Instead, it might be better to rely on SR triggering by polling only, as this has the benefit that the NW would always know when a SR transmission is performed (and can then preschedule resources, e.g. including scheduling blind UL retransmissions to enhance reliability of the SR).

Observation 2: Missing PDU detection SR trigger is less useful in NTN HARQ disabled case

Proposal 2: Consider the possibility to disable the missing PDU detection SR trigger (as in NB-IoT)

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: Additional and/or delayed STATUS transmission may happen in case of failed TBs in a burst

Proposal 1: Consider introducing t-Reassembly-delay timer to avoid additional and/or delayed STATUS PDUs

Observation 2: Missing PDU detection SR trigger is less useful in HARQ disabled case

Proposal 2: Consider the possibility to disable the missing PDU detection SR trigger (as in NB-IoT)

# References

- 3GPP TR 38.821 V16.0.0

- R2-2010170, “Additional RLC and PDCP aspects for NTN”, Sequans Communications