3GPP TSG-RAN WG2 Meeting #113-e	R2-2101524

# Online, Jan 25th - Feb 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1040 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CG type 1 resources handling when timeAlignmentTimer is expired-Opt 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-1-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current R16 spec, the configured grant type 1 resources would be cleared by the MAC entity if the related timealignmentTimer is expired 1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with the PTAG: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> else if the timeAlignmentTimer is associated with an STAG, then for all Serving Cells belonging to this TAG: 3> flush all HARQ buffers; 3> notify RRC to release PUCCH, if configured; 3> notify RRC to release SRS, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG. However, the configured grant uplink type 1 resources is configured in RRC layer not in MAC layer. Even though the cg type 1 resources is cleared in MAC layer when TA timer is expired, It is still unclear how to initialize the RRC configured CG type 1 resource when timeAlignmentTimer is running again. The detail can be referred to the related discussion paper. This CR is for the case when proposal 1 in discussion paper is NOT agreed, the proposal 1 in discussion paper is shown as below for convenience: Proposal 1: RAN2 confirm that the RRC configuration for type 1 configured grant will not be released in case the timeAlignmentTimer expires (i.e. delta configuration is allowed. e.g. for pusch-RepTypeIndicator-r16). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: Release the configured grant type 1 resources when the timeAlignmentTimer of the associated TAG is expired. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC, (NG)EN-DC Impacted functionality: Configured UL Grant type 1 Inter-operability: If UE implements according to this CR but NW does not, no inter-operability can be found. If NW implements according to this CR but UE does not, no inter-operability can be found. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configured grant type 1 configuration is still saved in the RRC layer when timeAlignmentTimer of the associated TAG is expired and the corresponding configured grant type1 resources cannot be initialized when timeAlignmentTimer is running again unless no reconfiguration of this type 1 CG is received. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | . |  |  |  |  |  |  |  |  |

Start of the change

5.2	Maintenance of Uplink Time Alignment

RRC configures the following parameters for the maintenance of UL time alignment:

-	timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned.

The MAC entity shall:

1>	when a Timing Advance Command MAC CE is received, and if an NTA (as defined in TS 38.211 [8]) has been maintained with the indicated TAG:

2>	apply the Timing Advance Command for the indicated TAG;

2>	start or restart the timeAlignmentTimer associated with the indicated TAG.

1>	when a Timing Advance Command is received in a Random Access Response message for a Serving Cell belonging to a TAG or in a MSGB for an SpCell:

2>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble:

3>	apply the Timing Advance Command for this TAG;

3>	start or restart the timeAlignmentTimer associated with this TAG.

2>	else if the timeAlignmentTimer associated with this TAG is not running:

3>	apply the Timing Advance Command for this TAG;

3>	start the timeAlignmentTimer associated with this TAG;

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5; or

3>	when the Contention Resolution is considered successful for SI request as described in clause 5.1.5, after transmitting HARQ feedback for MAC PDU including UE Contention Resolution Identity MAC CE:

4>	stop timeAlignmentTimer associated with this TAG.

2>	else:

3>	ignore the received Timing Advance Command.

1>	when an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE as specified in clause 5.1.4a:

2>	apply the Timing Advance Command for PTAG;

2>	start or restart the timeAlignmentTimer associated with PTAG.

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with the PTAG:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3> notify RRC to release configured uplink grant type 1 configuration for all Serving Cells;

3>	clear any configured downlink assignments and configured uplink grants of configured uplink grant type 2 associated with all Serving Cells;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

2>	else if the timeAlignmentTimer is associated with an STAG, then for all Serving Cells belonging to this TAG:

3>	flush all HARQ buffers;

3>	notify RRC to release PUCCH, if configured;

3>	notify RRC to release SRS, if configured;

3> notify RRC to release configured uplink grant type 1 configuration for all Serving Cells;

3>	clear any configured downlink assignments and configured uplink grants type 2 associated with all Serving Cells;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

When the MAC entity stops uplink transmissions for an SCell due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running. Furthermore, when the timeAlignmentTimer associated with the PTAG is not running, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell.

Start of the change